| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G604 |
| --- | --- |

| Title: | Non-CE6: Cross-checking of JCTVC-G598: Intra prediction based on weighted template matching predictors (WTM) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | P.Bordes, P.Salmon | Tel: Email: | +33 2 99 27 32 42 philippe.bordes@technicolor.com philippe.salmon@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This document is a cross check report for the Intra prediction based on weighted template matching predictors (WTM) proposed in JCTVC-G598 [1].

The corresponding performance results are reported. The simulation results reportedly match with the results provided by the proponents.

- Code verification and test conditions

INRIA provided Technicolor with the source code to be tested.

All the modifications in the HM4.0 corresponding to WTM implementation are encapsulated in-between the macros listed below.

The tested configuration corresponds to the following settings:

#define INRIA_WTM		1		// INRIA_WTM define

#if INRIA_WTM				// TypeDef.h add in order to locate this

// portion of code

#define OBSERVER		1           // Observer define

#define SWTM_4         	1           // Define for 4 neighboors used

#endif

In the config parameter file, the parameter STM and STMObserver has been set to 1 and 0 respectively. The value of parameters STM4x4, STM8x8, STM16x16, STM32x32, STM64x64 is set depending on the sequence class as depicted in Table 1.

|  | HE |  |  |  |  | LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4x4 | 8x8 | 16x16 | 32x32 | 64x64 | 4x4 | 8x8 | 16x16 | 32x32 | 64x64 |
| Class A | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| Class B | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 |
| Class C | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| Class D | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| Class E | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 |
| Class F | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 |

Table 1: STM4x4, STM8x8, STM16x16, STM32x32, STM64x64 parameters settings depending on the sequence class.

The source code have been successfully compiled and tested on a cluster grid Linux (RedHat 5.5) with the following characteristics:

- CPU: 2 x Intel(R) Xeon(R) E5680 @ 3.33GHz, 6 cores.

- RAM: 96 Go per 6 cores.

# Simulation Results

We present the simulation results obtained for A,B,C,D,E,F classes (Figure 1), in Intra only High-Efficiency and Intra only Low-Complexity.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,0% | -2,0% | -2,6% | -0,1% | -1,4% | -2,0% |
| Class B | -0,8% | -0,9% | -0,8% | -0,9% | -0,7% | -0,6% |
| Class C | -0,9% | -0,5% | -0,7% | -1,0% | -0,6% | -0,8% |
| Class D | -0,4% | -0,3% | -0,4% | -0,5% | -0,4% | -0,4% |
| Class E | -0,5% | -0,4% | -0,4% | -0,7% | -0,6% | -0,2% |
| Class F | -3,1% | -2,9% | -2,8% | -3,5% | -3,5% | -3,4% |
| Overall | -1,0% | -1,2% | -1,3% | -1,1% | -1,2% | -1,2% |
|  | -1,0% | -1,2% | -1,3% | -1,1% | -1,2% | -1,2% |
| Enc Time[%] | 115% |  |  | 120% |  |  |
| Dec Time[%] | 104% |  |  | 109% |  |  |

Figure 1: results obtained for A,B,C,D,E,F classes.

# Conclusion

The results presented by INRIA in JCTVC-G598 are confirmed.

# References

- T.Guionnet & L.Guillo, “ Intra prediction based on weighted template matching predictors (WTM),” Doc. JCTVC-G598, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Geneva, Switzerland, November 2011.