| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G607 WG11 Number: m22172 |
| --- | --- |

| Title: | High-Level Syntax for Bitstream Extraction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg, Thomas Rusert Ericsson Research 164 80 Stockholm Sweden | Tel: Email: | +46 761153563 rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes changes to the NAL unit header and SPS, aiming at providing high-level signaling to support bitstream extraction for both temporal scalability (as supported in the current WD) and any other bitstream scalability (as defined in the future). Compared to the current WD, it is proposed to remove the second byte from the NAL unit header, and instead introduce a NAL unit header extension bit in the first byte. If the extension bit is equal to 0, the NAL unit header has only one byte. If the extension bit is equal to 1, a byte-aligned SPS identifier is sent in the NAL unit header, which co-serves as generic layer identifier and priority indicator. Complementary, syntax for signaling of dependencies between scalable layers and layer properties such as temporal_id and level_idc are introduced in the SPS. The proposed mechanism is claimed to be beneficial both in the context of temporal scalability as supported in the current WD, and when HEVC is extended with new scalability tools in the future. HM-based software implementation of the proposed signaling and bitstream extraction are available as part of Ericsson’s response to MPEG’s Call for Proposals on 3D Video Coding Technology.

Claimed advantages

It is claimed that setting the proposed extension bit to 0 in applications where no temporal (and no other) scalability is required avoids a small but unnecessary bitrate overhead that is present according to the current WD. It is further claimed that if temporal scalability is supported in the bitstream, the proposed mechanism enables encoders to provide a separate level_idc indication for each temporal layer, and thus more accurate signaling of required decoder capabilities than in the current WD. It is also claimed that when new mechanisms for scalability are introduced in the future, the proposed design can be extended to provide signaling for both existing and new scalability mechanisms without changing the NAL unit header. Finally, it is claimed that if temporal scalability is used (or other scalabilities in the future), the proposed mechanism facilitates a bitstream extraction process that can associate both VCL and non-VCL NAL units with scalable layers in a simple way.

# Introduction

The NAL unit header in the current WD has a length of either 1 or 2 bytes. The second NAL unit header byte is present if and only if the NAL unit carries a coded slice (IDR, CDR, or non-IDR/CDR). If present, the second NAL unit header byte carries the temporal_id and output_flag syntax elements. We have identified some shortcomings with that concept that are addressed in this contribution.

Extensible NAL unit header definition (“scalability hook”)

One of the key purposes of the NAL unit header is to reveal high-level information about the NAL unit contents to the network. Based on information carried in the NAL unit, a network node can decide on operations such as packet dropping in case of network congestion or bitstream extraction in case of selective forwarding of bitstream subsets when scalable coding is used. Currently, the WD supports temporal scalability, and the temporal_id syntax element is signaled in the NAL unit header, thus revealing information about temporal layers.

It is anticipated that future versions of the HEVC specification will contain functionalities for scalable coding such as spatial scalability and multiview coding, similar as recent versions of AVC include SVC and MVC extensions, respectively. Such functionalities will require mechanisms for signaling of scalability layer information in the NAL unit header.

When AVC was initially designed, it was not foreseen that scalable extensions would be added later. Thus the NAL unit header did not include any general extension mechanisms for scalability signaling. When SVC and MVC were added, since it was required to retain compatibility with existing AVC implementations, the basic AVC NAL unit header design had to be left untouched. Instead, new NAL unit types were introduced to represent coded slices of scalable layers, those NAL unit types carrying extended NAL unit headers revealing information about scalable layers. Furthermore, prefix NAL units were designed to reveal information about scalable layers for coded slices in the AVC-compatible base layer. The resulting high-level syntax design is somewhat complicated, and it complicates implementation of media aware network elements since they need to parse a variety of layer identifiers, take care of associations between adjacent NAL units (prefix and following NAL unit), and need to be aware of the profile of the video stream to be able to distinguish between and then parse SVC and MVC NAL unit headers.

Having the AVC lesson in mind and anticipating that HEVC will have scalable extensions, we suggest starting over and designing a clean and “future-proof” (that is, extensible) NAL unit header already in the HEVC “core” specification.

The proposed NAL unit header is designed in such a way that it caters for signaling of both the current temporal scalability mechanism as well as future scalability mechanisms, without requiring changes to the NAL unit header when new scalability mechanisms are introduced. Additionally, it includes a generic priority signaling mechanism that allows for simple bitstream thinning without any need for a network node to know about the profile or scalability mechanism used.

Association of non-VCL NAL units with scalable layers

There is no simple mechanism in the current WD to associate non-VCL NAL units carried in a scalable HEVC bitstream (such as SPS, PPS and SEI NAL units) with scalable layers, and thus no simple way for a bitstream extractor to discard non-VCL NAL units that are not required in an extracted bitstream subset. When a certain scalable layer is to be extracted, a bitstream extractor would either have to parse the payloads of the respective non-VCL NAL units in order to understand whether they are needed in the extracted sub-stream, or (much simpler) just include all those NAL units in the bitstream, which causes unnecessary bit rate overhead for the extracted bitstream.

The proposed NAL unit header has an option to include information about scalable layer associations in any NAL unit, thus enabling association of both VCL and non-VCL NAL units with scalable layers.

Signaling of required decoder capabilities for temporal scalability

In the current WD, required decoder capabilities can be signaled in the SPS via syntax elements such as profile_idc and level_idc. When one or several of the highest temporal enhancement layers are removed from the bitstream, then the required decoder capabilities such as the required level may decrease. For instance, when a temporal enhancement layer is discarded thus reducing the frame rate of the decoded video from 60 fps to 30 fps, the number of pixels decoded per second is reduced. Decoders that would not be able to decode the bitstream at 60 fps frame rate may be able to do so if the frame rate is reduced to 30 fps. Thus it may be useful to signal required decoder capabilities for sub-streams of temporally scalable bitstreams. However currently there is no way to do so.

The proposed scheme can be used to signal required decoder capabilities for each (temporal) layer that can be extracted from the bitstream.

NAL unit header when no bitstream scalability is required

In the current WD, the temporal_id and thus the second NAL unit header byte is signaled for coded slices, even if the application does not require any bitstream scalability (that is, in applications where temporal_id=0 can be used for all coded slices). In such cases, the second NAL unit header byte causes a small but unnecessary bitrate overhead (around 0.067% for 1500 byte NAL units).

The proposed NAL unit header allows omitting the second NAL unit header byte in case there is no need to reveal any information about scalable layers to the network.

# Proposed Basic Design

The proposed design has two major elements:

- Optional signaling of SPS identifier seqParameterSetId in the NAL unit header
(note that seqParameterSetId corresponds to seq_parameter_set_id in the current WD)

If present in the NAL unit header, seqParameterSetId serves as a generic scalable layer identifier. NAL units with the same value of seqParameterSetId in their headers together define a subset of the bitstream, which represents the respective scalable layer. The properties of that scalable layer are described by the associated SPS, which is carried in an SPS NAL unit with the same value of seqParameterSetId. seqParameterSetId may be present in both VCL and non-VCL NAL units, which makes it possible to associate any type of NAL unit with a certain scalable layer. Additionally, seqParameterSetId co-serves as a priority identifier, which makes it possible for network nodes to perform simple bitstream thinning operations.

Figure 1c illustrates an example of a NAL unit stream where each NAL unit is associated with an individual value of seqParameterSetId, and NAL units belonging to the same scalable layers are indicated through identical values of seqParameterSetId. The actual values of seqParameterSetId indicate the relative priority of the associated scalable layer, where seqParameterSetId=0 indicates the most important layer.

- Extension of the SPS syntax structure to specify dependencies between scalable layers

In the extended syntax, an SPS that is associated with a base layer (i.e. a scalable layer that does not depend on any other scalable layers) is very similar to the SPS design in the current WD, and it is denoted as “base SPS”.

An SPS that is associated with an enhancement layer (i.e., a scalable layer that has decoding dependencies with respect to other scalable layers) is used to signal decoding dependencies as well as scalable layer properties. It is denoted as “enhancement SPS”. Decoding dependencies of a scalable layer associated with a certain enhancement SPS with respect to other scalable layers are signaled by means of references to SPSs that are associated with those other scalable layers (can be either enhancement SPSs or base SPSs). Each enhancement SPS refers to one base SPS, either directly or indirectly through other referenced enhancement SPSs.

Figure 1a illustrates an example of such dependency relationship. The base SPS does not refer to any other scalable layers, thus indicating that the associated base layer video can be independently decoded. A first enhancement SPS refers directly to the base SPS, which indicates that NAL units associated with both the base SPS and first enhancement SPS are required for decoding the associated video representation. A second enhancement SPS refers to the first enhancement SPS, and NAL units associated with all three SPSs are required for decoding of the associated video representation. The base SPS in this example serves as base SPS to both enhancement SPSs.

Each coded sequence contains one base SPS that represents the base layer of the coded video sequence. Enhancement layers are represented by respective enhancement SPSs. An enhancement SPS contains a reference to its respective reference SPS, as well as information that is particular for the respective layer, such as temporal_id and level_idc. Syntax elements that are not present in the enhancement SPS are inherited from its directly and indirectly referenced SPSs.

In the current WD, only temporal scalability is supported. With the proposed design, when further types of scalability are defined in future extensions to HEVC, the NAL unit header can remain unchanged while information that is specific for the specified type of enhancement layers (such as e.g. view_id for multiview coding) can be added in the enhancement SPS. An example of such extension is outlined in Sect. 4.

Figure 1: Example of proposed design when three temporal layers are used according to “low delay” common coding condition. (a) One base SPS and two enhancement SPSs, with references between the SPSs indicating decoding dependencies. (b) Inter-prediction structure with three temporal layers, each layer being associated with one SPS (note: a simplified prediction structure is depicted). (c) Corresponding NAL unit stream, indicating the value of seqParameterSetId and the NAL unit type for each NAL unit.

Figure 1 illustrates an example with three temporal layers according to “low delay” common coding condition. Each temporal layer has one associated SPS. The base SPS (seqParameterSetId=0) represents the temporal base layer (temporal_id=0), and it carries all syntax elements that the SPS in the current WD would carry. The first enhancement SPS (seqParameterSetId=1) refers to the base SPS by using a reference identifier (temporalRefSpsId=0). Compared to the referenced base SPS, the first enhancement SPS overrides syntax elements that are specific to its associated scalable layer (such as temporal_id and level_idc), while other syntax elements are inherited from the base SPS. Likewise, the second enhancement SPS (seqParameterSetId=2) refers to the first enhancement SPS using a reference identifier (temporalRefSpsId=1), overrides specific syntax elements, and otherwise inherits the syntax elements from the first enhancement SPS (and thus for some syntax elements, indirectly, from the base SPS).

Note that the SPSs in the example have different values of level_idc associated with their respective scalable layer, which reflects the fact that the three layers represent a hierarchy of different frame rates (e.g. 15 fps, 30 fps and 60 fps), and the first enhancement layer (e.g. 30 fps video) has higher decoder requirements than the base layer (e.g. 15 fps video), and the second enhancement layer (e.g. 60 fps video) has higher decoder requirements than the first enhancement layer.

Note also that non-VCL NAL units (SPS and PPS) are linked to the respective scalable layers. If for instance the sub-bitstream associated with the first enhancement layer is to be extracted, all NAL units associated with the first enhancement SPS (seqParameterSetId=1) and, as indicated through its reference identifier, all NAL units associated with the base SPS (seqParameterSetId=0) should be included. This includes the PPS (which is associated with the base layer) and the two SPSs associated with the base layer and the first enhancement layer. Similarly, SEI messages can be associated with scalable layers, and thus be considered appropriately in the extraction process.

# Proposed Changes

The proposed changes are described in detail below. Essential changes apply to the NAL unit header and SPS syntax definitions and associated semantics. Further changes apply to the picture parameter set and slice header syntax definitions. Furthermore, an update to the SPS activation process is provided, and a bitstream extraction process description is provided.

## NAL Unit Header

The variable seqParameterSetId is introduced as a generic layer identifier and priority indicator in the NAL unit header. The presence of seqParameterSetId is optional, indicated by the seq_parameter_set_present_flag syntax element. Note that compared to the current WD, according to the decisions in the Torino meeting, the nal_ref_idc syntax element has been renamed to nal_ref_flag and its length reduced from 2 bits to 1 bit. The spare bit is used for the seq_parameter_set_present_flag syntax element.

seq_parameter_set_present_flag equal to 0 (i.e. seqParameterSetId is not defined) indicates that the NAL unit is structurally linked to the base layer (see extraction process Sect. 3.6). In applications where no scalability is required, all NAL units can be linked to the base layer, and thus seqParameterSetId can be omitted for all NAL units except SPS and PPS. For SPS and PPS, seqParameterSetId is mandatory as it replaces the syntax element seq_parameter_set_id present inside the SPS/PPS syntax structures in the current WD (see Sects. 3.2 and 3.3). Compared to the current WD, the NAL unit header for VCL NAL units is simplified to be a one-byte header, which leads to a bitrate reduction of around 0.067% for 1500 byte NAL units.

If present, seqParameterSetId is coded using a byte-aligned variable length code, so as to cover potential future scalability extensions with a basically unlimited number of layers. Practically some limitation on the code length is certainly necessary, and corresponding bitstream constraints can be imposed through profile definitions.

The syntax elements temporal_id and output_flag that are present for coded slices in the current WD have been removed from the NAL unit header. The temporal_id has been moved to the sequence parameter set, see Sect. 3.2. The output_flag has been moved to the slice header, see Sect. 3.4.

### Syntax

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_flag | u(1) |
| nal_unit_type | u(5) |
| seq_parameter_set_id_present_flag | u(1) |
| nalUnitHeaderBytes = 1 |  |
| if( seq_parameter_set_id_present_flag = = 1 ) { |  |
| seqParameterSetId = 0 |  |
| do { |  |
| sps_id_7bits | u(7) |
| seq_parameter_set_id_extension_flag | u(1) |
| seqParameterSetId = seqParameterSetId * 128 + sps_id_7bits |  |
| nalUnitHeaderBytes += 1 |  |
| } while( seq_parameter_set_id_extension_flag = = 1 ) |  |
| } |  |
| if( nal_unit_type = = 1 ¦ ¦ nal_unit_type = = 4 ¦ ¦ nal_unit_type = = 5 ) { |  |
| temporal_id | u(3) |
| output_flag | u(1) |
| reserved_one_4bits | u(4) |
| nalUnitHeaderBytes += 1 |  |
| } |  |
| NumBytesInRBSP = 0 |  |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

### Semantics

seq_parameter_set_id_present_flag equal to 1 indicates that the variable seqParameterSetId is calculated from one or more instances of the syntax element sps_id_7bits present in the NAL unit header. seq_parameter_set_id_present_flag equal to 0 indicates that no instances of the syntax element sps_id_7bits are present in the NAL unit header, and that the variable seqParameterSetId is undefined. If seq_parameter_set_id_present_flag is equal to 0 and nal_unit_type is equal to 1, 4 or 5, then the value of temporal_id corresponding to the coded slice in the NAL unit is inferred to be 0. If nal_unit_type is equal to 7 or 8, then the value of seq_parameter_set_id_present_flag shall be equal to 1.

sps_id_7bits specifies 7 bits of the sequence parameter set identifier.

seq_parameter_set_id_extension_flag equal to 1 indicates that more bits are needed to represent the sequence parameter set identifier in the NAL unit header. If seq_parameter_set_id_extension_flag is equal to 0, no further bits are needed.

The variable seqParameterSetId specifies the sequence parameter set identifier. It also specifies a priority identifier for the NAL unit.

## Sequence Parameter Set

The proposed SPS design has two modes, the “base SPS” mode and the “enhancement SPS” mode.

In the “base SPS” mode, the SPS contains essentially the same syntax elements that the SPS in the current WD contains. In the syntax table below, temporal_dependency_flag equal to 0 indicates the “base SPS” mode. As can be seen in the function base_seq_parameter_set_rbsp( ) below, the syntax element seq_parameter_set_id is not conveyed in the SPS syntax structure, since the SPS identifier is conveyed as seqParameterSetId in the NAL unit header instead (seqParameterSetId must be present for SPS NAL units, see Sect. 3.1). The “base SPS” is associated with the base layer of the coded sequence.

temporal_dependency_flag equal to 1 indicates the “enhancement SPS” mode. One enhancement SPS is associated with each temporal enhancement layer (note that in the same way, one enhancement SPS can be associated with each e.g. spatial layer, SNR layer, or view in future scalability extensions to HEVC, see example in Sect. 4). Each enhancement SPS has a reference SPS, the identifier of which is indicated by the variable temporalRefSpsId. The enhancement SPS infers the values of all syntax elements that are present or inferred in its reference SPS but not present in the enhancement SPS. The values level_idc and temporal_id in the reference SPS are overridden by the values present in the enhancement SPS (see example illustration in Figure 1a). The value of profile_idc is identical across temporal layers so as to enable simple parameter set activation similar to the parameter set activation in the current WD (see Sect. 3.5). While it does not appear to be a strong restriction to have identical profiles for different temporal layers of the same bitstream, it is likely that future scalability extensions to HEVC will benefit from use of different profiles across scalable layers. For such extensions, the constraints on profile_idc may be relaxed, so that enhancement SPSs associated with new scalability mechanisms and corresponding scalability signaling can be indicated through new values of profile_idc (see example in Sect. 4).

The hierarchical relationships of enhancement SPSs and base SPSs as defined by the temporalRefSpsId reflect the decoding relationships in the coded video sequence. Using an enhancement SPS as “entry point” to a coded video sequence, the reference layers needed for decoding can be determined by following the respective variables temporalRefSpsId until a base SPS is reached. Thus each enhancement SPS has an associated base SPS that it references either directly or indirectly (see example illustration in Figure 1a, where both enhancement SPSs share the same base SPS).

### Syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| temporal_dependency_flag | u(1) |
| if( temporal_dependency_flag = = 1 ) { |  |
| temporalRefSpsId = 0 |  |
| do { |  |
| ref_sps_id_7bits | u(7) |
| ref_sps_id_extension_flag | u(1) |
| temporalRefSpsId = temporalRefSpsId * 128 + ref_sps_id_7bits |  |
| } while( ref_sps_id_extension_flag = = 1 ) |  |
| temporal_id | u(3) |
| } else { |  |
| base_seq_parameter_set_rbsp( ) |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| base_seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| pcm_loop_filter_disable_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| inter_4x4_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

### Definitions

- Base sequence parameter set (base SPS): A sequence parameter set that does not refer to any reference sequence parameter sets.

- Enhancement sequence parameter set (enhancement SPS): A sequence parameter set that refers to one reference sequence parameter set as identified through its inferred variable temporalRefSpsId. For a given enhancement sequence parameter set, the values of all syntax elements that are present in or inferred by its reference sequence parameter set but not present in the enhancement sequence parameter set are inferred from the reference parameter set. Each enhancement SPS has an associated base SPS that it refers to either directly or indirectly.

- Reference sequence parameter set (reference SPS): A sequence parameter set identified through the inferred variable temporalRefSpsId in an enhancement sequence parameter set.

### Semantics

profile_idc indicates the profile to which the coded video sequence associated with the current sequence parameter set conforms. The value of profile_idc shall be equal to the value of the associated base SPS (note: this restriction may be relaxed when bitstream scalability mechanisms other than temporal scalability are introduced).

reserved_zero_8bits shall be equal to 0. Decoders shall ignore the value of reserved_zero_8bits.

level_idc indicates the level to which the coded video sequence associated with the current sequence parameter set conforms. The value of level_idc overrides the value of level_idc present in the reference sequence parameter set for the current sequence parameter set.

- temporal_dependency_flag equal to 1 specifies that a reference to a reference sequence parameter set is present. temporal_dependency_flag equal to 0 specifies that no reference to a reference sequence parameter set is present.

ref_sps_id_7bits specifies 7 bits of the sequence parameter set identifier that identifies the reference sequence parameter set.

ref_sps_id_extension_flag equal to 1 indicates that more bits are needed to represent the sequence parameter set identifier of the reference sequence parameter set. If ref_sps_id_extension_flag is equal to 0, no further bits are needed.

temporal_id specifies a temporal identifier for the NAL units that refer to the current sequence parameter set. The value of temporal_id shall be the same for all NAL units of an access unit.

## Picture Parameter Set

The syntax element seq_parameter_set_id is removed from the PPS since the sequence parameter set identifier seqParameterSetId is carried in the NAL unit header of the PPS NAL unit instead (seqParameterSetId must be present for PPS NAL units, see Sect. 3.1).

### Syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Slice Header

The syntax element output_flag is moved from the second byte of the NAL unit header to the slice header. The semantics are copied from the current WD.

### Syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| output_flag | u(1) |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| … |  |

### Semantics

output_flag affects the decoded picture output and removal processes as specified in Annex Error: Reference source not found.

## Parameter Set Activation

Compared to the WD, no changes are needed for PPS activation. The only change for the PPS is that the sequence parameter set identifier is conveyed in the NAL unit header (seqParameterSetId) instead of the PPS syntax structure, see Sect. 3.3.

Following the proposed SPS syntax structure modifications introducing “base SPS” and “enhancement SPS” modes (Sect. 3.2), SPS activation is slightly changed compared to the WD. In the same way as in the WD, SPS activation is triggered when an SPS is referred to by activation of a PPS (or referred to by a buffering period SEI message). If the referred SPS is a base SPS, then it is activated as usual. If the referred SPS is an enhancement SPS, then the associated base SPS is activated instead. This way only a single SPS is active at the same time, in the same sense as in the WD. The required changes are indicated in Sect. 3.5.1.

Note that in future scalability extensions to HEVC, it may be beneficial to allow that multiple SPSs are active at the same time, such as one SPS for each scalable layer (for instance for spatial scalability or multiview coding such as in SVC and MVC). In the context of temporal scalability as is supported in the current WD, it does not appear to be necessary that multiple SPSs are active at the same time.

### Semantics for Sequence Parameter Set Activation

- A sequence parameter set RBSP includes parameters that can be referred to by one or more picture parameter set RBSPs or one or more SEI NAL units containing a buffering period SEI message. Each sequence parameter set RBSP is initially considered not active at the start of the operation of the decoding process. At most one sequence parameter set RBSP is considered active at any given moment during the operation of the decoding process, and the activation of any particular sequence parameter set RBSP results in the deactivation of the previously-active sequence parameter set RBSP (if any).

- When a base sequence parameter set RBSP (with a particular value of seqParameterSetId) is not already active and it is referred to by activation of a picture parameter set RBSP (using that value of seqParameterSetId) or is referred to by activation of a picture parameter set RBSP using a value of seqParameterSetId that refers to an enhancement sequence parameter set RBSP that has an associated base sequence parameter set RBSP with that value of seqParameterSetId or is referred to by an SEI NAL unit containing a buffering period SEI message (using that value of seqParameterSetId), it is activated. This base sequence parameter set RBSP is called the active sequence parameter set RBSP until it is deactivated by the activation of another base sequence parameter set RBSP. A base sequence parameter set RBSP, with that particular value of seqParameterSetId, shall be available to the decoding process prior to its activation. An activated base sequence parameter set RBSP shall remain active for the entire coded video sequence.

## Bitstream Extraction

In the proposed design, each SPS represents one scalable layer of the bitstream, and the syntax elements in the SPS describe the properties of the scalable layer and, through the proposed mechanism to reference other SPSs, decoding dependencies towards other scalable layers.

By parsing available SPSs for a coded video sequence, a decoder or network node can understand layer properties and dependencies in the bitstream. In order to extract a certain scalable representation (e.g. conforming to a certain level), a decoder or network node can select an SPS that suits the requirements (i.e. an SPS with a certain value of level_idc) and use that as an “entry point” to the bitstream. Starting from the “entry point” SPS, following the respective reference SPSs until the base SPS is reached, a set of sequence parameter set identifiers can be determined that represents all the scalable layers that are required to decode the desired video representation.

Using a set of relevant sequence parameter set identifiers, it is straightforward to extract the associated NAL units from the bitstream: A NAL unit is extracted from the bitstream if its seq_parameter_set_id_present_flag is equal to 0 or if its seq_parameter_set_id_present_flag is equal to 1 and the given value of seqParameterSetId is present in the set of relevant sequence parameter set identifiers. Semantics for such bitstream extraction are given in Sect. 3.6.1.

Note that in order to obtain a set of relevant sequence parameter set identifiers, a network node needs to obtain and parse the relevant SPS NAL units. If it should not be possible for the network node to obtain or parse SPS NAL units, an alternative simple extraction mechanism is facilitated by interpreting the variable seqParameterSetId in the NAL unit header as a priority indicator. A simple network node can use the priority indications to select NAL units that are to be discarded.

### Semantics for Bitstream Extraction

Input to the bitstream extraction process is a set of sequence parameter set identifiers, denoted by the variable seqParameterSetIdSet.

Output of the process is an extracted bitstream.

The extracted bitstream is derived by removing all NAL units from the bitstream for which the following condition is true:

- The syntax element seq_parameter_set_id_present_flag is equal to 1 and the value of the variable seqParameterSetId is not present in seqParameterSetIdSet.

# Example for Extensibility: Incorporation of View Scalability

The proposed design allows for introduction of high-level scalability signaling mechanisms for future scalability extensions to HEVC, without requiring any changes to the NAL unit header definitions. If a new type of scalability is introduced, only the definition of the sequence parameter set needs to be updated to enable signaling of the new additional scalability type.

An example of such extension in given in the syntax table in Sect. 4.1. Compared to the proposed SPS definition in Sect. 3.2, new syntax elements are introduced to indicate that the current scalable layer has a dependency towards another view. If such dependency is present, the identifier of the reference SPS (viewRefSpsId) and the view identifier (view_id) for the current view are signaled. The respective syntax elements are only present if the profile_idc in the SPS indicates that the scalable layer conforms to an imaginary “multiview profile” (that would have to be defined in the respective future HEVC extension). Note that the “base view” in the bitstream could conform to a profile that is different from the imaginary “multiview profile”, thus providing an entry point for decoders that do not support the “multiview profile”, i.e. backwards compatibility.

HM-based software implementation of the proposed signaling and bitstream extraction are available as part of Ericsson’s response to MPEG’s Call for Proposals on 3D Video Coding Technology.

## Sequence Parameter Set Syntax for View Scalability

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| baseSeqParameterSetFlag = 1 |  |
| temporal_dependency_flag | u(1) |
| if( temporal_dependency_flag = = 1 ) { |  |
| temporalRefSpsId = 0 |  |
| do { |  |
| ref_sps_id_7bits | u(7) |
| ref_sps_id_extension_flag | u(1) |
| temporalRefSpsId = temporalRefSpsId * 128 + ref_sps_id_7bits |  |
| } while( ref_sps_id_extension_flag = = 1 ) |  |
| temporal_id | u(3) |
| baseSeqParameterSetFlag = 0 |  |
| } |  |
| if( profile_idc = = multiview_profile ) { |  |
| view_dependency_flag | u(1) |
| if( view_dependency_flag = = 1 ) { |  |
| viewRefSpsId = 0 |  |
| do { |  |
| ref_sps_id_7bits | u(7) |
| ref_sps_id_extension_flag | u(1) |
| viewRefSpsId = viewRefSpsId * 128 + ref_sps_id_7bits |  |
| } while( ref_sps_id_extension_flag = = 1 ) |  |
| view_id | u(10) |
| baseSeqParameterSetFlag = 0 |  |
| } |  |
| } |  |
| if( baseSeqParameterSetFlag = = 1 ) { |  |
| base_seq_parameter_set_rbsp( ) |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).