| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 Nov, 2011 | Document: JCTVC-G608 WG11 Number: m22174 |
| --- | --- |

| Title: | Unified Deblocking and SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrew Segall and Jie Zhao 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | Tel: +1-360-817-8487 asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

In current HEVC test model HM4.0, there are three in-loop filtering processes ― deblocking filter, Sample Adaptive Offset (SAO) and Adaptive Loop filter (ALF). They are processed sequentially. SAO is applied to the debocked pictures. Due to dependency on future deblocked pixels, SAO can not be done immediately after a pixel is fully deblocked. This makes it hard to integrate SAO into parallel deblocking process.

This contribution proposes a simple modification to SAO classification process ― use horizontal deblocked pixels instead of fully deblocked pixels for EO type SAO classification. This eliminates the dependency of SAO of a pixel on future deblocked pixels, and makes it possible to integrate SAO into parallel deblocking process. On average, there is only about 0.1% rate increase on coding performance.

# Introduction

# Deblocking filter in HM4.0 applies filtering on 8x8 or larger block edges. It performs horizontal deblocking on vertical edges first, and then performs vertical deblocking on horizontal edges. For a block edge, 4 pixels on each side are used in making deblocking decision, and up to 3 pixels on each side of the edge may be modified. All vertical edges can be deblocked in parallel and all horizontal edges can be deblocked in parallel. Vertical deblocking use the horizontal deblocking output as input. After deblocking, SAO is applied to the final deblocked pixels.

# The process of SAO is to add offsets to reconstructed pixels (deblocked pixels) and make them closer to original. A picture is divided into multiple LCU-aligned regions, and each region is assigned 1 out of 6 SAO types or no SAO processing based on pixel characteristics. Pixels in a region are further classified into different categories based on their characteristic. The SAO type of each region and the offsets of each category are derived at encoder and sent in bitstream. Decoder receives the SAO information. It performs the SAO classification process to find out which category a pixel belongs to and obtain its offset. The offset is then added to the deblocked pixel to generate SAO output pixel.

# There are six SAO types – 2 Band Offset (BO) types and 4 Edge Offset (EO) types. BO (Band offset) classifies all pixels of a region into multiple bands where each band contains pixels of the same intensity interval and has a derived offset. Each BO type consists of 16 categories. The category is determined by the intensity of a pixel alone. There are four EO (edge offset) types ― 0-degree EO, 45-degree EO, 90-degree EO, 135-degree EO, and no processing. Each EO type consists of 4 categories. Depending on the EO type, two neighbor pixels are selected. These are marked as the blue box in the Figure 1 below. The classification method of EO type is shown in the table 1.

| (a) 0-degree (b) 90-degree (c) 135-degree (d) 45 degree Figure 1. EO types and neighbor pixels used in EO classification | Table 1. classification method of EO type |
| --- | --- |

# Problem Statement

# If an SAO offset is known, SAO process is simply an add operation, it would be desirable to be added to a pixel while the pixel is deblocked. The benefits are

# Unify the SAO and deblocking process

# Reduce complexity by avoiding a separate pass of SAO processing and reduce memory access.

# Enable parallelization. SAO can be done together with parallel vertical deblocking.

# But problem of current SAO design is, for a region of EO type, the neighbor pixel needed for SAO category classification may have not been deblocked yet. Therefore, decoder can not determine its SAO offset, so it can not add SAO offset immediately after a pixel is debocked. This makes SAO difficult to be combined into parallel deblocking process.

# Fig. 2 illustrates such kind of dependency issue on the neighbor pixels. For example, if a region has EO type 135 degree, SAO process of the red pixel in the Fig. 2 needs the two yellow pixels in columns to the left and right to determine the SAO category and then determine its offset. If these columns are sequentially processed, when the red pixel is just deblocked, the yellow pixel to the right has not been deblocked yet.  If these columns are parallel deblocked, both neighbors marked by yellow may have not be deblocked yet. Therefore SAO classification can not be done immediately after a pixel is deblocked if the region has an EO type. For region of BO type, since SAO classification only requires current pixel, it does not have the above dependency problem.

Fig. 2 Illustrations of SAO dependency on neighboring deblocked pixels

# Proposed Method

# We propose, for an SAO region of EO type, use pixels after horizontal deblocking in the SAO category classification. For the offset adding step, the offset is still added to the fully deblokced pixels. For BO type, no modification needed, and its SAO classification can still use fully deblocked pixels. Once the category of a pixel is known, SAO offset can be obtained. The offset can then be added to the vertical deblocked pixel immediately after it is deblocked.

Figures below illustrate the difference of the proposed unified SAO and deblocking to HM4.0. Fig. 3a is the flowchart of what is currently implemented in HM4.0. Here, a picture is horizontal deblocked first, secondly it is vertically deblocked, and then it goes through the SAO process. This Fig 3b shows the flowchart of the proposed method, where SAO is integrated into the vertical deblocking, and it reduces the number of passes to access the frame buffer.

With such simple modification, it eliminates the dependency of SAO on future deblocked pixels, and therefore SAO can be integrated into the parallel vertical deblocking process. This unifies the deblocking and SAO process, and simplifies the overall in-loop filtering process.

| Fig. 3a Flow chart of deblocking and SAO processing in HM4.0 at frame level | Fig. 3b Flow chart of proposed deblocking and SAO processes at frame level |
| --- | --- |

# Results

We implemented the proposed method of using horizontal deblocked pixel for EO type SAO classification into HM4.0, and tested using common test condition [4]. As can be seen from table 2 below, there is only a small rate increase of about 0.1% on average.

Table 2. Results of modified SAO classification vs. HM4.0

# Conclusion

We proposed to use horizontal deblocked pixels for EO type SAO classification. This makes SAO able to be immediately added to final deblocked pixels, so that SAO can be integrated into parallel deblocking process. The proposed method has only about 0.1% rate increase.

Given the simplicity and the important benefit of this modified SAO classification approach to unify SAO with deblocking filter, we propose the JCT-VC adopt these changes into HM.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCTVC-E049, “CE13: Sample Adaptive Offset with LCU-Independent Decoding,” 5th JCT-VC Meeting, Geneva, March. 2011.

- JCTVC-F800d4, “WD4: Working Draft 4 of High-Efficiency Video Coding,” 6th JCT-VC meeting, Torino, July. 2011.

- HM4.0, http://hevc.kw.bbc.co.uk/svn/jctvc-hm/

- JCTVC-F900, “Common test conditions and software reference configurations,” 6th JCT-VC meeting, Torino, July. 2011.