| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G609 |
| --- | --- |

| Title: | CE8.a.2: Directional feature calculation on subset of pixels |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 Takashi Watanabe1 Takeshi Chujoh1 Ching-Yeh Chen2 Chih-Ming Fu2 Chia-Yang Tsai2 Yu-Wen Huang2 Shawmin Lei2 Marta Karczewicz3 In Suk Chong3 Tomonori Ikai4 Andrew Segall4 Tomoyuki Yamamoto4 Yukinobu Yasugi 4 | Email: | tomoo.yamakage@toshiba.co.jp takashi39.watanabe@toshiba.co.jp takeshi.chujoh@toshiba.co.jp chingyeh.chen@mediatek.com chihming.fu@mediatek.com chiayang.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com martak@qualcomm.com ichong@qualcomm.com ikai.tomohiro@sharp.co.jp asegall@sharplabs.com yamamoto.tomoyuki@sharp.co.jp yasugi.yukinobu@sharp.co.jp |
| Source: | TOSHIBA Corporation1, MediaTek Inc.2, Qualcomm3 and Sharp4 |  |  |

_____________________________

# Abstract

This evaluates a directional feature calculation on subset of pixels to improve coding efficiency. This technique uses the internal and the surrounding pixels (window pixels) of the 4x4 block unit which have different complexity and performance trade off. It also evaluates the changes in computational direction to diagonal directions from horizontal/vertical directions. Coding efficiency gain for luma is 0.1 %, 0.2 %, 0.2% and 0.4 % in HE-AI, RA, LB, and LP without encoding/decoding time increase on average. When merged with CE8.a.1 the coding efficiency is improved by is 0.2 %, 0.3 %, 0.3% and 0.4 % in HE-AI, RA, LB, and LP.

# Introduction

In the previous meeting, new block adaptation (BA) mode computation with less complexity was adopted which uses only subset of points to compute BA metric. That proposed BA computation requires to access 5x5 pixels. In this proposal, new BA mode computation based on different subset of points are proposed, which  requires less number of pixels in vertically in order to mitigate line buffer requirements in ALF implementation.

And filter shape dependent BA mode computation is proposed to exploit inherent dependencies between filter shapes and BA directionality.

# Description

## BA mode computations

- BA mode computation in HM4.0

This BA mode computation requires to access pixels of 5x5 windows with 1 additions/pixel.

- BA mode computation with 4 points

This BA mode computation requires to access pixels of 4x4 windows with 0.625 additions/pixel.

- BA mode computation with 8 points

This BA mode computation requires to access pixels of 6x4 windows with 1.25 additions/pixel.

## Filter shape dependent BA mode computations

Depending on the filter shapes, BA mode direction computation can be changed. Cross shape inherently follows vertical/horizontal (BA)  direction. And star shape can follow either vertical/horizontal or diagonal direction. Therefore we tied filter shapes with directionality.

## Other configurations testing

We tested additionally following conditions.

- Condition 1: merge with CE8.a.1

# Changes in Working Draft Text

- 7.3.3.5	Adaptive loop filter parameter syntax

| alf_param() { | C | Descriptor |
| --- | --- | --- |
| adaptive_loop_filter_flag | 2 | u(1) |
| if ( adaptive_loop_filter_flag ) { |  |  |
| alf_region_adaptation_flag | 2 | u(1) |
| alf_filter_shape | 2 | u(1) |
| alf_no_filters_minus1 | 2 | ue(v) |
| if (alf_no_filters_minus1 == 1) |  |  |
| alf_start_second_filter | 2 | ue(v) |
| else if (alf_no_filters_minus1 > 1) { |  |  |
| for (i=1; i< 16; i++) |  |  |
| alf_filter_pattern[i] | 2 | u(1) |
| } |  |  |
| ... |  |  |

- 8.6.6.6	Derivation process for filter index array for luma samples

…

…

- When alf_region_adaptation_flag is equal to 1, the following ordered steps apply.

- The variables xIdx and yIdx are derived as

regionTab[16] = { 0, 1, 4, 5, 15, 2, 3, 6, 14, 11, 10, 7, 13, 12, 9, 8 }	(8-1)
	offset = 1 << (Log2MaxCUSize – 1) 		(8-2)
	xInterval = ( ( ( PicWidthInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2	(8-3)
	yInterval = ( ( ( PicHeightInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2	(8-4)
	xIdx = Min( 3, Floor( ( xC + x ) / ( xInterval << Log2MaxCUSize ) ) )	(8-5)
	yIdx = Min( 3, Floor( ( yC + y ) / ( yInterval << Log2MaxCUSize ) ) )	(8-6)

- The filter index fIdx[ x, y ] with x, y = 0..(nS)-1 is derived as

fIdx[ x ][ y ] = regionTab[ ( yIdx << 2 ) + xIdx ]		(8-7)

- Otherwise (alf_region_flag is equal to 0), the following ordered steps apply.

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ] and varTemp1[ x ][ y ] with x, y = 0,2,4,...(nS)-1is derived as

// for filter shape dependent BA mode

if alf_filter_shape is equal to 0,

varTempH[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x-1, yC+y-1 ] – s’’[ xC+x+1, yC+y+1 ] |	(8-8)
		varTempV[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x+1, yC+y-1 ] – s’’[ xC+x-1, yC+y+1 ] |	(8-9)

otherwise

varTempH[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x-1, yC+y ] – s’’[ xC+x+1, yC+y ] |	(8-10)
		varTempV[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x, yC+y-1 ] – s’’[ xC+x, yC+y+1 ] |	(8-11)

- The variables varTemp3[ x, y ], varTempH1[ x, y ] and varTempV1[ x, y ] with x, y = 0..( (nS) – 1 )>>2 are derived as

//  for 4 points

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 1, 2 	(8-12)
	varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 1, 2 	(8-13)
 	varTemp3[ x ][ y ] = (varTempH1[ x ][ y ] + varTempV1[ x ][ y ]) >> 2	(8-14)

//  for 8 points

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i = 0..3, j = 1, 2 	(8-15)
	varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i = 0..3, j = 1, 2 	(8-16)
 	varTemp3[ x ][ y ] = (varTempH1[ x ][ y ] + varTempV1[ x ][ y ]) >> 3	(8-17)

# Simulation results

- BA mode computation with 4 points

Table 1 shows BD-rate results for the proposed 4 points BA mode.

Table 1. BD-rate results

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.1% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.1% | -0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.1% |
| Class E |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | -0.1% | 0.0% |
| Class E | 0.1% | -0.3% | 0.7% |
| Overall | 0.0% | -0.1% | 0.2% |
|  | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% |
| Class C | 0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.2% | -0.4% |
| Class E | -0.1% | -0.5% | 0.2% |
| Overall | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

- BA mode computation with 8 points

Table 2. BD-rate results

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% |
| Class B | -0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% | 0.0% | 0.0% |
| Overall | -0.1% | 0.0% | 0.0% |
|  | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.1% | -0.2% | -0.1% |
| Class B | -0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.0% |
| Class E |  |  |  |
| Overall | -0.1% | 0.0% | 0.0% |
|  | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.1% | 0.0% | -0.2% |
| Class C | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | 0.4% | 0.3% |
| Class E | 0.0% | -0.3% | 0.6% |
| Overall | -0.1% | 0.0% | 0.1% |
|  | -0.1% | -0.1% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.1% | 0.0% | 0.2% |
| Class C | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | 0.1% | -0.1% |
| Class E | -0.1% | 0.0% | 0.0% |
| Overall | -0.1% | 0.0% | 0.0% |
|  | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 102% |  |  |

- BA mode computation with 8 points + filter shape dependent BA mode

Table 3. BD-rate results

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | -0.1% | 0.0% | 0.0% |
| Class C | -0.2% | 0.0% | 0.0% |
| Class D | -0.2% | 0.1% | 0.1% |
| Class E | -0.2% | 0.0% | 0.0% |
| Overall | -0.1% | 0.0% | 0.0% |
|  | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.2% | -0.1% | -0.2% |
| Class B | -0.2% | -0.1% | 0.0% |
| Class C | -0.2% | -0.1% | 0.0% |
| Class D | -0.3% | 0.0% | 0.1% |
| Class E |  |  |  |
| Overall | -0.2% | -0.1% | 0.0% |
|  | -0.2% | -0.1% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.2% | -0.1% | -0.2% |
| Class C | -0.2% | -0.1% | -0.1% |
| Class D | -0.2% | 0.4% | -0.1% |
| Class E | -0.3% | -0.3% | 0.4% |
| Overall | -0.2% | 0.0% | 0.0% |
|  | -0.2% | -0.1% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.3% | 0.0% | 0.0% |
| Class C | -0.3% | 0.1% | 0.0% |
| Class D | -0.4% | 0.1% | -0.5% |
| Class E | -0.6% | -0.5% | 0.0% |
| Overall | -0.4% | 0.0% | -0.1% |
|  | -0.4% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

- Merging (3) with CE8.a.1

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% |
| Class B | -0.1% | 0.0% | 0.0% |
| Class C | -0.2% | 0.0% | 0.0% |
| Class D | -0.2% | 0.1% | 0.1% |
| Class E | -0.2% | 0.0% | 0.0% |
| Overall | -0.2% | 0.0% | 0.0% |
|  | -0.2% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.2% | 0.0% | 0.0% |
| Class B | -0.3% | 0.0% | -0.1% |
| Class C | -0.3% | 0.0% | 0.1% |
| Class D | -0.3% | 0.2% | 0.2% |
| Class E |  |  |  |
| Overall | -0.3% | 0.0% | 0.0% |
|  | -0.3% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.2% | 0.0% | -0.1% |
| Class C | -0.3% | 0.2% | -0.1% |
| Class D | -0.3% | 0.2% | 0.2% |
| Class E | -0.4% | 0.8% | 0.3% |
| Overall | -0.3% | 0.2% | 0.1% |
|  | -0.3% | 0.2% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.4% | 0.1% | 0.1% |
| Class C | -0.4% | -0.1% | 0.0% |
| Class D | -0.4% | 0.1% | -0.4% |
| Class E | -0.7% | 0.0% | 0.1% |
| Overall | -0.4% | 0.0% | -0.1% |
|  | -0.4% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusions

In this proposal, the coding efficiency is improved by adopting new BA mode computation and filter shape dependent BA mode computation. Coding efficiency gain for luma is 0.1 %, 0.2 %, 0.2% and 0.4 % in HE-AI, RA, LB, and LP. There was no encoding/decoding time increase on average. When merged with CE8.a.1 the coding efficiency is improved by is 0.2 %, 0.3 %, 0.3% and 0.4 % in HE-AI, RA, LB, and LP.

# Patent rights declaration(s)

TOSHIBA Corporation, MediaTek Inc., Sharp Corporation, and Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).