| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G613 WG11 Number: m22179 |
| --- | --- |

| Title: | CE8 Subset d.2: Cross-Verification of Samsung’s Filter Shapes and Coefficients constraints (JCTVC-G648) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | I. S. Chong and M. Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: 1-858-845-5477 Email: ichong@qualcomm.com |  |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports verification results of JCTVC-G648 for CE8 Subset d.2.  Coding efficiency results are confirmed.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time have some degree of deviation.  Considering the difference of operating the environments, results are confirmed.

# Introduction

This contribution is a report of verification results of JCTVC-G648, which is a contribution to CE8 Subset d.2. Verification is performed under the CE8 test conditions that refer to JCTVC-E700.  In addition, source code is reviewed.

# Experimental Results

Our encoding environment is using 64bit linux cluster and decoding was done on sinlge 64 bit windows PC. There was no mismatch between encoder and decoder. There is no mismatch of overall BD-rate between the verification results and the proponent’s results.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.2% | -0.1% | -0.1% |
| Class B | -0.1% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | -0.1% |
| Class D | -0.1% | 0.0% | -0.1% |
| Class E | -0.2% | 0.0% | 0.0% |
| Overall | -0.1% | 0.0% | -0.1% |
|  | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.3% | -0.2% | -0.4% |
| Class B | -0.1% | -0.2% | -0.1% |
| Class C | -0.1% | -0.2% | -0.3% |
| Class D | -0.3% | -0.2% | -0.5% |
| Class E |  |  |  |
| Overall | -0.2% | -0.2% | -0.3% |
|  | -0.2% | -0.2% | -0.3% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.2% | -0.5% | -0.4% |
| Class C | -0.2% | -0.4% | -0.3% |
| Class D | -0.3% | -0.4% | -0.4% |
| Class E | -0.6% | -0.3% | -0.2% |
| Overall | -0.3% | -0.4% | -0.3% |
|  | -0.3% | -0.4% | -0.3% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.2% | -0.3% | -0.3% |
| Class C | -0.1% | -0.2% | -0.4% |
| Class D | 0.0% | 0.0% | -0.7% |
| Class E | -0.5% | -0.1% | -0.4% |
| Overall | -0.2% | -0.2% | -0.4% |
|  | -0.2% | -0.2% | -0.5% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 101% |  |  |

Table 1. Samsung’s proposal on CE8 Tool d.2

# Source code review

Samsung’s code follows the description of CE8 Tool d.2