| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G616_r1 WG11 Number: m22189 |
| --- | --- |

| Title: | On parallel deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The proposal discusses the parallel deblocking. The proposed solution is based on a parallel deblocking proposal G089 from Texas Instruments and includes some modifications to it. The proposal also addresses implementation of parallel deblocking in case of deblocking in CU order. The proposal invites to a discussion on parallel deblocking features in HEVC.

# Introduction

A merge of a parallel deblocking and parallel deblocking decisions has been adopted at Geneva meeting in March. The adopted solution includes filtering all vertical edges in the picture first and then all the horizontal edges. This allows high degree of parallelism since the all the vertical edges can be filtered in parallel by, for example, several processors, and then all the horizontal edges are can be filtered in parallel too. The adopted parallel decisions from Panasonic, in its turn, include filtering decisions based on the reconstructed (not filtered) pixels. Therefore, either the filtering decisions should be saved or the reconstructed pixels data in addition to the pixels after vertical edge filtering.

However, in the hardware implementations both vertical and horizontal deblocking filtering is usually done in the CU order. If the CU deblocking order is implemented (in a way similar to how it is described in WD4.0) filtering of horizontal edges adjacent to the next CU in the filtering order (horizontal or vertical) cannot be done before deblocking of the rightmost vertical edge of the next CU to the right has been done. After the next CU has been reconstructed and the vertical edge has been filtered, the deblocking of the horizontal edges in the CU to the left needs to be performed. This mean that the deblocking filter should return to the previous CU. This effect becomes even more pronounced in case of tile order processing, where all the left-most LCUs in the tile have their right-most horizontal edges not deblocked before the next tile is reconstructed.

CU

CU

CU

CU

CU

LCU

Decoded CUs

Delayed

filtering

Figure 1. Decoding followed by deblocking filtering in parallel mode. Filtering of blue edges is delayed until red edges are filtered. In case the parallel deblocking decisions are used, the information about decisions (or original pixel values) needs to be saved in the memory.

CU

CU

CU

CU

CU

LCU

Decoded CUs

Figure 2. Decoding followed by deblocking filtering in H.264-like deblocking mode. No filtering of horizontal is delayed.

It has been proposed in contribution G089 from Texas Instruments to modify the existing HEVC deblocking by splitting decisions for the 8-pixel edge into two parts by having two separate decisions (for each 4-line part of the block boundary instead) while the deblocking filtering is still done on the 8 pixel grid. Therefore, the parallel deblocking can be achieved on the 8 level blocks.

# Solutions to parallel deblocking

The solution, proposed in G089 (which is also similar to G590) has the following property. With this modification to the current LCU deblocking, it is possible to have high level deblocking parallelism without always filtering vertical edges first and then horizontal edges. In fact, since deblocking decisions for lines 0..3 are independent of deblocking decisions for lines 4..7, deblocking can easily be run in parallel on rectangular areas with splitting the reconstructed picture between the lines 3 and 4 on 8x8 blocks.

8x8

block

8x8

block

8x8

block

8x8

block

Parallel group 1

Parallel group 2

Parallel group 3

deblocking boundaries (8x8)

Parallel group 0

8x8

block

Parallel group 4

Figure 3. All these parallel groups (0-4) can be filtered in parallel. No dependencies between the parallel groups exist. Many any other parallel configurations are also possible.

# Proposed test

## Modifications compared to G089

The proposal tests the following modifications compared to G089. In HEVC, the boundary strength Bs is obtained for the 8-pixel block boundary. It is proposed in this contribution to find Bs separately for each 4-pixel boundary in luma deblocking to align it with deblocking decisions on 4-pixel edges (and with 4x4 transforms, as an example). By doing so, the complexity of the reference software also decreases, since the Bs derivation unit size is aligned with the minimum transform block.

Another modification is to use lines 0 and 3 for deblocking decision instead of lines 1 and 2 on the 4 pixels boundary. This can make the decisions more robust to signal variations in the block, since the signal is tested on both sides of the block 4-pixel edge.

These modifications are turned on in the code by

#define PAR_DBL                      1

## Modifications related to parallel deblocking

Test 1

This test therefore uses deblocking decisions proposed in G089 by TI with parallel deblocking turned off.

This is done by setting macros the following macro-definitions

#define PARALLEL_DEBLK_DECISION      0 // JCTC-E224: Parallel decisions

#define PARALLEL_MERGED_DEBLK        0 // JCTC-E224, JCTVC-E181: Parallel //decisions + Parallel filtering

#define PAR_DBL                      1

Test 2

This is compared to the case where parallel deblocking is still turned on.

#define PARALLEL_DEBLK_DECISION      0 // JCTC-E224: Parallel decisions

#define PARALLEL_MERGED_DEBLK        1 // JCTC-E224, JCTVC-E181: Parallel //decisions + Parallel filtering

#define PAR_DBL                      1

The main difference between these two solutions is the order of filtering of between the horizontal and edge on the block boundary. The first solution favors the H.264-like processing order, while the second solution favors the delayed deblocking of the edges in the previous CU. One advantage of having parallel filtering (all vertical edges first and all horizontal edges last) is that the order of filtering the “crosses” of vertical and horizontal edges is always the same. However, it leads to the delayed deblocking of the edges.

## Experimental results

Table 1 BD-rate results for test 1

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | 0.2% |
| Class B | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class C | -0.1% | -0.1% | 0.1% | -0.1% | 0.0% | 0.1% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | -0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% |
| Class C | -0.2% | 0.0% | -0.1% | -0.1% | 0.3% | -0.1% |
| Class D | -0.1% | 0.2% | 0.2% | -0.1% | 0.3% | -0.1% |
| Class E | -0.4% | 0.0% | 0.5% | -0.2% | 0.6% | -0.4% |
| Overall | -0.2% | 0.1% | 0.1% | -0.1% | 0.3% | -0.1% |
|  | -0.2% | 0.1% | 0.0% | -0.1% | 0.2% | -0.1% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |

Table 2 BD-rate results for test 2

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% | 0.0% | 0.1% | 0.1% |
| Class B | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.1% |
| Class C | -0.2% | -0.1% | -0.1% | -0.2% | 0.1% | -0.2% |
| Class D | -0.1% | 0.3% | 0.0% | -0.1% | 0.0% | 0.1% |
| Class E | -0.3% | -0.1% | 0.5% | -0.2% | -0.1% | -0.4% |
| Overall | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |
|  | -0.2% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |

The subjective quality of both test1 and test2 has been studied and was found similar to that of the anchor.

# Conclusions

The contribution discusses the issue with edge order in parallel deblocking. It also suggests modifications to parallel deblocking contribution G089 from TI. It is proposed to include modifications from Section 3.1 in the next version of HM and the working draft in case G089 is adopted. It is proposed to have a discussion in the group on which processing order of the vertical and horizontal edges is preferred.

# References

- “Non-CE12: Enable low-complexity and fully parallelized de-blocking filter with minor changes”, JCTVC-G089, Geneva, Nov. 2011.

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).