| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G618_r1 WG11 Number: m22184 |
| --- | --- |

| Title: | Level restrictions on tiles for line buffer reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin, Rickard Sjöberg Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The document proposes a solution to reduce the line-buffer problem. It is proposed to introduce level constraints on the maximum tile width. This reportedly decreases the amount of on-chip line memory that is required for in-loop filtering operations.

# Introduction

The line memory problem is widely recognized and has recently drawn a lot of attention in JCT-VC that has resulted in a number of contributions, particularly in CE8 (non-deblocking in-loop filtering) and CE12 (deblocking filtering). The examples of contributions are F053, F054, F055, F272, F215 etc…

The nature of the line buffer problem has to do with memory bandwidth in hardware decoder implementations. In order to decrease the number of read and write accesses to the memory, the in-loop filtering is generally done in decoding order. In the decoder, the CU is reconstructed, then the deblocking is applied for the internal block boundaries and the boundaries with left and upper neighbors, followed by SAO and ALF and then the in-loop filtered CU pixels are written to memory. However, deblocking filtering cannot be applied to block boundaries where neighboring blocks have not been reconstructed yet. Moreover, blocks not yet processed by deblocking filtering cannot be used as input to SAO and ALF which leads to an increase in required line memory. Therefore, lines of pixels at the bottom LCU boundary, not yet processed by loop filters (or needed for them as input) are kept in the on-chip memory (line buffer). The size of the luma line buffer is currently equal to 8 pixels [2]. This is significantly more then the line buffer in H.264 which constitutes only 4 lines. If one considers that HEVC is applied to coding 4Kx2K video, the amount of luma line memory in the HEVC decoder would be 4 times the luma buffer in H.264.

# Proposed solution

It is proposed to introduce restrictions on the maximum tile width when levels are defined. A restriction example can be that for a certain level, the tile width cannot exceed a certain number of LCU blocks. If the resolution of the video frame exceeds this number, the frame shall be split in tiles so that the minimum tile width would not exceed the predefined number of LCUs. Therefore, when designing decoder for a certain level, the amount of line buffer memory that decoder need to have can be decreased.

Usage of line buffer memory for tiles is shown in Figure 1. It can be seen that when decoding in the tile order, only yellow pixels need to be kept the line buffer. One observation that can be done here is that splitting picture in the “vertical” tiles can increase the size of the vertical “column” buffer that would need to hold the right-most columns of pixels in the rightmost LCUs in the tile (shown in green in Figure 1). In case of tile-order decoding, these columns cannot be processed before the LCU’s in the next tile to the right have been decoded. However, the decoder does not need to hold these columns in the line buffer. Instead, these blocks can be written to the frame memory and read back to the on-chip memory when needed. Since such a reading would happen only for one LCU in the row of LCU in the slice, the bandwidth would not be affected as much as in case of line memory. However, a restriction on the minimum tile width can also be introduced, so that the amount of reading from the “outer” memory that is done in the beginning of each LCU row is restricted.

LCU

#0

LCU

#1

LCU

#6

LCU

#7

LCU

#14

Vert. tile boundary

Tile #1

Line buffer

Column buffer

(saved in the memory)

decoded LCU’s

LCU’s to be decoded

Tile #0

Figure 1. Line buffer requirements in case of two “vertical” tiles per picture. Yellow lines should be kept in the “line buffer” while green columns can be written to memory and then read again

Another observation is that when the picture has horizontal tile boundaries, storing/reading of line buffer is done when switching between the tiles in the raster tile scan order. Therefore, another restriction on the minimum tile height can be introduced as well.

## Example of level restrictions

In Table 1, an example of minimum and maximum tile sizes are introduced.

Table 1 Example on restrictions on Tile size

| Level | MaxTileWidthInLCU | MinTileWidthInLCU | MinTileHeightInLCU |
| --- | --- | --- | --- |
| N | X1 | Y2 | Z1 |

Another example on possible restrictions on tile size would be as shown in Table 2.

Table 2 Another example of restrictions on Tile size

| Level | MaxTileWidthInLCU | MinTileBorderInLCU |
| --- | --- | --- |
| N | X1 | Y2 |

where MinTileBorder is equal to MinTileHeightInLCU + MinTileWidthInLCU.

# Conclusions

The proposal suggests a way of decreasing the minimum line-buffer size requirement for the decoder by introducing level limits on the maximum tile size. In this way, reduction of decoder line buffer memory can be achieved even without any special processing of the pixels at the horizontal LCU boundaries. However, the proposed method can also be combined with any other methods targeted at line buffer reduction, thus reducing required amount of line memory even more.

It is proposed to include limitations on tile size into HEVC levels definitions and start working on determining the required limits for tile size (related to the line buffer size).

# References

- Fuldseth (Cisco), M. Horowitz, S. Xu (eBrisk Video), A. Segall (Sharp), M. Zhou (TI), “Tiles”, JCTVC-F335, Italy, July 2011.

- C.-M. Fu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei, “CE8.c.2: Single-source SAO and ALF virtual boundary processing”, JCTVC-G204, Geneva, 21-30 November, 2011

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).