| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G619_r2 WG11 Number: m22185 |
| --- | --- |

| Title: | Non-CE12: deblocking parameters signaling in slice header |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin, Rickard Sjöberg, Kenneth Andersson, Jack Enhorn Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The document is a proposal for signaling deblocking filter parameters in the slice header. It is proposed to signal three deblocking parameters that control which pixels are being filtered and one clipping parameter that determines the largest possible modification of the pixels in the weak filtering mode.

# Introduction

The deblocking filtering can affect subjective quality of the reconstructed video. In H.264 the deblocking filter can be controlled at the slice header level thus making it possible for the control provide to control the amount and degree of deblocking filtering. However, the Working Draft text does not specify the deblocking filter parameters. Signaling of deblocking filter parameters in the current WD4.0 text should be modified in order to reflect the changes made to deblocking filter since H.264.

# Proposed solution

The current signaling of the deblocking filter parameters in Working Draft 4.0 [1] is shown in Table 1:

Table 1 Current syntax for deblocking parameters signaling

| if( deblocking_filter_control_present_flag ) { |  |
| --- | --- |
| disable_deblocking_filter_idc | ue(v) |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 | se(v) |
| slice_beta_offset_div2 | se(v) |
| } |  |

These parameters are the legacy of H.264 and need to be modified in order to control the deblocking loop filter. For example, the alpha parameter is not present in the deblocking filter.

It is proposed to change the deblocking parameters syntax as in the following:

Table 2 Proposed syntax for deblocking parameters signaling

| if( deblocking_filter_control_present_flag ) { |  |
| --- | --- |
| disable_deblocking_filter_idc | ue(v) |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| deblocking_parameters_flag | u(1) |
| if(deblocking_parameters_flag != 1 ) { |  |
| slice_tc_offset_div2 | se(v) |
| slice_beta_offset | se(v) |
| slice_side_beta_offset_div4 | se(v) |
| slice_nat_edge_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |

The following semantics are proposed for these parameters.

The deblocking filter is adapted to the QP by extracting the threshold values for filtering decisions from the corresponding beta_table and qp_table.

The parameters are set to beta = beta_table[QP] and tc = tc_table[QP+intra_tc_offset]. The beta parameter controls what edges are filtered and is connected to the “flatness” of the blocks forming the block boundary. Parameter tc is a clipping parameter that determined how much modification to the pixel values is allowed in the weak filtering mode.

However, the HEVC deblocking filter also uses the tc and beta parameters in order to find some other parameters used in the deblocking filtering decisions. One of these parameters is the side threshold which is approximately equal to beta/6. Another one is the threshold used in the comparison for natural edges, which is equal to tc*10. The side threshold controls how many pixels (1 or 2) from the block boundary are filtered.

A possible solution would be to send offsets for tc and beta parameters. However, one can see that in order to change the side threshold parameter by one, the beta parameter should be increased by 6. Therefore, it is suggested to send a separate offset divided by 4 for the side beta value. This would also allow more flexibility for adapting the deblocking filtering since the beta parameter determines what edges are being filtered and the side beta parameter determines how many pixels from the block boundary are filtered. In the same way the deblocking parameters that controls turning off the deblocking at the natural edge boundaries can be decoupled from the parameter controlling how much modification to the pixel values is allowed.

So, the following semantics of the deblocking filter control parameters is suggested:

IndexTC = Clip3(0, MAX_QP+2, QP + Tc_IntraOffset + slice_tc_offset_div2<<1 );

IndexB = Clip3(0, MAX_QP, QP + slice_beta_offset );

IndexThrCut=Clip3(0, MAX_QP+2, QP + uiTcOffset + slice_nat_edge_offset_div2<<1);

IndexSideBeta = Clip3(0, MAX_QP, iQP + slice_side_beta_offset_div4<<2);

Beta = betatable[IndexB]*iBitdepthScale;

BetaSide = betatable_8x8[iIndexSideBeta]*iBitdepthScale;

iTc = tctable [iIndexTC]*iBitdepthScale;

iTcCut = tctable [iIndexThrCut]*iBitdepthScale;

SideThreshold = (iBetaSide+(iBetaSide>>1))>>3;

ThrCut = iTcCut*10;

The BetaSide and TcCut parameters are also used in the decision between the strong and the weak filtering. Therefore, the slice_side_beta_offset_div4 controls how many pixels are filtered at each side of the block boundary: one, two, or three pixels.

The divisors for the offsets are chosen based on the form of beta and tc tables in order to minimize the cost of signaling the offset while still allowing fine granularity of threshold values.

# Experimental results

A simple encoder estimating parameter values has been implemented. The control tries deblocking with different values of the first parameter and chooses the value, which produces the lowest SSD. Then the next parameter is evaluated. After values for all four parameters have been evaluated, the obtained deblocking filtering results with all four modified values are compared based on RD-cost with sending no parameter updates and then the decision is taken whether to send the deblocking filter parameters.

We would like to notice here that such this optimization is not optimal in terms of BDR (since it does not searching the global SSD minimum) and was just a quick test to see if it is possible to obtain improvements in BDR by sending the deblocking filter parameters. Two tests have been done: one for normal test conditions and one for higher QPs [32, 37, 42, 47].

Table 3 BD-rate results, normal QPs

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Class B | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% |
| Class C | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| Class D | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| Class E | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
|  | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | 0.0% | 0.1% | -0.3% | 0.3% | 0.3% |
| Class B | -0.4% | -0.3% | -0.2% | -0.3% | -0.4% | -0.3% |
| Class C | -0.3% | -0.4% | -0.3% | -0.3% | -0.3% | -0.4% |
| Class D | -0.2% | -0.2% | -0.2% | -0.2% | -0.4% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -0.2% | -0.2% | -0.3% | -0.2% | -0.2% |
|  | -0.3% | -0.2% | -0.1% | -0.3% | -0.2% | -0.2% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -0.3% | -0.2% | -0.3% | 0.1% | 0.1% |
| Class C | -0.4% | -0.2% | -0.1% | -0.3% | 0.1% | -0.2% |
| Class D | -0.3% | 0.7% | -0.3% | -0.3% | -0.1% | 0.1% |
| Class E | -0.4% | 0.5% | 0.5% | -0.2% | 0.3% | -0.1% |
| Overall | -0.4% | 0.1% | -0.1% | -0.3% | 0.1% | 0.0% |
|  | -0.4% | 0.1% | -0.1% | -0.3% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 4 BD-rate results high QPs [32, 37, 42, 47]

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.1% | 0.1% | -0.1% | 0.1% | 0.1% |
| Class B | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | 0.1% | 0.1% |
| Class D | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.1% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% |
|  | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.2% | 0.4% | -0.3% | 0.8% | 0.3% |
| Class B | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% | 0.0% |
| Class C | -0.2% | -0.2% | -0.1% | -0.3% | -0.1% | 0.2% |
| Class D | -0.1% | 0.0% | 0.1% | -0.1% | -0.4% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | 0.0% | 0.1% | -0.2% | 0.0% | 0.1% |
|  | -0.2% | -0.1% | 0.1% | -0.2% | 0.1% | 0.1% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.2% | 0.0% | -0.2% | -0.4% | 0.1% |
| Class C | -0.1% | 0.0% | 0.0% | -0.2% | -0.2% | 0.0% |
| Class D | 0.0% | 1.4% | 0.6% | -0.2% | -0.3% | -0.1% |
| Class E | 0.3% | 1.2% | 0.0% | -0.1% | -0.5% | 0.2% |
| Overall | 0.0% | 0.6% | 0.1% | -0.2% | -0.4% | 0.0% |
|  | 0.0% | 0.7% | 0.2% | -0.2% | -0.3% | 0.0% |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

The encoding times are not reliable since the results have been obtained on a heterogeneous computer cluster.  The decoding times are obtained from a single PC with fast disk.

# Conclusions

Signaling of the deblocking filter parameters in slice header has been proposed. The proposed signaling sends offsets for the parameter tables for three parameters that control what pixels are modified by the deblocking filter and one parameter that controls the amount of modification that the deblocking filtering can do to the pixel values. We propose to adopt this signaling to the Working Draft text and implement it in the reference software.

We also propose to test the encoder control to finding the proposed parameters in the CE on deblocking filtering in order to determine, whether having such a control in the reference software is beneficial.

# References

- “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803, Torino, IT, 14-22 July, 2011

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).