| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G620 WG11 Number: m22186 |
| --- | --- |

| Title: | Clean-up of deblocking filter description |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The document proposes certain simplifications to the deblocking filter description and HM 4.0 reference software. In particular, it is proposed to remove clipping operations from strong filtering and to reduce the number of Bs values.

# Removing clipping operations in strong filtering

Current deblocking filter in WD 4.0 has the following description.

“Depending on dSam, the following applies:

–	When the variable dSam is equal to 1, the following strong filtering applies while nDp and nDq are set equal to 3:

p0’ = Clip1Y( ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )	(8-1)

p1’ = Clip1Y( ( p2 + p1 + p0 + q0 + 2 ) >> 2 )		(8-2)

p2’ = Clip1Y( ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 )		(8-3)

q0’ = Clip1Y( ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )	(8-4)

q1’ = Clip1Y( ( p0 + q0 + q1 + q2 + 2 ) >> 2 )		(8-5)

q2’ = Clip1Y( ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )		(8-6)”

It is proposed to change the description by removing Clip1Y() since having an input confined in the (0, x) interval will always produce an output confined to the same interval (0, x) for the filtering operations below:

“Depending on dSam, the following applies:

–	When the variable dSam is equal to 1, the following strong filtering applies while nDp and nDq are set equal to 3:

p0’ = ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 		(8-7)

p1’ = ( p2 + p1 + p0 + q0 + 2 ) >> 2 		(8-8)

p2’ = ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 		(8-9)

q0’ = ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 		(8-10)

q1’ = ( p0 + q0 + q1 + q2 + 2 ) >> 2 		(8-11)

q2’ = ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 		(8-12)”

The number of operations is reduced while the output is exactly the same.

# Limiting possible values for Bs to 0, 1 and 2

Currently, in the WD and HM4.0 software, five values of Bs are used, namely 0,1…4.

However, the software and the working draft only differentiate only between values

Bs = 0:  no filtering

Bs = 1 and 2:  filtering, tc_offset = 0

Bs = 3 and 4:  filtering, tc_offset = 2

It is therefore suggested to replace the current Bs values 1 and 2 with Bs value 1, and Bs values 3 and 4 with Bs values 2, as in the following:

Bs = 0:  no filtering

Bs = 1:  filtering, tc_offset = 0

Bs = 2:  filtering, tc_offset = 2

# Conclusions

It is proposed to remove clipping to (0, max_value) operations in the deblocking strong filtering. It is also proposed to reduce the number of possible Bs values since ranges of Bs values are currently used. There changes will make the working draft text simpler and more consistent, while the first change will also decrease the number of operations in the deblocking filtering. The proposed changes do not change the result of deblocking operations.

# References

- WD4: Working Draft 4 of High-Efficiency Video Coding, JCTVC-F803, Torino, IT, 14-22 July, 2011.

# Annex. WD changes related to Bs values

#### Derivation process of boundary filtering strength

[……..]

- Depending on the value of filterDir, the variable bS[ filterDir ][ xEk ][ yEj ] is derived as follows.

- If the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 2.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the block edge is also a transform unit edge and the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The sample p0 or q0 is in a transform unit which contains non-zero transform coefficient level.

- Otherwise, if any of the following conditions are true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The prediction unit containing sample p0 has different reference pictures or a different number of motion vectors with the prediction unit containing the sample q0.

- […]

- Otherwise, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 0.

##### Filtering process for luma block edge

A variable tC is specified as follows:

–	If bS is greater than 1, the variable tC is specified as Error: Reference source not found with luma quantization parameter Clip3(0, 55, qPL + 2 ) as input,

–	Otherwise (bS is equal to 1), the variable tC is specified as Error: Reference source not found with luma quantization parameter qPL as input.

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).