| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G627 |
| --- | --- |

| Title: | Cross-check of JCTVC-G199, "AHG4: Wavefront tile parallel processing" |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | A. Fuldseth, Cisco Systems | Tel: Email: | +47 92414082 arild.fuldseth@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This contribution presents results of cross-check of "AHG4: Wavefront tile parallel processing" proposed by Mediatek in document JCTVC-G199.

# Introduction

The cross check was done for high efficiency configurations, AI, RA, and LB with the following settings:

- WTPP - 2 threads

- WTPP - 4 threads

# Results

The BD-rate results match those of document JCTVC-G199. Regarding encoding and decoding times, the simulations were not run with comparable computing resources. Taking the variations in  the computing environment into account, the encoding and decoding times are significantly closer to the anchor than what appears in Table 1. A summary of the results are shown below.

Table 1. BD-rate results for WTPP, 2 threads

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0 % | -2.0 % | -2.7 % |
| Class B | 0.0 % | -0.1 % | -0.3 % |
| Class C | 0.0 % | 0.2 % | 0.1 % |
| Class D | 0.1 % | 0.2 % | 0.2 % |
| Class E | 0.0 % | -0.3 % | -0.3 % |
| Overall | 0.0 % | -0.4 % | -0.6 % |
|  | 0.0 % | -0.4 % | -0.5 % |
| Enc Time[%] | 118.8 % |  |  |
| Dec Time[%] | 138.0 % |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.1 % | -2.5 % | -3.0 % |
| Class B | 0.2 % | 0.0 % | -0.2 % |
| Class C | 0.4 % | 0.3 % | 0.4 % |
| Class D | 1.0 % | 1.0 % | 0.9 % |
| Class E |  |  |  |
| Overall | 0.4 % | -0.3 % | -0.5 % |
|  | 0.4 % | -0.3 % | -0.4 % |
| Enc Time[%] | 109.5 % |  |  |
| Dec Time[%] | 118.4 % |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.3 % | 0.1 % | -0.4 % |
| Class C | 0.4 % | 0.4 % | 0.3 % |
| Class D | 1.2 % | 1.7 % | 1.3 % |
| Class E | 1.4 % | -0.6 % | 1.2 % |
| Overall | 0.8 % | 0.4 % | 0.5 % |
|  | 0.8 % | 0.5 % | 0.5 % |
| Enc Time[%] | 111.1 % |  |  |
| Dec Time[%] | 108.4 % |  |  |

Table 2. BD-rate results for WTPP, 4 threads

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1 % | -1.9 % | -2.8 % |
| Class B | -0.1 % | -0.1 % | -0.2 % |
| Class C | 0.0 % | 0.3 % | 0.4 % |
| Class D | 0.2 % | 0.4 % | 0.4 % |
| Class E | -0.1 % | 0.0 % | -0.1 % |
| Overall | 0.0 % | -0.3 % | -0.5 % |
|  | 0.0 % | -0.3 % | -0.4 % |
| Enc Time[%] | 118.1 % |  |  |
| Dec Time[%] | 134.8 % |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.2 % | -2.4 % | -2.9 % |
| Class B | 0.3 % | 0.2 % | -0.1 % |
| Class C | 0.6 % | 0.8 % | 0.7 % |
| Class D | 1.7 % | 1.7 % | 1.7 % |
| Class E |  |  |  |
| Overall | 0.7 % | 0.1 % | -0.2 % |
|  | 0.7 % | 0.1 % | -0.1 % |
| Enc Time[%] | 109.4 % |  |  |
| Dec Time[%] | 118.9 % |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.4 % | 0.3 % | -0.2 % |
| Class C | 0.7 % | 1.0 % | 0.8 % |
| Class D | 1.9 % | 2.0 % | 2.4 % |
| Class E | 2.4 % | 1.0 % | 2.4 % |
| Overall | 1.2 % | 1.1 % | 1.2 % |
|  | 1.2 % | 1.1 % | 1.2 % |
| Enc Time[%] | 110.95 % |  |  |
| Dec Time[%] | 109.34 % |  |  |