| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G627 |
| --- | --- |

| Title: | Cross-check of JCTVC-G199, "AHG4: Wavefront tile parallel processing" |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | A. Fuldseth, Cisco Systems | Tel: Email: | +47 92414082 arild.fuldseth@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This contribution presents results of cross-check of "AHG4: Wavefront tile parallel processing" proposed by Mediatek in document JCTVC-G199.

# Introduction

The cross check was done for high efficiency configurations, AI, RA, and LB with the following settings:

- WTPP - 2 threads

- WTPP - 4 threads

# Results

The BD-rate results match those of document JCTVC-G199. A summary of the results are shown below.

Table 1. BD-rate results for WTPP, 2 threads

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0 % | -2.0 % | -2.7 % |
| Class B | 0.0 % | -0.1 % | -0.3 % |
| Class C | 0.0 % | 0.2 % | 0.1 % |
| Class D | 0.1 % | 0.2 % | 0.2 % |
| Class E | 0.0 % | -0.3 % | -0.3 % |
| Overall | 0.0 % | -0.4 % | -0.6 % |
|  | 0.0 % | -0.4 % | -0.5 % |
| Enc Time[%] | 118.8 % |  |  |
| Dec Time[%] | 138.0 % |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.1 % | -2.5 % | -3.0 % |
| Class B | 0.2 % | 0.0 % | -0.2 % |
| Class C | 0.4 % | 0.3 % | 0.4 % |
| Class D | 1.0 % | 1.0 % | 0.9 % |
| Class E |  |  |  |
| Overall | 0.4 % | -0.3 % | -0.5 % |
|  | 0.4 % | -0.3 % | -0.4 % |
| Enc Time[%] | 109.5 % |  |  |
| Dec Time[%] | 118.4 % |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.3 % | 0.1 % | -0.4 % |
| Class C | 0.4 % | 0.4 % | 0.3 % |
| Class D | 1.2 % | 1.7 % | 1.3 % |
| Class E | 1.4 % | -0.6 % | 1.2 % |
| Overall | 0.8 % | 0.4 % | 0.5 % |
|  | 0.8 % | 0.5 % | 0.5 % |
| Enc Time[%] | 111.1 % |  |  |
| Dec Time[%] | 108.4 % |  |  |

Table 2. BD-rate results for WTPP, 4 threads

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1 % | -1.9 % | -2.8 % |
| Class B | -0.1 % | -0.1 % | -0.2 % |
| Class C | 0.0 % | 0.3 % | 0.4 % |
| Class D | 0.2 % | 0.4 % | 0.4 % |
| Class E | -0.1 % | 0.0 % | -0.1 % |
| Overall | 0.0 % | -0.3 % | -0.5 % |
|  | 0.0 % | -0.3 % | -0.4 % |
| Enc Time[%] | 118.1 % |  |  |
| Dec Time[%] | 134.8 % |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.2 % | -2.4 % | -2.9 % |
| Class B | 0.3 % | 0.2 % | -0.1 % |
| Class C | 0.6 % | 0.8 % | 0.7 % |
| Class D | 1.7 % | 1.7 % | 1.7 % |
| Class E |  |  |  |
| Overall | 0.7 % | 0.1 % | -0.2 % |
|  | 0.7 % | 0.1 % | -0.1 % |
| Enc Time[%] | 109.4 % |  |  |
| Dec Time[%] | 118.9 % |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.4 % | 0.3 % | -0.2 % |
| Class C | 0.7 % | 1.0 % | 0.8 % |
| Class D | 1.9 % | 2.0 % | 2.4 % |
| Class E | 2.4 % | 1.0 % | 2.4 % |
| Overall | 1.2 % | 1.1 % | 1.2 % |
|  | 1.2 % | 1.1 % | 1.2 % |
| Enc Time[%] | 110.95 % |  |  |
| Dec Time[%] | 109.34 % |  |  |