| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G628 |
| --- | --- |

| Title: | Comparison of core transform proposals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | A. Fuldseth, Cisco Systems G. Bjøntegaard, Cisco Systems M. Budagavi, Texas Instruments Inc., USA | Tel: Email: Tel: Email Tel: Email | +47 92414082 arild.fuldseth@cisco.com gbjonteg@cisco.com +1-214-480-1861 madhukar@ti.com |
| Source: | Cisco Systems, Texas Instruments Inc |  |  |

_____________________________

# Abstract

This document provides comparisons between different core transform techniques. In particular 8-bit partial factorization is compared with 14-bit full factorization.

# Introduction

Two of the proposals for core transform design in HEVC can be expressed as a matrix multiplication for all transform sizes. In [1] 8-bit transform matrices supporting matrix multiplication and partial factorization (partial butterfly)  are proposed. This proposal is identical to what is currently used in HM4.0. In [2] 14-bit transform matrices supporting matrix multiplication, partial factorization and full factorization are proposed. For generic NxN transform matrices, a brute force 2D matrix multiplication requires 2N3 multiplications/additions. While partial factorization exploits the same symmetry properties as the DCT to reduce the number of arithmetic operations, full factorization exploits additional trigonometric relations to reduce the number of arithmetic operations further, The aim of this contribution is to compare partial factorization using 8-bit transform coefficients (PF8) and full factorization using 14-bit coefficients. The 8x8 matrices for PF8 and FF14 are shown  below.

Figure 1a 8x8 PF8 matrix

{ 64, 64, 64, 64, 64, 64, 64, 64}

{ 89, 75, 50, 18,-18,-50,-75,-89}

{ 83, 36,-36,-83,-83,-36, 36, 83}

{ 75,-18,-89,-50, 50, 89, 18,-75}

{ 64,-64,-64, 64, 64,-64,-64, 64}

{ 50,-89, 18, 75,-75,-18, 89,-50}

{ 36,-83, 83,-36,-36, 83,-83, 36}
{ 18,-50, 75,-89, 89,-75, 50,-18}

Figure 1b 8x8 FF14 matrix

{ 4096, 4096, 4096, 4096, 4096, 4096, 4096, 4096,}

{ 5696, 4800, 3264, 1088,-1088,-3264,-4800,-5696,}

{ 5312, 2304,-2304,-5312,-5312,-2304, 2304, 5312,}

{ 4770,-1080,-5670,-3240, 3240, 5670, 1080,-4770,}

{ 4096,-4096,-4096, 4096, 4096,-4096,-4096, 4096,}

{ 3240,-5670, 1080, 4770,-4770,-1080, 5670,-3240,}

{ 2304,-5312, 5312,-2304,-2304, 5312,-5312, 2304,}
{ 1088,-3264, 4800,-5696, 5696,-4800, 3264,-1088,}

# Comparisons

Table 1 summarizes the two proposals with respect to some fundamental properties.

Table 1 Some fundamental properties

| Property | PF8 | FF14 |
| --- | --- | --- |
| Matrix multiplication | Yes | Yes |
| Partial factorization | Yes | Yes |
| Full factorization | No | Yes |
| Matrix coefficients | 8 bits | 14 bits |
| Bit depths of intermediate buffers | 16 | 16 |
| Dequantization matrix size | 6 bytes | 6 bytes |
| Embedded structure | Yes | Yes |
| Symmetric "odd-half" matrices | Yes | No |

In Table 1, the two techniques differs only in support for full factorization, bit depth of the transform coefficients and support for symmetric "odd-half" matrices. "Odd-half" matrices of size N/2xN/2 are extracted from full NxN matrices by considering only the first half of each odd (anti-symmetric) basis vector. The 4x4 "odd-half" matrices extracted from the full 8x8 matrices for PF8 and FF14 are shown in Figure 2a and Figure 2b respectively. As can be seen from the figures, only the PF8 has a symmetric "odd-half" matrix. The same is true for other transform sizes as well. This property is useful in joint optimization of forward and inverse transforms in hardware.

Figure 2a "odd half" from 8x8 PF8 matrix

{ 89, 75, 50, 18}

{ 75,-18,-89,-50}

{ 50,-89, 18, 75}

{ 18,-50, 75,-89}

Figure 2b "odd-half" from 8x8 FF14 matrix

{ 5696, 4800, 3264, 1088

{ 4770,-1080,-5670,-3240

{ 3240,-5670, 1080, 4770}

{ 1088,-3264, 4800,-5696

Table 2 summarizes the number of arithmetic operations (add, mult, shift) for these methods. Typically, for software implementations, multipliers are either 16 bit or 32 bit and with 16-bit multipliers being most efficient in terms of CPU cycles. Thus, the table distinguishes between 16-bit and 32-bit multipliers where that information is available. For FF14, the exact distribution of 16-bit and 32-bit multipliers is not available, but we believe it is a well recognized fact that since combination of 16-bit input signals before multiplication happens to a large extent, a significant portion of the multipliers in a software implementation of FF14 are 32-bit.

Table 2 Arithmetic operations

|  | PF8 |  |  |  | FF14 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transform size | Mult16 | Mult32 | MultTot | Add | Mult16 | Mult32 | MultTot | Add |
| 4x4 | 48 | 0 | 48 | 64 | NA | NA | NA | NA |
| 8x8 | 352 | 0 | 352 | 448 | NA | NA | 176 | 464 |
| 16x16 | 2752 | 0 | 2752 | 3200 | NA | NA | 992 | 2593 |
| 32x32 | 21888 | 0 | 21888 | 23808 | NA | NA | 5568 | 5568 |

Table 3 addresses the complexity of partial inverse transforms where only the KxK low frequency coefficients of the input to a 2D NxN inverse transform are non-zero.  as can be seen from the table, there is a cross-over point where the PF8 implementation has less multiplications than FF14.

Table 3 Arithmetic operations for partial inverse transforms of various sizes

|  |  | PF8 | FF14 |
| --- | --- | --- | --- |
| NxN | KxK | Mult | Mult |
| 8x8 | 8x8 | 352 | 176 |
| 8x8 | 4x4 | 132 | 108 |
| 16x16 | 16x16 | 2752 | 992 |
| 16x16 | 8x8 | 1032 | 672 |
| 16x16 | 4x4 | 420 | 440 |
| 32x32 | 32x32 | 21888 | 5568 |
| 32x32 | 16x16 | 8208 | 3792 |
| 32x32 | 8x8 | 3400 | 2440 |
| 32x32 | 4x4 | 1476 | 1656 |

In Table 5, the number of unique numbers (ignoring sign values) in the transform matrices of size NxN are shown for PF8 and FF14 respectively. In general for an infinite precision DCT, the number of unique numbers in a NxN DCT matrix is N-1.  For example, the first half of the N/2 odd (anti-symmetric) basis vectors all share the same numbers but in a different order. Also the unique  numbers of smaller DCTs is a subset of the unique numbers in larger DCTs.

When designing integer-approximations to the DCT (such as PF8 and FF14)  the number of  unique numbers might decrease or increase. In case of PB8, the number of unique numbers has actually decreased from 31 to 29 since some of the real-valued DCT coefficients happened to be rounded to the same integer. The unique numbers of the PB8 32x32 transform are highlighted in Appendix A. A small number of unique numbers is also beneficial in hardware implementations to reduce the number of multipliers.

Table 5 Number of unique multipliers

| N | PB8 | FF14 |
| --- | --- | --- |
| 4 | 3 | 3 |
| 8 | 7 | 11 |
| 16 | 15 | 43 |
| 32 | 29 | 171 |

In Table 6, the accumulator bit depths at the end of each transform stage, but before right shift are shown. These bit depth assumes worst case dynamic range of all input samples to the forward and inverse 2D transforms respectively:

Forward:	(2B-1), where B is the bit depth of the input signal.

Inverse:		(215-1)

For software implementations, accumulator bit widths exceeding 32 bit is a major concern.

Table 6 Accumulator bit width

|  | PB8 |  |  |  | FF14 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FW1 | FW2 | IT1 | IT2 | FW1 | FW2 | IT1 | IT2 |
| 4 | 17 | 24 | 24 | 24 | 23 | 30 | 30 | 30 |
| 8 | 18 | 25 | 25 | 25 | 24 | 31 | 31 | 31 |
| 16 | 19 | 26 | 26 | 26 | 25 | 32 | 32 | 32 |
| 32 | 20 | 27 | 27 | 27 | 26 | 33 | 33 | 33 |

# Conclusion

The 8-bit partial factorization structure and the 14-bit full factorization structure have been compared with respect to the following properties:

- Symmetric "odd-half" matrices

- Arithmetic operations for full and partial transforms

- Number of unique numbers

- Accumulator bit widths

# References

- A. Fuldseth, G. Bjøntegaard, M. Budagavi, “Core transform design for HEVC,” JCTVC-G495, Geneva, November, 2011.

- E. Alshina, A. Alshin, I.-K. Kim, P. Topiwala, "Full Factorization Core Transforms for HEVC," JCTVC-G737, Geneva, November, 2011.

Appendix A

Below, the unique numbers of the PF8 32x32 transform are highlighted.

Green:		Unique numbers of even basis vectors - shared with smaller transform sizes.

Yellow:		Unique numbers of the odd basis vectors.

Red:		Coefficients that would be unique numbers of a floating point DCT, but not in the 
		particular integer approximations used for PF8.

{64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64}

{90 90 88 85 82 78 73 67 61 54 46 38 31 22 13  4 -4-13-22-31-38-46-54-61-67-73-78-82-85-88-90-90}

{90 87 80 70 57 43 25  9 -9-25-43-57-70-80-87-90-90-87-80-70-57-43-25 -9  9 25 43 57 70 80 87 90}

{90 82 67 46 22 -4-31-54-73-85-90-88-78-61-38-13 13 38 61 78 88 90 85 73 54 31  4-22-46-67-82-90}

{89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89 89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89}

{88 67 31-13-54-82-90-78-46 -4 38 73 90 85 61 22-22-61-85-90-73-38  4 46 78 90 82 54 13-31-67-88}

{87 57  9-43-80-90-70-25 25 70 90 80 43 -9-57-87-87-57 -9 43 80 90 70 25-25-70-90-80-43  9 57 87}

{85 46-13-67-90-73-22 38 82 88 54 -4-61-90-78-31 31 78 90 61  4-54-88-82-38 22 73 90 67 13-46-85}

{83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83}

{82 22-54-90-61 13 78 85 31-46-90-67  4 73 88 38-38-88-73 -4 67 90 46-31-85-78-13 61 90 54-22-82}

{80  9-70-87-25 57 90 43-43-90-57 25 87 70 -9-80-80 -9 70 87 25-57-90-43 43 90 57-25-87-70  9 80}

{78 -4-82-73 13 85 67-22-88-61 31 90 54-38-90-46 46 90 38-54-90-31 61 88 22-67-85-13 73 82  4-78}

{75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75 75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75}

{73-31-90-22 78 67-38-90-13 82 61-46-88 -4 85 54-54-85  4 88 46-61-82 13 90 38-67-78 22 90 31-73}

{70-43-87  9 90 25-80-57 57 80-25-90 -9 87 43-70-70 43 87 -9-90-25 80 57-57-80 25 90  9-87-43 70}

{67-54-78 38 85-22-90  4 90 13-88-31 82 46-73-61 61 73-46-82 31 88-13-90 -4 90 22-85-38 78 54-67}

{64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64}

{61-73-46 82 31-88-13 90 -4-90 22 85-38-78 54 67-67-54 78 38-85-22 90  4-90 13 88-31-82 46 73-61}

{57-80-25 90 -9-87 43 70-70-43 87  9-90 25 80-57-57 80 25-90  9 87-43-70 70 43-87 -9 90-25-80 57}

{54-85 -4 88-46-61 82 13-90 38 67-78-22 90-31-73 73 31-90 22 78-67-38 90-13-82 61 46-88  4 85-54}

{50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50 50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50}

{46-90 38 54-90 31 61-88 22 67-85 13 73-82  4 78-78 -4 82-73-13 85-67-22 88-61-31 90-54-38 90-46}

{43-90 57 25-87 70  9-80 80 -9-70 87-25-57 90-43-43 90-57-25 87-70 -9 80-80  9 70-87 25 57-90 43}

{38-88 73 -4-67 90-46-31 85-78 13 61-90 54 22-82 82-22-54 90-61-13 78-85 31 46-90 67  4-73 88-38}

{36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36}

{31-78 90-61  4 54-88 82-38-22 73-90 67-13-46 85-85 46 13-67 90-73 22 38-82 88-54 -4 61-90 78-31}

{25-70 90-80 43  9-57 87-87 57 -9-43 80-90 70-25-25 70-90 80-43 -9 57-87 87-57  9 43-80 90-70 25}

{22-61 85-90 73-38 -4 46-78 90-82 54-13-31 67-88 88-67 31 13-54 82-90 78-46  4 38-73 90-85 61-22}

{18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18 18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18}

{13-38 61-78 88-90 85-73 54-31  4 22-46 67-82 90-90 82-67 46-22 -4 31-54 73-85 90-88 78-61 38-13}
{ 9-25 43-57 70-80 87-90 90-87 80-70 57-43 25 -9 -9 25-43 57-70 80-87 90-90 87-80 70-57 43-25  9}
{ 4-13 22-31 38-46 54-61 67-73 78-82 85-88 90-90 90-90 88-85 82-78 73-67 61-54 46-38 31-22 13 -4}