| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G628 |
| --- | --- |

| Title: | Comparison of core transform proposals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | A. Fuldseth, Cisco Systems G. Bjøntegaard, Cisco Systems M. Budagavi, Texas Instruments Inc., USA | Tel: Email: Tel: Email Tel: Email | +47 92414082 arild.fuldseth@cisco.com gbjonteg@cisco.com +1-214-480-1861 madhukar@ti.com |
| Source: | Cisco Systems, Texas Instruments Inc |  |  |

_____________________________

# Abstract

This document provides comparisons between different core transform proposal.

# Introduction

Two of the proposals for core transform design in HEVC can be expressed as a matrix multiplication for all transform sizes. In [1] 8-bit transform matrices supporting matrix multiplication (MM) and partial factorization (PF) are proposed. This proposal is identical to what is currently used in HM4.0. In [2] 14-bit transform matrices supporting matrix multiplication, partial factorization and full factorization are proposed.

# Comparisons

Table 1 summarizes the two proposals with respect to some fundamental properties.

Table 1 Some fundamental properties

| Property | [1] | [2] |
| --- | --- | --- |
| Matrix multiplication | Yes | Yes |
| Partial factorization | Yes | Yes |
| Full factorization | No | Yes |
| Matrix coefficients | 8 bits | 14 bits |
| Bit depths of intermediate buffers | 16 | 16 |
| Dequantization matrix size | 6 bytes | 6 bytes |
| Embedded structure | Yes | Yes |

In the following, two methods using partial factorization with 8-bit matrix coefficients [1] and full factorization with 14-bit matrix coefficients [2] are compared more closely. These two methods are referred to as PF8 and FF14 respectively.

Table 2 summarizes the number of arithmetic operations (add, mult, shift) for these methods. The table distinguishes between 16-bit and 32-bit multipliers where that information is available.

Table 2 Arithmetic operations

|  | PF8 |  |  |  | FF14 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transform size | Mult16 | Mult32 | MultTot | Add | Mult16 | Mult32 | MultTot | Add |
| 4x4 | 48 | 0 | 48 | 64 | NA | NA | NA | NA |
| 8x8 | 352 | 0 | 352 | 448 | NA | NA | 176 | 464 |
| 16x16 | 2752 | 0 | 2752 | 3200 | NA | NA | 992 | 2593 |
| 32x32 | 21888 | 0 | 21888 | 23808 | NA | NA | 5568 | 5568 |

Table 3 Arithmetic operations, partial transforms of size KxK within a full transform of size NxN

|  |  | PF8 |  | FF14 |  |
| --- | --- | --- | --- | --- | --- |
| NxN | KxK | Mult | Add | Mult | Add |
| 8x8 | 8x8 | 352 | 448 | 176 | 464 |
| 8x8 | 4x4 | 132 | 228 |  |  |
| 16x16 | 16x16 | 2752 | 3200 | 992 | 2593 |
| 16x16 | 8x8 | 1032 | 1512 |  |  |
| 16x16 | 4x4 | 420 | 820 |  |  |
| 32x32 | 32x32 | 21888 | 23808 | 5568 | 5568 |
| 32x32 | 16x16 | 8208 | 10320 |  |  |
| 32x32 | 8x8 | 3400 | 5320 |  |  |
| 32x32 | 4x4 | 1476 | 3060 |  |  |

Table 4 Complexity analysis available

| Property | PB8 | FF14 |
| --- | --- | --- |
| Software, full size | Yes | Yes |
| Software, partial size | Yes | ? |
| Hardware, inverse 32x32 | Yes | Yes |
| Hardware, forward 32x32 | Yes | Yes |
| Hardware, forward&inverse 32x32 | Yes | No |

# References

- A. Fuldseth, G. Bjøntegaard, M. Budagavi, “Core transform design for HEVC,” JCTVC-G495, Geneva, November, 2011.

- E. Alshina, A. Alshin, I.-K. Kim, P. Topiwala, "CE10: Full-factorized core transform test by Samsung/FastVDO," JCTVC-F2251, Torino, July, 2011.