| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 |  |  |  | Document: JCTVC-G629 WG 11Number :m22196 |
| --- | --- | --- | --- | --- |
| Title: | Harmonization of SDIP and Mode-Dependent Secondary Transforms |  |  |  |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Ankur Saxena, Youji Shibahara Felix C. Fernandes Takahiro Nishi | Tel: Email: | 1-9727617761 asaxena@sta.samsung.com, +81-50-3587-2044 shibahara.youji@jp.panasonic.com, felix.f@sta.samsung.com, nishi.takahiro@jp.panasonic.com |  |
| Source: | Samsung Electronics Co., Ltd & Panasonic |  |  |  |

_____________________________

# Abstract

This contribution provides the harmonization of SDIP and JCTVC-G108: mode-dependent secondary transforms (MD-ST). The mode-dependent secondary transforms are implemented on top of SDIP, and can operate on square as well as on rectangular blocks. Experimental results are provided with HM 4.0+SDIP as anchor. Average BD Rate gains by harmonizing the proposed MD-ST scheme on top of SDIP are upto 0.8 %, 0.8 %, 0.4 % and 0.4% for Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency, and Random Access Low complexity settings respectively.

# Introduction

Mode-dependent secondary transforms were previously proposed in [1,2] and their harmonization with rectangular SDIP blocks was also presented in [1,2]. JCTVC-G108 [3] describes applying mode-dependent secondary transform on square blocks  in HM 4.0. In Short Distance Intra Prediction, rectangular blocks such as 2x8, 8x2, 4x16, 16x4, 8x32 and 32x8 can also be present. In this proposal, we apply the mode-dependent secondary transform (MD-ST) scheme for these rectangular blocks as well. Simulation results are provided for the proposed MD-ST scheme with HM 4.0+SDIP as anchor.

# Simulation Results

The test conditions are as stipulated in [4]. Here the anchor was HM 4.0+SDIP with the conventional DCT being applied as the transform for a block with horizontal (or respectively vertical) dimension 8 or larger. The performance of the proposed 8x8 MD-ST transform scheme in [3] on top of HM4.0+SDIP is evaluated for the following settings: Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency, and Random Access Low Complexity. The simulations were performed in a Linux 64 bit environment. The results are shown in Table 1. Detailed results are available in the attached Excel sheet JCTVC-G629_Test1.xls.

Table 1: Performance of Mode-Dependent Secondary Transform with HM4.0+ SDIP as anchor

|  |  | All Intra HE |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -1.0% | -0.6% | -0.8% | -0.9% | -0.8% | -1.1% |  |
|  | Class B | -0.9% | -0.9% | -0.9% | -0.9% | -0.9% | -1.0% |  |
|  | Class C | -0.6% | -0.5% | -0.5% | -0.7% | -0.6% | -0.6% |  |
|  | Class D | -0.4% | -0.3% | -0.4% | -0.5% | -0.4% | -0.4% |  |
|  | Class E | -1.3% | -0.7% | -0.9% | -1.3% | -0.8% | -0.9% |  |
|  | Overall | -0.8% | -0.6% | -0.7% | -0.8% | -0.7% | -0.8% |  |
|  |  | -0.8% | -0.7% | -0.7% | -0.8% | -0.7% | -0.8% |  |
|  | Enc Time[%] | 102% |  |  | 104% |  |  |  |
|  | Dec Time[%] | 103% |  |  | 104% |  |  |  |
|  |  | Random Access HE |  |  | Random Access LC |  |  |  |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.4% | 0.0% | -0.6% | -0.4% | -0.1% | -0.7% |  |
|  | Class B | -0.5% | -0.4% | -0.7% | -0.5% | -0.5% | -0.6% |  |
|  | Class C | -0.3% | -0.3% | -0.2% | -0.4% | -0.5% | -0.4% |  |
|  | Class D | -0.2% | 0.0% | -0.3% | -0.3% | -0.4% | -0.2% |  |
|  | Class E |  |  |  |  |  |  |  |
|  | Overall | -0.4% | -0.2% | -0.5% | -0.4% | -0.4% | -0.5% |  |
|  |  | -0.4% | -0.2% | -0.4% | -0.4% | -0.4% | -0.5% |  |
|  | Enc Time[%] | 100% |  |  | 100% |  |  |  |
|  | Dec Time[%] | 100% |  |  | 101% |  |  |  |

Next, we present the results for MD-ST at sizes 4x4, 5x5, 6x6 and 8x8 on top of SDIP. Here the anchor for simulations was SDIP Case 3 with bug-fix, as presented in JVTVC-G556. Tables 2 to 5 show the results for SDIP+MD-ST harmonization at MD-ST sizes 8x8, 6x6, 5x5 and 4x4 respectively. Detailed results are available in the accompanying excel sheets JCTVC-G629-MDST8.xls, JCTVC-G629-MDST6.xls, JCTVC-G629-MDST5.xls and JCTVC-G629-MDST4.xls respectively.

Table 2: Performance of Mode-Dependent Secondary Transform (8x8) with HM4.0+ SDIP as anchor

|  |  | All Intra HE |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.9% | -0.6% | -0.7% | -0.9% | -0.8% | -1.2% |  |
|  | Class B | -0.9% | -0.9% | -0.8% | -0.8% | -1.0% | -1.1% |  |
|  | Class C | -0.5% | -0.5% | -0.5% | -0.6% | -0.6% | -0.5% |  |
|  | Class D | -0.3% | -0.4% | -0.5% | -0.5% | -0.3% | -0.4% |  |
|  | Class E | -1.3% | -0.9% | -1.0% | -1.2% | -0.9% | -0.9% |  |
|  | Overall | -0.8% | -0.6% | -0.7% | -0.8% | -0.7% | -0.8% |  |
|  |  | -0.8% | -0.6% | -0.7% | -0.8% | -0.7% | -0.8% |  |
|  | Enc Time[%] | 102% |  |  | 104% |  |  |  |
|  | Dec Time[%] | 103% |  |  | 104% |  |  |  |

Table 3: Performance of Mode-Dependent Secondary Transform (6x6) with HM4.0+ SDIP as anchor

|  |  | All Intra HE |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.9% | -0.5% | -0.5% | -0.8% | -0.7% | -1.0% |  |
|  | Class B | -0.9% | -0.7% | -0.7% | -0.8% | -0.8% | -0.9% |  |
|  | Class C | -0.5% | -0.5% | -0.5% | -0.6% | -0.6% | -0.5% |  |
|  | Class D | -0.3% | -0.4% | -0.5% | -0.4% | -0.4% | -0.4% |  |
|  | Class E | -1.2% | -0.7% | -0.9% | -1.1% | -0.7% | -0.9% |  |
|  | Overall | -0.7% | -0.6% | -0.6% | -0.7% | -0.6% | -0.7% |  |
|  |  | -0.8% | -0.6% | -0.6% | -0.7% | -0.6% | -0.7% |  |
|  | Enc Time[%] | 101% |  |  | 101% |  |  |  |
|  | Dec Time[%] | 102% |  |  | 102% |  |  |  |

Table 4: Performance of Mode-Dependent Secondary Transform (5x5) with HM4.0+ SDIP as anchor

|  |  | All Intra HE |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.7% | -0.3% | -0.3% | -0.7% | -0.6% | -0.8% |  |
|  | Class B | -0.8% | -0.6% | -0.5% | -0.7% | -0.7% | -0.8% |  |
|  | Class C | -0.5% | -0.4% | -0.4% | -0.5% | -0.5% | -0.5% |  |
|  | Class D | -0.3% | -0.4% | -0.4% | -0.4% | -0.3% | -0.3% |  |
|  | Class E | -1.1% | -0.6% | -0.8% | -1.0% | -0.6% | -0.7% |  |
|  | Overall | -0.7% | -0.5% | -0.5% | -0.6% | -0.5% | -0.6% |  |
|  |  | -0.7% | -0.5% | -0.5% | -0.6% | -0.5% | -0.6% |  |
|  | Enc Time[%] | 101% |  |  | 102% |  |  |  |
|  | Dec Time[%] | 101% |  |  | 102% |  |  |  |

Table 5: Performance of Mode-Dependent Secondary Transform (4x4) with HM4.0+ SDIP as anchor

|  |  | All Intra HE |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.6% | -0.2% | -0.2% | -0.5% | -0.4% | -0.5% |  |
|  | Class B | -0.6% | -0.5% | -0.4% | -0.6% | -0.5% | -0.6% |  |
|  | Class C | -0.4% | -0.4% | -0.4% | -0.4% | -0.5% | -0.4% |  |
|  | Class D | -0.2% | -0.3% | -0.3% | -0.3% | -0.2% | -0.3% |  |
|  | Class E | -1.0% | -0.5% | -0.6% | -0.9% | -0.5% | -0.6% |  |
|  | Overall | -0.5% | -0.4% | -0.4% | -0.5% | -0.4% | -0.5% |  |
|  |  | -0.6% | -0.4% | -0.4% | -0.5% | -0.4% | -0.5% |  |
|  | Enc Time[%] | 101% |  |  | 100% |  |  |  |
|  | Dec Time[%] | 101% |  |  | 101% |  |  |  |

The results in Tables 2 to 5 were cross-checked in JCTVC-G937.

From the above experimental results, average BD rate gains of 0.8% and 0.8 % are obtained for Intra High Efficiency and Intra Low complexity settings, while there are gains of 0.4%, 0.4% in Random Access-HE and LC settings. This shows that when MD-ST tool is applied on top of SDIP, additional gains are achieved, due to the application of secondary transform on rectangular SDIP blocks.

# Conclusions

In this contribution, we have presented the results of harmonizing mode-dependent secondary transforms for intra-prediction residual coding with SDIP. Compression gains of upto 0.8 % are obtained for Intra High Efficiency and Intra Low Complexity settings.

# References

1. Y. Shibahara and T. Nishi, “CE7: Mode Dependent 2-step Transform for Intra Coding,” JCTVC-F224, Torino, Italy, July 2011.

2. A. Saxena and F. Fernandes, “On secondary transforms for intra prediction residual,” JCTVC-F554, Torino, Italy, July 2011.

3. A. Saxena, Y. Shibahara, F. Fernandes and T.Nishi, “CE 7: On secondary transforms for intra prediction residual,” JCTVC-G108, Geneva, Switzerland, Nov 2011.

4. F. Bossen., “Common test conditions and software reference configurations”, JCTVC-F900, Torino, IT, July 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd., may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).