| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G630 WG 11Number :m22197 |
| --- | --- |

| Title: | Cross Check of JCTVC-G345: Restricted Mode-Dependent 8x8 DST for Intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ankur Saxena, Felix C. Fernandes 1301 E. Lookout Drive, Richardson, TX – 75082, USA. | Tel: Email: | 1-9727617761, 1-9727617427 asaxena@sta.samsung.com, felix.f@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution summarizes Samsung’s cross-check for JCTVC-G345 proposal on restricted mode-dependent 8x8 DST for intra prediction. The simulation results as verified by Samsung match closely with the results provided by the proponents in JCTVC-G345. In addition, simulation results are also provided for the restricted 8x8 DCT/KLT scheme using the KLT matrix proposed in JCTVC-F138.

# Introduction

Mode-dependent DCT/DST was first published in literature in [1]. In HM, the mode-dependent DCT/DST is present for only 4x4 blocks. Mode-dependent DCT/DST scheme for 8x8 blocks was presented in [2] for 8x8 intra blocks. JCTVC-G345 [3] proposes two methods for improving the compression gains for the 8x8 mode-dependent DCT/DST scheme. Following is the summary of the changes introduced in HM in the two methods:

Method 1

(a) Apply 8x8 DCT/DST scheme for 8x8 Transform Unit (TU), when TU size = Prediction Unit (PU) size = 8x8.

(b) Apply 4x4 DCT/DST scheme for 4x4 TU, when TU size = PU size = 4x4. In HM, the 4x4 DCT/DST is applied at TU size = 4x4, irrespective of PU size.

Method 2

(a) Apply 8x8 DCT/DST scheme for 8x8 TU, when TU size = PU size = Coding Unit (CU) size = 8x8.

(b) No change in application of 4x4 DCT/DST as compared to HM.

We denote the tests performed by the above methods as Test 1 and Test 2 respectively. In addition, we also conducted simulations by using the 8x8 KLT matrix in JCTVC-F138 [4] instead of 8x8 DST in the above two methods. We term these tests as Tests 3 and Test 4 respectively.

# Simulation Results

The test conditions as stipulated in Core Experiment 7 [5] were followed for conducting the simulations. Tests were conducted on Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency, and Random Access Low Complexity configurations.

## R-D Results

Tables 1 and 2 shows the simulation results for JCTVC-G345 proposal as cross-checked by Samsung. Detailed results are available in the attached Excel sheets. The test results matched closely with those provided by the proponents.

Table 1: Simulation Results for JCTVC-G345 proposal: Test 1 as cross-checked by Samsung

|  |  | All Intra HE |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.6% | 0.2% | 0.2% | -0.3% | -0.4% | -0.6% |  |
|  | Class B | -0.3% | 0.3% | 0.3% | -0.1% | -0.4% | -0.4% |  |
|  | Class C | -0.2% | 0.0% | 0.1% | -0.3% | -0.3% | -0.3% |  |
|  | Class D | -0.1% | -0.1% | -0.1% | -0.3% | -0.2% | -0.2% |  |
|  | Class E | -0.4% | 1.1% | 0.8% | -0.3% | -0.2% | -0.3% |  |
|  | Class F |  |  |  |  |  |  |  |
|  | Overall | -0.3% | 0.3% | 0.2% | -0.2% | -0.3% | -0.4% |  |
|  |  | -0.3% | 0.2% | 0.2% | -0.3% | -0.3% | -0.4% |  |
|  | Enc Time[%] | 102% |  |  | 101% |  |  |  |
|  | Dec Time[%] | 101% |  |  | 99% |  |  |  |
|  |  | Random Access HE |  |  | Random Access LC |  |  |  |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.1% | 0.3% | 0.4% | -0.1% | -0.5% | -0.6% |  |
|  | Class B | -0.1% | 0.3% | 0.2% | -0.1% | -0.2% | -0.3% |  |
|  | Class C | -0.1% | 0.0% | 0.1% | -0.2% | -0.5% | -0.1% |  |
|  | Class D | -0.1% | 0.1% | -0.2% | -0.1% | -0.2% | -0.1% |  |
|  | Class E |  |  |  |  |  |  |  |
|  | Class F |  |  |  |  |  |  |  |
|  | Overall | -0.1% | 0.2% | 0.1% | -0.1% | -0.3% | -0.3% |  |
|  |  | -0.1% | 0.1% | 0.1% | -0.1% | -0.3% | -0.3% |  |
|  | Enc Time[%] | 103% |  |  | 101% |  |  |  |
|  | Dec Time[%] | 103% |  |  | 103% |  |  |  |

Table 2: Simulation Results for JCTVC-G345 proposal: Test 2 as cross-checked by Samsung

|  |  | All Intra HE |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.6% | 0.1% | 0.0% | -0.37% | -0.43% | -0.70% |  |
|  | Class B | -0.2% | 0.0% | 0.0% | -0.16% | -0.50% | -0.53% |  |
|  | Class C | -0.1% | -0.2% | -0.2% | -0.29% | -0.49% | -0.46% |  |
|  | Class D | -0.1% | -0.3% | -0.3% | -0.24% | -0.31% | -0.33% |  |
|  | Class E | -0.4% | 0.7% | 0.4% | -0.46% | -0.31% | -0.40% |  |
|  | Class F |  |  |  |  |  |  |  |
|  | Overall | -0.3% | 0.0% | 0.0% | -0.3% | -0.4% | -0.5% |  |
|  |  | -0.3% | 0.0% | -0.1% | -0.3% | -0.4% | -0.5% |  |
|  | Enc Time[%] | 102% |  |  | 102% |  |  |  |
|  | Dec Time[%] | 100% |  |  | 99% |  |  |  |
|  |  | Random Access HE |  |  | Random Access LC |  |  |  |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.2% | -0.1% | -0.5% | -0.1% | -0.2% | -0.6% |  |
|  | Class B | -0.2% | 0.2% | -0.1% | -0.1% | -0.2% | -0.1% |  |
|  | Class C | -0.1% | -0.3% | -0.1% | -0.2% | -0.4% | -0.3% |  |
|  | Class D | -0.1% | 0.1% | -0.4% | -0.1% | -0.3% | -0.1% |  |
|  | Class E |  |  |  |  |  |  |  |
|  | Class F |  |  |  |  |  |  |  |
|  | Overall | -0.1% | 0.0% | -0.2% | -0.1% | -0.3% | -0.3% |  |
|  |  | -0.1% | 0.0% | -0.3% | -0.1% | -0.3% | -0.3% |  |
|  | Enc Time[%] | 103% |  |  | 102% |  |  |  |
|  | Dec Time[%] | 103% |  |  | 104% |  |  |  |

Tables 3 and 4 shows the simulation results when the 8x8 KLT matrix in [4] is used instead of 8x8 DST in Methods 1 and 2 respectively.

Table 3: Simulation Results for method 1 in JCTVC-G345, where the 8x8 DST is replaced by 8x8 KLT in JCTVC-F138

|  |  | All Intra HE |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.7% | 0.4% | 0.3% | -0.4% | -0.4% | -0.7% |  |
|  | Class B | -0.4% | 0.4% | 0.5% | -0.2% | -0.4% | -0.3% |  |
|  | Class C | -0.3% | 0.2% | 0.2% | -0.3% | -0.3% | -0.3% |  |
|  | Class D | -0.2% | 0.0% | 0.0% | -0.3% | -0.2% | -0.2% |  |
|  | Class E | -0.4% | 1.4% | 1.0% | -0.3% | 0.0% | -0.2% |  |
|  | Class F |  |  |  |  |  |  |  |
|  | Overall | -0.4% | 0.4% | 0.4% | -0.3% | -0.3% | -0.3% |  |
|  |  | -0.4% | 0.4% | 0.3% | -0.3% | -0.3% | -0.3% |  |
|  | Enc Time[%] | 102% |  |  | 102% |  |  |  |
|  | Dec Time[%] | 101% |  |  | 100% |  |  |  |
|  |  | Random Access HE |  |  | Random Access LC |  |  |  |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.2% | 0.3% | 0.2% | -0.1% | -0.4% | -0.5% |  |
|  | Class B | -0.2% | 0.3% | 0.2% | -0.2% | -0.2% | -0.2% |  |
|  | Class C | -0.1% | 0.0% | 0.1% | -0.2% | -0.3% | -0.1% |  |
|  | Class D | -0.1% | 0.0% | -0.4% | -0.1% | -0.3% | -0.1% |  |
|  | Class E |  |  |  |  |  |  |  |
|  | Class F |  |  |  |  |  |  |  |
|  | Overall | -0.1% | 0.2% | 0.0% | -0.1% | -0.3% | -0.2% |  |
|  |  | -0.2% | 0.1% | 0.1% | -0.2% | -0.3% | -0.2% |  |
|  | Enc Time[%] | 103% |  |  | 102% |  |  |  |
|  | Dec Time[%] | 104% |  |  | 103% |  |  |  |

Table 4: Simulation Results for method 2 in JCTVC-G345, where the 8x8 DST is replaced by 8x8 KLT in JCTVC-F138

|  |  | All Intra HE |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.7% | 0.2% | 0.1% | -0.4% | -0.5% | -0.7% |  |
|  | Class B | -0.4% | 0.1% | 0.2% | -0.3% | -0.5% | -0.5% |  |
|  | Class C | -0.3% | -0.1% | 0.0% | -0.4% | -0.5% | -0.4% |  |
|  | Class D | -0.2% | -0.2% | -0.2% | -0.3% | -0.3% | -0.3% |  |
|  | Class E | -0.5% | 0.9% | 0.5% | -0.4% | -0.1% | -0.3% |  |
|  | Class F |  |  |  |  |  |  |  |
|  | Overall | -0.4% | 0.1% | 0.1% | -0.4% | -0.4% | -0.5% |  |
|  |  | -0.4% | 0.1% | 0.1% | -0.4% | -0.4% | -0.5% |  |
|  | Enc Time[%] | 102% |  |  | 103% |  |  |  |
|  | Dec Time[%] | 100% |  |  | 100% |  |  |  |
|  |  | Random Access HE |  |  | Random Access LC |  |  |  |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.2% | 0.5% | 0.0% | -0.1% | -0.1% | -0.2% |  |
|  | Class B | -0.2% | 0.2% | 0.1% | -0.2% | -0.1% | -0.1% |  |
|  | Class C | -0.1% | -0.2% | 0.0% | -0.2% | -0.3% | 0.0% |  |
|  | Class D | -0.1% | 0.0% | -0.2% | -0.1% | -0.3% | -0.2% |  |
|  | Class E |  |  |  |  |  |  |  |
|  | Class F |  |  |  |  |  |  |  |
|  | Overall | -0.2% | 0.1% | 0.0% | -0.1% | -0.2% | -0.1% |  |
|  |  | -0.2% | 0.1% | 0.0% | -0.1% | -0.2% | -0.1% |  |
|  | Enc Time[%] | 104% |  |  | 102% |  |  |  |
|  | Dec Time[%] | 104% |  |  | 104% |  |  |  |

We note that the 8x8 restricted DST scheme presented in [3] improves upon [2] in terms of compression efficiency (The gains in [2] were 0.2 % BD-Rate, which improves to around 0.3 % in [3]). However, for the 8x8 KLT in [4], the increase in gains by using the scheme presented in [3] is marginal (In [4], the gains for 8x8 DCT/KLT scheme were around 0.4 % BD-Rate, and similar results are obtained here).

## Complexity

The 8x8 DST transform is implemented via full matrix multiplication.

Operation Counts: The number of operations for performing 8x8 DST matrix is 1024 multipliers, 1024 adds, and 128 shifts.

Bit Shift at Intermediate Levels: In 8x8 DST implementation, the proponents have shifted the output of 8x8 DST (used either as vertical or horizontal transform) by 7-bits similar to the 7-bit shift for DCT in HM 4.0 and earlier presented in [2].

Scanning: In JCTVC-G345, the proponents have retained conventional HM 4.0 scans.

An accompanying excel sheet OperationCounts.xls gives the number of worst case and average case operations for the 2 methods presented in JCTVC-G345.

# Conclusions

We have cross-checked JCTVC-G138, studied their algorithms and verified the simulation results.

# References

1. J. Han, A. Saxena and K. Rose, “Towards jointly optimal spatial prediction and adaptive transform in video/image coding,” IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), March 2010, pp. 726-729.

2. A. Saxena and F. Fernandes, “CE7: Mode-dependent 8x8 DCT/DST for Intra Prediction,” JCTVC-F282, Torino, Italy, July 2011.

3. A. Ichigaya, Y. Sugito, S. Sakaida, “Non-CE7: Restricted mode-dependent 8x8 DST for Intra prediction,” JCTVC-G345, Geneva, Switzerland, November 2011.

4. R. Joshi et al., “CE7: Tool 1 - Mode dependent intra residual coding for 8x8 blocks,” JCTVC-F138, Torino, Italy, July 2011.

5. R. Cohen et al., “Description of Core Experiment 7 (CE7): Additional Transforms,” JCTVC-F907, Torino, Italy, July 2011.