| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G635 |
| --- | --- |

| Title: | Coding with a unified reference picture list |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s): | M. Naccari (BBC)*, G. Van Wallendael (Ghent University) **, M. Mrak (BBC) *, D. Flynn (BBC) * * BBC R&D, Centre House, 56 Wood Lane, W12 7SB London, UK ** Ghent University, Gaston Crommenlaan 8 P.O. Box 201, 9050 Ghent-Ledeberg, Belgium |  | matteo.naccari@bbc.co.uk glenn.vanwallendael@ugent.be marta.mrak@bbc.co.uk davidf@rd.bbc.co.uk |
| Source: | BBC Research & Development, Ghent University |  |  |

_____________________________

# Abstract

The unified reference picture list (LU - List Unified) was presented in contribution JCTVC-F549 with the aim of providing a simpler and more flexible structure to map the reference picture used during the inter coding process in the HM codec. The main idea behind the LU is to simplify mapping of reference pictures by using only a single reference list whereby reference frame pairs are stored. A reference pair consists of two reference frames (in case of bi-directional prediction) or one reference frame and a null element (in case of uni-directional prediction). The usage of the LU reduces the bitstream parsing and enables adding/removing some combinations of references in a more flexible fashion regarding the two HM reference lists. In this context, this contribution addresses the reference list indexes usage in the current HM 4.0 codec and an implementation of the LU based on the default HM reference settings. The experimental results obtained for this implementation show that the LU can handle usual HM conditions, while providing a space for more flexible selection of reference frames.

# Introduction

The Unified Reference Picture List (LU - List Unified) [1] was introduced to simplify the bitstream parsing for the reference frame indexes as well as to make the reference frame pairs selection more flexible. More precisely, the LU addresses the following design issues in the HM codec:

- The selection of reference picture pairs is limited due to dependency to reference lists; adding or removing some combination of reference pictures for the codec is currently done by modifying reference lists, which in general may add or remove more pairs than needed.

- The mapping direction modes for motion vectors to actual pictures from which prediction is done unnecessary depends on indexing into two reference lists, which further complicates mapping related to motion vector prediction.

The LU consists in a single list of reference frame pairs where a pair contains two indexes each one corresponding to the position, in the reference buffer, of the reference frame where the index refers to. Moreover, the pair contains two valid indexes to model the concept of bi-directional prediction or one valid index and a null index to model the concept of uni-directional prediction. According to what described in [1], the use of two reference lists in the HM codec imposes to consider all the combinations of possible reference frame indexes during the inter frame coding process. On the other hand, an encoder different then HM may use only a small subset of reference frame index combinations. In such case the current HM solution with List 0 (L0), List 1 (L1) and List Combined (LC) may be suboptimal. The LU allows to select the number of reference frame index pairs that are actually needed for a given encoding configuration.

This contribution demonstrates a way to preserve HM reference list setting, but by producing the bitstream using unified list. In this way it is demonstrated that LU can handle common HM conditions.

# Coding with the reference frame index pairs in the HM codec

## Coding setup

To integrate Unified List in the current HM design, only one modification needs to be added. Instead of signaling an inter direction and indices of the reference frames inside the reference lists, a single index containing this information can be signaled. In other words, on the PU level, reference information must be signaled by means of reference pairs instead of by an inter direction and one or two reference indices. Therefore, conceptually, just a different binarization is used to signal the reference information in the bitstream. Figure 1 depicts how the LU works and compares it with the current HM design. In this figure, U denotes unified and refers to a prediction unit whose motion vector is from either L0 or L1. The prediction directions depicted in Figure 1.a represent the current HM design and assume the values 1, 2 and 3, whereby 1 denotes prediction from L0, L1 or both lists, respectively. Figure 1.c represents the reference frame indexes associated to the prediction directions in Figure 1.a. Finally, Figure 1.c depicts the LU compact representation for inter directions and reference indexes. From Figure 1.c, it should be also noted that the current HM architecture is perfectly compatible with the proposed Unified List method.

Figure 1: The prediction direction and reference indexes modeled by the reference index pair in the LU.

The LU method is able to provide more flexibility compared to the current HM architecture because this list containing reference information can be signaled in the bitstream. Currently, HM determines L0 and L1 references in hard coded way. For content that is currently used during evaluation, this hard coded reference list creation may seem optimal, but leaves less room for future optimization. In this proposal, the reference pairs are transmitted to the decoder as follows. Considering that the used reference pairs are as shown in Figure 2, the encoder transmits in the slice header the number of used pairs (five in this example) and the binary codeword associated to the reference index for LIST 0 and LIST 1. Therefore, for the example in Figure 2, the transmitted information in the slice header is the one depicted in Table 1. As may be noted, once the decoder knows the actual reference frame indexes used, it only needs to receive, for each prediction unit, an index corresponding to the reference pair used for motion compensation and it will be able to retrieve the predictor for any prediction unit.

Figure 2: Example of reference pairs used during the slice coding process.

Table 1: Transmitted information associated to the reference pairs in Figure 2.

| Syntax element description | Binary codeword |
| --- | --- |
| Number of used pairs (5 in this example) | 101 |
| L0 and L1 reference indexes for the used pairs ((0,0) (0,U) (U,0) (0,1) (U,1)) | (01,01) (01,00) (00,01) (01,10) (00,10) |

## Implementation in the HM codec

The LU implementation in the HM codec implies to modify the entropy coding at the prediction unit and slice header level.

- Prediction unit level: From what explained in the Introduction, both the prediction direction and the reference frame index is included in the pair concept. Thus for each prediction unit only the reference pair index must be encoded as it implicitly contains also the prediction direction. The HM source code has then been modified as follows: during the motion information encoding for each prediction unit, the functions that encode both the prediction direction as well as the reference frame indexes have been replaced by a single function that encodes the reference pair index.

- Slice header level: At this level takes place the signaling of the reference frame indexes for the used pairs. In particular, the number of reference pairs is encoded first and then the encoding for all the reference pairs follows whereby the index from the LIST 0 is encoded first (see Table 1).

In order to select the reference pairs used for each slice, the HM encoder has been modified to carry out the following two phases:

- Phase 1 (reference index pairs counting): After the slice compression is finished, the encoder loops over all the inter predicted prediction units and counts the number of times a given reference index pair is used.

- Phase 2 (reference index pairs sorting): All the used reference index pairs are now sorted in the descendent order with respect to the number of times that the pair has been used. The sorting is needed for the entropy coding at the prediction unit level. In fact, by means of the aforementioned sorting the most frequently selected reference index pairs will receive shorter code words and the entropy coding will be more efficient.

## Syntax changes in the WD

The LU design simplifies the bitstream parsing related to the prediction units. In the following table, the bitstream parsing changes are highlighted in red and green. More precisely, green denotes the additional parsing whereby the strikethrough red denotes the parsing that will be pruned from the WD by the LU design.

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2PUWidth >= Log2IPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| Else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| if( IntraPredMode[ x0 ][ y0 ] == 2 ) |  |
| planar_flag_luma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| SignaledAsChromaDC = ( chroma_pred_from_luma_enabled_flag ? intra_chroma_pred_mode[ x0 ][ y0 ] == 3 : intra_chroma_pred_mode[ x0 ][ y0 ] == 2 ) |  |
| if( IntraPredMode[ x0 ][ y0 ] != 2 && IntraPredMode[ x0 ][ y0 ]!=34 && SignaledAsChromaDC ) |  |
| planar_flag_chroma[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } |  |
| } else { /* MODE_INTER */ |  |
| if( entropy_coding_mode_flag ¦¦ PartMode != PART_2Nx2N ) |  |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| ref_pair_idx | ue(v) ¦ ae(v) |
| if( slice_type = = B ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| combined_inter_pred_ref_idx | ue(v) |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) |
| } else |  |
| inter_pred_flag[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_LC ) { |  |
| if( num_ref_idx_lc_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_lc_minus4[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_lc[ x0 ][ y0 ] | ae(v) |
| } |  |
| mvd_lc[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_lc[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_lc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| else { /* Pred_L0 or Pred_BI */ |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l0_minusX[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_flag[ x0 ][ y0 ] = = Pred_BI ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) { |  |
| if( !entropy_coding_mode_flag ) { |  |
| if( combined_inter_pred_ref_idx == MaxPredRef ) |  |
| ref_idx_l1_minusX[ x0 ][ y0 ] | ue(v) |
| } else |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

# Experimental results

The LU implementation in the HM 4.0 has been tested over the Random Access – High Efficiency (HE), Random Access – Low Complexity (LoCo), Low Delay - High Efficiency (HE) and Low Delay - Low Complexity (LoCo) configurations [2].  The results are presented as BD-rates regarding the HM 4.0 anchors and are shown in Table 2 and Table 3. As may be noted, the LU implementation provides only negligible losses in the BD rates for both the RA configuration whereas for the LD the losses are larger. The PSNRs of each sequence are identical to the PSNRs of HM-4.0 sequences.

Table 2: BD-rates for the ra_he and ra_loco configurations.

Table 3: BD-rates for the ld_he and ld_loco configurations.

# Conclusions and future work

This proposal has presented the LU design implemented in the HM 4.0 codec. The LU provides similar rate distortion performance as the HM anchors while allows for a more flexible selection of the reference frame indexes and a simpler bitstream parsing. Possible extensions to the work presented in this proposals, may involve the usage of predefined reference pairs, with values collected over the JCT-VC test material, to include the LU also in the rate distortion optimization phase and different signaling methods for the reference indexes in the slice header.

# References

- D. Flynn, N. Šprljan and M. Mrak, “Coding with a single, unified reference picture list”, JCT-VC F549, 6th meeting, Torino, Italy, July 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCT-VC F900, 6th meeting, Torino, Italy, July 2011.

# Patent rights declaration(s)

The British Broadcasting Corporation (BBC) and Ghent University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, are prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).