| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G637 m22204 |
| --- | --- |

| Title: | AHG21: Long-term pictures and pruning of reference picture sets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg, Jonatan Samuelsson Ericsson Research 164 80 Stockholm Sweden | Tel: Email: | +46 761153563 rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This document expresses Ericssons support of the AHG21 candidate Working Draft. Further, it is proposed to adopt the long-term syntax and semantics from the October 18 candidate Working Draft (ftp://ftp.hhi.de/ahg21/JCTVC-F803_d1_Buffer_Descriptions_display_process_suggestion_r1.doc) from AHG21. In addition, this document also proposes to include syntax for omitting reference pictures from the PPS reference picture set.

# Reference picture set support

We support the AHG21 candidate Working Draft text for Reference Picture Set as described in JCTVC-G021.

# Long-term pictures

There are use cases in which the set of reference pictures consists of some reference pictures with small POC (picture order count) difference compared to the current picture (short-term pictures) and some reference pictures with large (negative) POC difference compared to the current picture (long-term pictures). We think the solution for reference picture set should handle this in a straight-forward and efficient way. We believe that it is important that the syntax for reference picture sets allows for signaling that a (short-term) reference picture set in the picture parameter set (PPS) is used together with one or more (long-term) reference pictures signaled explicitly in the slice header.

We therefore propose that syntax, semantics and decoding process effects for long-term pictures according to the AHG21 WD suggestion that was uploaded to the AHG21 FTP site on October 18 [1] are adopted.

# Pruning of PPS reference picture sets

The most bit-efficient way of signaling reference picture sets is to signal them in the PPS and reference them in the slice headers. Signaling reference picture sets explicitly in the slice header generally cost more bits, at least for cyclic picture structures where the same reference picture set has to be repeated in the slice headers of several pictures.

A random access points may break a cyclic picture structure, in the sense that some pictures can not be used for reference according the cyclic structure. The pictures that can not be used for reference should not be included in the reference picture set, thus the same reference picture set index in the list of reference picture sets in the PPS can not be used. Instead, either the reference picture set has to be signaled explicitly in the slice header, or the PPS must contain an extra reference picture set for this specific picture. Both alternatives cost bits.

Consider the AHG21 config file for random access below. The highlighted reference pictures are not available in the first GOP following a random access point. This means that for each of those pictures, either an additional reference picture set has to be added to the set of reference picture sets in the PPS, or the reference picture sets have to be explicitly signaled.

#        Type POC QPoffset QPfactor temporal_id ref_buf_size ref_pic #ref_pics reference pictures

Frame1:  B    8   1        0.442    0           4            1       4         -8 -10 -12 -14

Frame2:  B    4   2        0.3536   0           2            1       3         -4 -6 4

Frame3:  B    2   3        0.3536   0           2            1       4         -2 -4 2 6

Frame4:  B    6   3        0.3536   0           2            1       5         -2 -4 -6 -8 2

Frame5:  B    1   4        0.68     0           2            0       6         -1 -3 1 3 5 7

Frame6:  B    3   4        0.68     0           2            0       5         -1 -3 1 3 5

Frame7:  B    5   4        0.68     0           2            0       4         -1 -3 1 3

Frame8:  B    7   4        0.68     0           2            0       4         -1 -3 -5 1

This document proposes to add an optional syntax element in the slice header to signal the number of reference pictures in increasing POC order starting from the lowest POC that should be removed from the PPS reference picture set for the current slice. This will enable existing entries in the PPS reference picture set to be re-used also for pictures following a random access point. Note that removal of reference pictures from the PPS reference picture set is not restricted to the specific use case of random access points, an encoder is free to use this feature for any purposes.

## Proposed syntax, semantics and decoding process

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| ... |  |
| omit_pics_info_present_flag | u(1) |
| ... |  |

omit_pics_info_present_flag equal to 0 specifies that no coded pictures referring to the picture parameter set omits reference pictures from the short-term reference picture set included in the picture parameter set. omit_pics_info_present_flag equal to 0 specifies that a coded picture referring to the picture parameter set can omit reference pictures from the short-term reference picture set included in the picture parameter set

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_pps_flag | u(1) |
| if( !short_term_ref_pic_set_pps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else { |  |
| short_term_ref_pic_set_idx | u(v) |
| if(omit_pics_info_present_flag ) |  |
| num_omitted_pics | ue(v) |
| } |  |
| ... |  |

num_omitted_pics specifies the number of the negative short-term reference pictures that are to be omitted from the reference picture set in the picture parameter set with index short_term_ref_pic_set_idx. The value of num_omitted_pics shall be in the range of 0 to NumNegativePics[ StRpsIdx ], inclusive. When not present, the value of num_long_term_pics shall be inferred to be equal to 0.

### Decoding process for reference picture set

for( i = 0, j = 0, k = 0; i < NumNegativePics[ StRpsIdx ] – num_omitted_pics ; i++ ) {
	if( UsedByCurrPicS0[ StRpsIdx ][ i ] )
		RefPicSetStCurr0[ j++ ] = PicOrderCnt + DeltaPocS0[ StRpsIdx ][ i ]
	else
		RefPicSetStFoll0[ k++ ] = PicOrderCnt + DeltaPocS0[ StRpsIdx ][ i ]
}

Full proposed Working Draft changes relative to the latest AHG21 candidate Working Draft text [2] are attached together with this contribution.

# References

[1] ftp://ftp.hhi.de/ahg21/JCTVC-F803_d1_Buffer_Descriptions_display_process_suggestion_r1.doc

[2] ftp://ftp.hhi.de/ahg21/JCTVC-F803_d5_RPS_r1.doc

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).