| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G638 |
| --- | --- |

| Title: | Deblocking boundary strength and filtering process simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Kotra Matthias Narroschke Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766-154 Anand.Kotra@eu.panasonic.com +49 6103 766-204 Matthias.Narroschke@eu.panasonic.com +49 6103 766-1194 Thomas.Wedi@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution presents three simplifications in boundary strength (bS) derivation and filtering process for deblocking in HM-4.0. The first simplification removes the “coding unit edge” check for Intra blocks in bS derivation. In the second simplification, the output boundary strength values of the boundary strength derivation are adjusted such that the bS value can be directly used in tc (threshold) value derivation. Therefore, the extra tcoffset (derived based on bS) used in HM-4.0 for tc derivation is not necessary anymore. The third simplification performs the calculation of thresholds tc and ß only when bS>0. These simplifications reduce the number of instructions in the deblocking operations while complying with current HM-4.0.

# Introduction

In current HM-4.0 the boundary strength value is derived as shown on left side of Fig 1 [1]. The first simplification introduced is to remove the coding unit edge check, shown in Fig 1. All the edges of an intra block are assigned the same bS, irrespective of the edge being a coding unit edge or not. The bS value assignment is also modified, shown in Fig 1

Second simplification: In current HM-4.0, the threshold (tc) is derived by first deriving  a tcoffset as follows:                                                                                                                                                                     if (bS > 2) then tcoffset=2,                                                                                                                                        else                                                                                                                                                                        tcoffset = 0;

In second step the index into tctable is derived by combining quantization parameter (QP) with tcoffset. indexTC = clip3(0, MAX_QP+4, QP+tcoffset).

Then in the third step tc is derived as tc =TcTable[indexTC]

The simplifications introduced in this proposal eliminates for an explicit calculation of tcoffset.                      indexTC is now derived as follows: indexTC = clip3(0, MAX_QP+4, QP+bS-1).

The third simplification introduced, eliminates the calculation of the thresholds tc and beta (ß) when bS for the edge is zero (bS=0).

HM 4.0

Proposal

Fig. 1: Boundary strength derivation process

# Results

The simplifications introduced in the proposal reduce the number of instructions performed in deblocking. The result of the deblocking is same as HM-4.0. The changes are made relative to the latest working draft [4]. The changes are highlighted using yellow color and strikes.

# Required Changes in Working Draft

#### Derivation process of boundary filtering strength

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a two-dimensional arrays of size (nS)x(nS), horEdgeFlags and verEdgeFlags.

Output of this process is an array of size (2)x(nS)x(nS), bS specifying the boundary filtering strength.

Let ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1 specify a set of edge sample locations where nE is set equal to ( ( 1 << log2CUSize ) >> 2 ), xE0 = 0, yE0 = 0, xEk+1 = xEk + 4 and yEj+1 = yEj + 4.

For ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1, the following applies.

- If horEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk ][ yC + yEj – 1 ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 1.

- Otherwise, if verEdgeFlags[ xEk ][ yEj ] is equal to 1,

- Set sample p0 = recPicture[ xC + xEk – 1 ][ yC + yEj ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

- The variable filterDir is set equal to 0.

- Depending on the value of filterDir, the variable bS[ filterDir ][ xEk ][ yEj ] is derived as follows.

- If the block edge is also a coding unit edge and the following condition is true, the variable bS[ 0 ][ xEk ][ yEj ] is set equal to 4.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 3.

- The sample p0 or q0 is in a coding unit coded with intra prediction mode

- Otherwise, if the block edge is also a transform unit edge and the following condition is true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 2  1.

- The sample p0 or q0 is in a transform unit which contains non-zero transform coefficient level.

- Otherwise, if any of the following conditions are true, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 1.

- The prediction unit containing sample p0 has different reference pictures or a different number of motion vectors with the prediction unit containing the sample q0.

NOTE – The determination of whether the reference pictures used for the two prediction are the same or different is based on which pictures are referenced, without regard to whether a prediction is formed using an index into list 0 or an index into list 1, and also without regard to whether or not the index position within a reference picture list is different or not.

- One motion vector is used to predict the prediction unit containing sample p0, one motion vector is used to predict the prediction unit containing sample q0, and the absolute difference between the horizontal or vertical component of the motion vector used is greater than or equal to 4 in units of quarter luma samples.

- [Ed.: (WJ) needs to be checked again whether this condition covers all 2-motion cases] Two motion vectors are used to predict the prediction unit containing sample p0, two motion vectors are used to predict the prediction unit containing sample q0, and at least one of the motion vector pairs corresponding the same reference pictures and the different boundary samples p0 and q0 satisfies the following condition:

- The absolute difference between the horizontal or vertical component of a motion vector used in the prediction of the two prediction units is greater than or equal to 4 in units of quarter luma samples.

- Otherwise, the variable bS[ filterDir ][ xEk ][ yEj ] is set equal to 0.

#### Filtering process for coding unit

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a variable log2CUSize specifying the coding unit size,

–	an array bS specifying the boundary filtering strength.

Output of this process is:

–	modified reconstruction of the picture.

The filtering process for luma edges in the current coding unit consists of the following ordered steps:

- The variable nD is set equal to 1 << ( log2CUSize – 3 ).

- All elements of the three-dimensional array of size (2)x(nD)x(nD), dEdge are initialized to zero.

- All elements of the three-dimensional array of size (2)x(nD)x(1<<log2CUSize), dSample are initialized to zero.

- All elements of the three-dimensional array of size (2)x(nD)x(nD), bStrength ae initialized to zero.

- For xDk set equal to xC+( k << 3 ), k=0..nD – 1, the following applies:

- For yDm set equal to yC+( m << 3 ), m=0..nD – 1, the following ordered steps apply:

    - Boundary filtering strength bSVer is derived as follows:

bSVer = Max( bS[ 0 ][ xDk ][ yDm + i ] ) for i = 0..7 	(8-1)

    - bStrength[1][k][m] is set equal to bSVer.

    - only if bSVer > 0 then the decision process in 8.6.4.1 and filter process in 8.6.4.2 are invoked

    - Decision process for luma block edge in subclause  is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, and the boundary filtering strength bSVer as inputs and the decision dEdge[1][k][m] and an array dS of size (8) as outputs dSample[1][k][(m<<3)+i] is set equal to dS[i] for i=0..7.

    - Boundary filtering strength bSHor is derived as follows:

bSHor = Max( bS[ 1 ][ xDk + i ][ yDm ] ) for i = 0..7 	(8-2)

    - bStrength[0][k][m] is set equal to bSHor.

    - only if bSHor > 0 then the decision process in 8.6.4.1 and filter process in 8.6.4.2 are invoked

    - Decision process for luma block edge in subclause  is invoked  with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, the boundary filtering strength bSHor as inputs, the decision dEdge[0][k][m] and an array dS of size (8) as outputs.

    - dSample[0][m][(k<<3)+i] is set equal to dS[i] for i=0..7.

##### Decision process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	variables dE, dEp1 and dEq1 containing decisions,

–	one-dimensional array of size (8), dS containing decisions.

Let s’ represent the luma sample array recPictureL of the current picture.

A variables β is specified as Error: Reference source not found with luma quantization parameter qPL as input.

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified specified as Error: Reference source not found with luma quantization parameter Clip3(0, 55, qPL + bS-1 ) as input, (Also applies for chroma block edge)

–	Otherwise (bS is equal or less than 2), the variable tC is specified specified as Error: Reference source not found with luma quantization parameter qPL as input.

# References

- JCTVC-F484 Improving Worst-Case performance of Deblocking filter, M. Sadafale (TI), 6th Meeting: Torino, 14-22 July, 2011

- HM 4.0 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-4.0

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011.

- B. Bross et al., “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803_d6, Torino, July 2011

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).