| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G640 |
| --- | --- |

| Title: | Deblocking bug fix for CU-Varying QP’s and IPCM blocks. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Kotra Matthias Narroschke Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766-154 Anand.Kotra@eu.panasonic.com +49 6103 766-204 Matthias.Narroschke@eu.panasonic.com +49 6103 766-1194 Thomas.Wedi@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution presents modifications in deblocking operations when CU-based multi-QP optimization is enabled in HM-4.0.

In this proposal, Individual tc and ß thresholds are derived for the blocks corresponding to an edge on which weak filtering is performed. tc and ß values are dependent on the Quantization parameter (QP) which can vary for individual blocks when CU-based multi-QP optimization is enabled. The separate tc values are used in the deriving the delta offsets for innermost samples in weak filtering and separate ß values are used in decision for filtering of outermost samples in weak filtering.

In this contribution it is proposed to use an average of the QP’s for deriving tc and ß values in filter decision process and strong filtering. For the experimental results, an adaptation control that randomly assigns the QP for coding units is used. On an average, the coding gains observed are as follows:  Intra HE: 0.1%, Intra LC: 0.1%, RA HE: 0.1%, RA LC: 0.1%, LDB HE: 0.1%, LDB LC: 0.0%, LDP HE: 0.1%, LDP LC: 0.1%

This contribution also proposes to use a separate QP (QP_PCM) which is associated to IPCM blocks whenever deblocking filtering is desired to be performed over IPCM blocks. QP_PCM can be transmitted in the slice header.

# Introduction

When CU-based multi-QP optimization is enabled in HM-4.0 adjacent blocks can have varying QP. QP of the left CU (block A - QPA) and right CU (block B – QPB) may not be equal. See Fig 1.

The weak filtering takes as input the thresholds tc and ß. In this proposal the separate individual values tcA and ßA and tcB and ßB are derived for block A and B respectively. These separate thresholds are used as input for the weak filter. The modified steps in weak filter are as follows:

Samples used to filter- Weak filter

Samples modified by filter – Weak filter

Fig 1

Δ=9⋅q0i−p0i−3⋅q1i−p1i+8>>4A value                                                                            is calculated.

Δ<10⋅tctcp0iq0iIf                   , (where     is a threshold value derived using QP and boundary strength), then the innermost samples           and           are modified. Here the tc is derived using an average of QPA and QPB

p1iq1i(dp<(βA(QPA)6))(dq<(βB(QPB)6))A second decision                                   and                                  is performed for filtering the samples                    .      and         . Here separate individual values for ßA and ßB are derived using QPA and QPB

dpdqβwhere        ,       ,         are inputs for the weak filter operation.

The weak filter operation is done as follows:

Δ1=Clip3−tc,tc,Δtc2A=tcA(QPA)>>1tc2B=tcB(QPB)>>1

Δ2p=Clip3−tc2A,tc2A,p2i+p0i+1>>1−p1i+Δ1>>1

Δ2q=Clip3−tc2B,tc2B,q2i+q0i+1>>1−q1i−Δ1>>1

p1i'=Clipp1i+Δ2pq1i'=Clipq1i+Δ2q

For IPCM coded blocks: The QP (QP_PCM)) is considered while deriving the values tc and ß. QP_PCM can be transmitted in the slice header.

To the PCM-coded blocks, a

QP_PCM is associated

To the non PCM-coded blocks, the

transmitted quantization parameter  QP         is associated

Fig 2

A

B

# Experimental Results

| An adaptation control where a random way of assigning QP’s for coding units is considered and the results of deblocking with current HM-4.0 and the applied modification are compared. Parameters changed in config files: (Both reference and Tested) |  |  |
| --- | --- | --- |
| MaxDeltaQP : 7 # CU-based multi-QP optimization |  |  |
| MaxCuDQPDepth : 3 # Max depth of a minimum CuDQP for sub-LCU-level delta QP |  |  |

An adaptation control where a random way of assigning QP’s for coding units is considered. The reference used is a modified version of deblocking in HM-4.0 where tc and ß values are derived by taking the average of the QP’s of two adjacent blocks. (Similar to AVC like deblocking).

An adaptation control where a random way of assigning QP’s for coding units is considered and the results of deblocking with current HM-4.0 and the applied modification are compared. In addition to the applied modification in the weak filter, the strong filter operations are also modified.

The separate individual tc and ß which are derived for adjacent blocks A (tcA) and B (tcB) are used in the strong filtering operation as follows:

p0’ = Clip1Y( p0  + Clip3 (tcA (QPA),  -(tcA (QPA),  ( p2 + 2*p1 - 6*p0 + 2*q0 + q1 + 4 ) >> 3 )))

p1’ = Clip1Y( p1 +  Clip3 ( tcA (QPA),  -(tcA (QPA),  ( p2 -3*p1 + p0 + q0 + 2 ) >> 2 )))

p2’ = Clip1Y( p2  + Clip3 ( tcA (QPA),  -(tcA (QPA),  ( 2*p3  - 5*p2 + p1 + p0 + q0 + 4 ) >> 3 )))

q0’ = Clip1Y( q0  +  Clip3( tcB (QPB),  - tcB (QPB),  ( p1 + 2*p0  - 6*q0 + 2*q1 + q2 + 4 ) >> 3 )))

q1’ = Clip1Y(q1   +  Clip3 ( tcB (QPB),  - tcB (QPB),  ( p0 + q0  -3*q1 + q2 + 2 ) >> 2 )))

q2’ = Clip1Y(q2   +  Clip3 ( tcB (QPB),  - tcB (QPB),   ( p0 + q0 + q1  - 5*q2 + 2*q3 + 4 ) >> 3 ))).

The results are shown below.

CE4 subtest 1 anchor with the following configuration is considered.

MaxCuDQPDepth                 : 3           # Max depth of a minimum CuDQP for sub-LCU-level delta QP

AdaptiveQP                    : 1           # QP adaptation based on a psycho-visual model (0: None, 1: TM5Step3)

MaxQPAdaptationRange          : 12          # dQP range of QP adaptation

The results for deblocking modifications applied over CE4 subtest1 anchor are as follows:

# Required Changes in Working Draft

The changes are made relative to the latest working draft [3]. The changes are highlighted using yellow color and strikes.

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| qp_pcm | ue(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } |  |
| } |  |

##### Decision process for luma block edge

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture,

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top left luma sample of the current coding unit,

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

- A variable qPadj representing the QP of the block containing samples qi

Output of this process is:

–	variables dE, dEp1 and dEq1 containing decisions,

–	one-dimensional array of size (8), dS containing decisions.

- Variables tCp  and tCq

Let s’ represent the luma sample array recPictureL of the current picture.

For IPCM blocks qPL= QP_PCM

A variables β is specified as Error: Reference source not found with luma quantization parameter qPL as input.

variables βp and ßq are specified as Error: Reference source not found with luma quantization parameter qPL   and qPadj as input.

qPL is changed to average of qPL   and qPadj  for derivation of tc.

Variables tCp  and tCq are derived in the same way as tc below, but the input QP’s are qPL   and  qPadj

A variable tC is specified as follows:

–	If bS is greater than 2, the variable tC is specified specified as Error: Reference source not found with luma quantization parameter Clip3(0, 55, qPL + 2 ) as input,

–	Otherwise (bS is equal or less than 2), the variable tC is specified specified as Error: Reference source not found with luma quantization parameter qPL as input.

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1, the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 2,5 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]		(8-1)

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]		(8-2)

- The variables dp, dq and d are derived as follows:

dp = | p2,2 – 2*p1,2 + p0,2 | + | p2,5 – 2*p1,5 + p0,5 |		(8-3)

dq = | q2,2 – 2*q1,2 + q0,2 | + | q2,5 – 2*q1,5 + q0,5 |		(8-4)

d = dp + dq		(8-5)

- The variables dE, dEp1 and dEq1 are set equal to 0.

- If bS is not equal to 0 and d is less than β, the following ordered steps apply:

- for each sample location ( xC + xB, yC + yB + k ), k = 0..7, the following ordered steps apply:

  - The decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, the boundary filtering strength bS and the variables d, β and tC as inputs and a decision dSam as output.

  - The variable dS[k] is set equal to dSam

- The variable dE is set equal to 1.

- If dp is less than ( β βp + ( ( β βp  >> 1 ) ) >> 3, the variable dEp1 is set equal to 1.

- If dq is less than ( ( β βq  + ( ( β βq  >> 1 ) ) >> 3, the variable dEq1 is set equal to 1.

–	Otherwise (verticalEdgeFlag is equal to 0), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 2,5 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]		(8-6)

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]		(8-7)

- The variables dp, dq and d are derived as follows:

dp = | p2,2 – 2*p1,2 + p0,2 | + | p2,5 – 2*p1,5 + p0,5 |		(8-8)

dq = | q2,2 – 2*q1,2 + q0,2 | + | q2,5 – 2*q1,5 + q0,5 |		(8-9)

d = dp + dq		(8-10)

- The variables dE, dEp1 and dEq1 are set equal to 0.

- If bS is not equal to 0 and d is less than β, the following ordered steps apply:

- For each sample location ( xC + xB + k, yC + yB ), k = 0..7, the following ordered steps apply:

  - The decision process for a luma sample specified in subclause Error: Reference source not found is invoked with sample values pi,k, qi,k with i = 0..3, the boundary filtering strength bS and the variables d, β and tC as inputs and a decision dSam as output.

  - The variable dS[k] is set equal to dSam.

- The variable dE is set equal to 1.

- If dp is less than ( β βp + ( ( β βp  >> 1 ) ) >> 3, the variable dEp1 is set equal to 1.

- If dq is less than ( ( β βq  + ( ( β βq  >> 1 ) ) >> 3, the variable dEq1 is set equal to 1.

##### Filtering process for a luma sample

Inputs of this process are:

–	sample values, pi and qi with i = 0..3,

–	a variable dSam containing a decision,

–	variables dEp1 and dEq1 containing decisions to filter pixels p1 and q1 respectively,

–	a variable tC

- Variables tCp  and tCq

Output of this process is:

–	number of filtered samples nDp and nDq,

–	filtered sample values, pi’ and qj’ with i = 0..nDp – 1, j = 0..nDq – 1

Depending on dSam, the following applies:

–	When the variable dSam is equal to 1, the following strong filtering applies while nDp and nDq are set equal to 3:

p0’ = Clip1Y( ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4 ) >> 3 )	(8-11)

p1’ = Clip1Y( ( p2 + p1 + p0 + q0 + 2 ) >> 2 )		(8-12)

p2’ = Clip1Y( ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4 ) >> 3 )		(8-13)

q0’ = Clip1Y( ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4 ) >> 3 )	(8-14)

q1’ = Clip1Y( ( p0 + q0 + q1 + q2 + 2 ) >> 2 )		(8-15)

q2’ = Clip1Y( ( p0 + q0 + q1 + 3*q2 + 2*q3 + 4 ) >> 3 )		(8-16)

–	Otherwise, the following weak filtering applies while nDp and nDq are set equal to 0:

 = ( 9 * ( q0 –  p0 ) – 3 * ( q1 – p1 ) + 8 ) >> 4		(8-17)

  - When abs() is less than tc*10, the following ordered steps apply:

    - The filtered sample values p0’ and q0’ are specified as follows:

 = Clip3( -tc, tc,  )		(8-18)

p0’ = Clip1Y( p0 +  )		(8-19)

q0’ = Clip1Y( q0 -  )		(8-20)

    - If dEp1 is equal to 1, the filtered sample value pi’ is specified as follows:

p = Clip3( -(tc >> 1), tc >> 1, ( ( ( p2 + p0 + 1 ) >> 1 ) – p1 +  ) >>1 )	(8-21)

p = Clip3( -(tcp >> 1), tcp >> 1, ( ( ( p2 + p0 + 1 ) >> 1 ) – p1 +  ) >>1 )	(8-22)

pi’ = Clip1Y( p1 + p )		(8-23)

    - If dEq1 is equal to 1, the filtered sample value qi’ is specified as follows:

q = Clip3( -(tc >> 1), tc >> 1, ( ( ( q2 + q0 + 1 ) >> 1 ) – q1 –  ) >>1 )	(8-24)

q = Clip3( -(tcq >> 1), tcq >> 1, ( ( ( q2 + q0 + 1 ) >> 1 ) – q1 –  ) >>1 )	(8-25)

qi’ = Clip1Y( q1 + q )		(8-26)

    - nDp is set equal to dEp1+1 and nDq is set equal to dEq1+1.

Each of the filtered sample values, pi’ with i = 0..nDp-1, is substituted by the corresponding input sample value pi if all of the following conditions are true.

–	pi is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

Similarly, each of the filtered sample values, qj’ with j = 0..nDq-1, is substituted by the corresponding input sample value qj if all of the following conditions are true.

–	qj is a sample of an I_PCM block.

–	pcm_loop_filter_disable_flag value is equal to 1.

[Ed. (WJ): for PCM case, deblocking filter applies first and the filtered pixels are restored. Rather than this, it’s better to skip the filtering itself for PCM samples since first filtering is actually not needed.]

# References

- HM 4.0 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-4.0

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011.

- B. Bross et al, “WD4: Working Draft 4 of High-Efficiency Video Coding”, JCTVC-F803_d6, Torino, July 2011

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).