| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G641 WG11 Number: 22208 |
| --- | --- |

| Title: | CE1: Cross-check of Subset C (Test 1 and Test 11) and Subtest B - multi-parameter probability update for CABAC (Test 11) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jinwen Zan 175 Columbia St. West Waterloo, Ontario N2L 5Z5 Canada | Tel: Email: | +1-519-888-7465 jzan@rim.com |
| Source: | Research in Motion Ltd. |  |  |

_____________________________

# Abstract

This contribution is a report on cross-check results for CE1 Subtest C Test 1 (BAC for low complexity coding mode with 8-bit init values); Subtest C Test 11 (V2V for high efficiency coding mode with 8-bit init values and table based counter (TBC)) [1]; and Subtest B (multi-parameter probability update for context model based on the setting of Subset C Test 11) [2]. This cross-check was conducted under “Core Experiment 1 - Entropy Coding" [3]. The test results match with those provided by HHI for Subtest C and those provided by Samsung for Subtest B.

- Introduction

In [1], a unified entropy coding technique is proposed based on a simplified entropy coding engine. In order to study the trade-offs of this entropy coding technique, various tests are conducted. In this contribution, two Subtest C tests, Test 1 and Test 11, are reported.

In [2], a novel probability update algorithm has been proposed, which showed better adaptation property that the context update in HM employing BAC. In order to test its performance on the unified entropy coding technique, Subtest B is conducted based on the settings defined in Subtest C, test 11.

## Subtest C – Test 1

In this test, the software is provided by HHI, and is configured by defining the following macros in “TypeDef.h”

#define HHI_SYS_PIPE                               0

#define HHI_TABLE_BASED_BITCOUNTER              0

#define HHI_8BIT_INIT                              1

#define HHI_CE1_LC_MODELING                       1

This configuration tests the performance of the BAC for low complexity setting, using 8-bit init, but without table based bit counter.

## Subtest C – Test 11

In this test, the software is provided by HHI, and configure by defining the following macros in “TypeDef.h”

#define HHI_SYS_PIPE                               1

#define HHI_TABLE_BASED_BITCOUNTER              1

#define HHI_8BIT_INIT                              1

#define HHI_CE1_LC_MODELING                       0

This configuration tests the performance of the PIPE for high efficiency setting, using both the 8-bit init and table based bit counter.

## Subtest B – Test 11

In this test, the software is provided by Samsung based on the one from HHI, beside the HHI configuration as defined in Subtest C, Test 11, as shown in Section 1.2, a macro is defined in “TypeDef.h”

#define MULTI_PAR_CABAC_PROB_UP_DATE   1

This enables the multi-parameter probability update.

- Experimental Condition

For CE1 Subtest C, the software used for the tests was provided by HHI, and anchor data used for its reference was generated from HM-4.0.

For CE1 Subtest B, the software used for the tests was provided by Samsung, which in turn is based on HHI’s implementation for Subtest C.

Both software implementations were checked under the common test condition described in JCTVC-F900 [4].

The computing platform is a Linux cluster and the executables were compiled with gcc 4.1.2, under 64-bit mode.

- Results

## Subtest C – Test 1

The BD-rate performance of this test is shown in Table 1.

Table 1BD-Rate for Subtest C - Test 1

|  | All Intra LC |  |  | Random Access LC |  |  | Low delay B LC |  |  | Low delay P LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | 1.31% | 5.55% | 5.16% | -1.07% | 12.85% | 10.63% |  |  |  |  |  |  |
| Class B | 0.29% | 4.85% | 4.70% | -0.58% | 10.06% | 9.48% | -1.27% | 12.08% | 13.16% | -1.49% | 13.57% | 14.60% |
| Class C | -0.27% | 4.90% | 5.18% | -0.49% | 8.52% | 8.89% | -1.57% | 10.60% | 10.72% | -1.61% | 11.04% | 12.06% |
| Class D | 0.00% | 4.45% | 4.75% | 0.35% | 8.99% | 9.24% | -0.49% | 13.27% | 13.35% | -0.71% | 13.59% | 13.85% |
| Class E | -1.01% | 6.32% | 6.78% |  |  |  | -0.66% | 7.89% | 7.13% | -1.54% | 7.49% | 7.64% |
| Class F | 2.85% | 6.21% | 6.59% | 2.23% | 7.71% | 7.83% | 1.27% | 8.47% | 8.54% | 1.42% | 9.42% | 9.91% |
| Overall | 0.58% | 5.32% | 5.44% | 0.06% | 9.64% | 9.23% | -0.58% | 10.67% | 10.88% | -0.78% | 11.33% | 11.96% |
|  | 0.57% | 5.45% | 5.62% | 0.25% | 9.71% | 9.30% | -0.57% | 10.59% | 10.95% | -0.76% | 11.33% | 11.97% |
| Enc Time[%] | 107% |  |  | 103% |  |  | 104% |  |  | 107% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  | 99% |  |  | 100% |  |  |

## Subtest C – Test 11 and Subtest B

The BD-rate performance of this test is shown in Table 2.

Table 2 BD-Rate for Subtest C - Test 11 and Subtest B

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.7% | -0.5% | -0.6% |
| Class B | -0.5% | -0.9% | -0.9% |
| Class C | -0.6% | -0.7% | -0.7% |
| Class D | -0.6% | -0.4% | -0.4% |
| Class E | -0.5% | -1.1% | -1.2% |
| Class F |  |  |  |
| Overall | -0.6% | -0.7% | -0.7% |
| Enc Time[%] | 126% |  |  |
| Dec Time[%] | 112% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.7% | -0.2% | -0.1% |
| Class B | -0.4% | -0.6% | -0.5% |
| Class C | -0.3% | -0.2% | -0.1% |
| Class D | -0.3% | 0.1% | 0.0% |
| Class E |  |  |  |
| Class F |  |  |  |
| Overall | -0.4% | -0.2% | -0.2% |
| Enc Time[%] | 125% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.4% | -0.5% | -0.5% |
| Class C | -0.2% | -0.3% | -0.3% |
| Class D | -0.1% | -0.2% | 0.0% |
| Class E | -0.1% | -0.9% | -0.7% |
| Class F |  |  |  |
| Overall | -0.2% | -0.5% | -0.4% |
| Enc Time[%] | 125% |  |  |
| Dec Time[%] | 103% |  |  |

- Discussion

For Subtest B, the following probability tables are defined

static const UShort fwdprob[256];

static const UChar  inverseprob[4096];

which requires extra memory for storage.

In summary, the results provided by the proponents for CE1 Subset C, Test 1 , Test 11, and Subtest B were confirmed.

- References

- H. Kirchhoffer et al., “Unified PIPE-Based Entropy Coding for HEVC”, JCTVC-F268, Torino, July 2011.

- E. Alshina et al., “Multi-parameter probability up-date for CABAC”, JCTVC-F254, Torino, July 2011.

- Rajan Joshi et al., “Description of Core Experiment 1: Entropy Coding”, JCTVC-F901, Torino, July 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011.