| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G642 WG11 Number: 22209 |
| --- | --- |

| Title: | Cross-check of Samsung Context Reduction in Significance Map Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jinwen Zan Dake He 175 Columbia St. West Waterloo, Ontario N2L 5Z5 Canada | Tel: Email: | +1-519-888-7465 {jzan,dhe}@rim.com |
| Source: | Research in Motion Ltd. |  |  |

_____________________________

# Abstract

This contribution is a cross-check report for Samsung’s proposals on context reduction for significance map coding, including: 1) G768 – Reduced contexts for significance map coding of large transform in CABAC; and 2) G781 – Reduced chroma contexts for significance map coding in CABAC. The test results match with those provided by Samsung.

- Introduction

In [1], a context grouping technique for the TUs above size 8x8 is proposed. For the luma transform coefficients, the contexts are grouped as follows: 1) one context for the DC coefficient; 2) three contexts for coefficients with PosX + PosY < 5 but other than the DC coefficient (non-zero coefficient; 1 or 2 non-zero coefficients; and more than 3 non-zero coefficients in the pre-defined neighborhood as in HM-4.0); and 3) three contexts for the rest coefficients (same derivation rule as of 2). For the chroma transform coefficients, the contexts are grouped as follows: 1) one context for the DC coefficient; and 2) three contexts for the rest coefficients (same derivation rule as of luma 2). After this definition, the number of contexts for significance map coding for TUs above 8x8 has been reduced from 26 to 11.

The software is provided by Samsung, based on HM-4.0. A macro

#define SAMSUNG_CTX_SIGMAP_LTU   1

has been defined in “TypeDef.h”, which takes care of the context derivation etc.

In [2], a context grouping for the TUs of size 4x4 and 8x8 is proposed. For the 4x4 TUs, only one context is used for the region PosX + PosY > 2; and for the 8x8 TUs, only one context is used for the region (PosX >>1) + (PosY>>1) > 2. After this definition, the number of contexts for chroma significance map coding of TUs of size 4x4 and 8x8, has been reduced from 31 to 14.

The software is provided by Samsung, based on HM-4.0. A macro

#define SAMSUNG_CTX_SIGMAP_T4T8_CHROMA        1

has been defined in “TypeDef.h”, which takes care of the context derivation etc.

It is noted for both proposals, the corresponding HM-4.0 initializations has been used for the Samsung’s contexts.

- Experimental Condition

The computing platform is a Linux cluster and the executables were compiled with gcc 4.1.2, under 64-bit mode. The test settings conform to those defined in JCTVC-F900 [3].

- Results

## Results for G768

The BD-rate performance of G768 is summarized in Table 1.

Table 1BD-Rate for G768

|  | All Intra HE |  |  |  |  | Random Access HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U |  |  | V | Y | U | V |
| Class A | 0.0524% | 0.3094% |  |  | 0.3800% | -0.0542% | 0.2253% | 0.3577% |
| Class B | 0.0214% | 0.1643% |  |  | 0.1888% | -0.0049% | 0.1333% | 0.2928% |
| Class C | 0.0333% | 0.1147% |  |  | 0.0783% | 0.0395% | -0.0090% | 0.0830% |
| Class D | 0.0352% | 0.0977% |  |  | 0.0590% | 0.0632% | 0.2466% | -0.1142% |
| Class E | 0.0952% | 0.0448% |  |  | 0.1019% |  |  |  |
| Class F |  |  |  |  |  |  |  |  |
| Overall | 0.0438% | 0.1522% |  |  | 0.1659% | 0.0100% | 0.1481% | 0.1629% |
|  | 0.0443% | 0.1431% |  |  | 0.1560% | 0.0297% | 0.1120% | 0.1623% |
| Enc Time[%] |  | 101.3611% |  |  |  |  | 101.7524% |  |
| Dec Time[%] |  | 99.8980% |  |  |  |  | 106.0522% |  |
|  | Low delay B HE |  |  |  |  | Low delay P HE |  |  |
|  | Y |  | U | V |  | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.0154% |  | -0.0307% | -0.1637% |  | 0.0083% | 0.1027% | -0.0850% |
| Class C | 0.1085% |  | -0.1177% | -0.1987% |  | 0.0984% | 0.1001% | -0.2022% |
| Class D | -0.0056% |  | -0.0423% | 0.2802% |  | 0.0825% | -0.1718% | -0.3299% |
| Class E | 0.0718% |  | -0.3151% | -0.3796% |  | -0.0265% | 0.0556% | -1.1902% |
| Class F |  |  |  |  |  |  |  |  |
| Overall | 0.0440% |  | -0.1087% | -0.1019% |  | 0.0428% | 0.0246% | -0.3827% |
|  | 0.0470% |  | -0.1369% | -0.1314% |  | 0.0450% | -0.0080% | -0.4056% |
| Enc Time[%] |  |  | 99.3727% |  |  |  | 99.8963% |  |
| Dec Time[%] |  |  | 102.4767% |  |  |  | 104.8652% |  |

## Results for G.781

The BD-rate performance of G781 is summarized in Table 2.

- Discussion

The results provided by Samsung for contributions G.768 and G.781 were confirmed.

Table 2 BD-Rate for G781

|  | All Intra HE |  |  | Random Access HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0008% | 0.0168% | 0.0843% | 0.0396% | -0.2368% | 0.0214% |
| Class B | 0.0037% | 0.0666% | 0.0817% | 0.0083% | 0.0786% | 0.1116% |
| Class C | 0.0039% | 0.1409% | 0.1042% | -0.0161% | 0.0120% | 0.2093% |
| Class D | 0.0151% | 0.1609% | 0.1992% | 0.0193% | 0.2264% | 0.0542% |
| Class E | 0.0024% | 0.0180% | 0.0089% |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0052% | 0.0831% | 0.0993% | 0.0125% | 0.0235% | 0.0998% |
| Enc Time[%] |  | 102.3968% |  |  | 102.4072% |  |
| Dec Time[%] |  | 101.6994% |  |  | 106.8064% |  |
|  | Low delay B HE |  |  | Low delay P HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0046% | -0.1859% | 0.0012% | 0.0073% | 0.0296% | -0.0268% |
| Class C | 0.0259% | -0.1277% | -0.1651% | 0.0255% | 0.1337% | 0.0465% |
| Class D | -0.0068% | 0.1754% | 0.2596% | 0.0097% | 0.0232% | 0.1543% |
| Class E | 0.1195% | -0.3005% | 0.2156% | -0.0030% | -0.1808% | -0.5771% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0286% | -0.1025% | 0.0644% | 0.0105% | 0.0146% | -0.0664% |
| Enc Time[%] |  | 99.2145% |  |  | 99.8029% |  |
| Dec Time[%] |  | 102.2161% |  |  | 104.7638% |  |

- References

- Y. Piao, J. Min, and J.H. Park, “Reduced contexts for significance map coding of large transform in CABAC”, JCTVC-G768, Geneva, CH, 21 – 30 November, 2011.

- Y. Piao, J. Min, E. Alshina, and J.H. Park, “Reduced chroma contexts for significance map coding in CABAC”, JCTVC-G781, Geneva, CH, 21 – 30 November, 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July 2011.