| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G643r1 |
| --- | --- |

| Title: | AHG21: On reference picture list construction and reference picture marking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Srikanth M. Gopalakrishna Visiokatu 1 33720 Tampere Finland | Tel: Email: | +358 7180 73151 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

The contribution proposes reference picture list description of lists 0 and 1 as well as an idle reference picture list to be used instead of reference picture list initialization and modification and reference picture marking of HEVC WD4 or reference picture sets and reference picture list initialization and modification of JCTVC-F493 and its later revisions.

The contribution also proposes

- a two-part coding of POC differences as a “short-term” part and a ue(v)-coded “long-term” part;

- temporal_id-based POC granularity for POC difference coding; and

- use of the two-part coding of POC differences for determining the marking between “used for short-term reference” or “used for long-term reference”.

It is asserted that proposals 1 to 3 above could be used with JCTVC-F493 and its later revisions too.

Revision 1 of the proposal included the following additions:

- reference picture list subset description, which enables to specify a reference picture list by removing reference pictures from the tail of a certain earlier reference picture list description

- POC difference coding relative to a POC prediction value

It is asserted that the number of bits needed in the picture parameter set to describe the reference picture lists for the random access and low delay common conditions is 157 and 132 bits, respectively, according to the syntax in revision 1 of the contribution.

# Summary

According to JCTVC-F493 and its later revisions, a reference picture set is specified to include reference pictures marked as “used for reference”, whereas all other reference pictures are marked as “unused for reference”. Pictures in a reference picture set are used for reference picture list initialization and the initial reference picture lists may be reordered with the reference picture list modification syntax, if present. A reference picture set is described in two loops within the ref_pic_set( idx ) syntax, the first loop including the reference pictures having a negative POC difference and the second loop including the reference pictures having a positive POC difference compared to the POC of the current picture. Furthermore, a flag for each loop entry indicates if the indicated picture is used as reference for the current picture.

This contribution explores alternatives to reference picture list construction and reference picture marking, which can potentially simplify the decoding process and provide compression improvements compared to HEVC WD and JCTVC-F493 and its later revisions.

The differences compared to HEVC WD and JCTVC-F493 (and its later revisions) may be summarized as follows. Proposal 1 is an alternative to JCTVC-F493 and its later revisions, whereas it is believed that proposals 2 to 4 may be used equally with JCTVC-F493 and its later revisions.

- Reference picture list description structures of reference picture lists 0, 1, and idle used for reference picture marking

This contribution proposes reference picture list description of lists 0 and 1 as well as an idle reference picture list to be used instead of reference picture list initialization and modification and reference picture marking of HEVC WD or reference picture sets and reference picture list initialization and modification of JCTVC-F493 and its later revisions. A reference picture list description is similar to a reference picture set of JCTVC-F493 but pictures are listed in the order they appear in the respective reference picture list.

The proposal also enables the describe reference picture lists as subsets of ealier reference picture list descriptions similarly to JCTVC-G198.

An idle reference picture list includes reference pictures not used for reference for the current picture but still marked as “used for reference”. Pictures that are not in any of the three reference pictures for the current slice are marked as “unused for reference”.

Consequently, the reference picture list initialization process as well as the reference picture list modification syntax structure, semantics, and decoding process become unnecessary. Likewise, reference picture sets are not used in this proposal.

- Two-part coding of POC differences as a “short-term” part and a ue(v)-coded “long-term” part

It is proposed to code a POC difference in a reference picture list description in two parts, a “short-term” part and a ue(v)-coded “long-term” part. The short-term part is coded with ue(v) in picture parameter sets, and with u(v) in slice headers. The maximum value of the u(v)-coded part, also referred to as POC cycle, is proposed to be indicated in the sequence parameter set. The “long-term” part is indicated in units of POC cycles. This two-part indication of POC differences is believed to provide efficient coding for POC values that are close to the POC of the current picture, while it is also believed to have the flexibility to address pictures that remain as “used for reference” for a relatively long period.

- temporal_id based POC granularity

It is asserted that encoders may choose to keep the POC value difference of any two consequent pictures in output order having the same temporal_id remains unchanged. For example, encoders may use a regular GOP pattern and assign POC values proportional to a constant picture sampling interval. It is therefore proposed that a POC granularity value is provided for each temporal_id in the sequence parameter set. The POC granularity value for a particular temporal_id indicates the POC interval between any two consecutive pictures in output order having that temporal_id. The POC granularity value is used to quantize the POC differences indicated in the reference picture list description syntax structures to improve compression efficiency.

POC granularity value equal to 1 can be indicated for bitstreams having an irregular POC assignment pattern.

- Marking as used for long-term reference or short-term reference

The proposed two-part POC difference signaling may also be used for the determination which reference picture are regarded as “long-term” reference pictures e.g. for motion vector scaling in motion vector predictor derivation process. That is, if the long-term part is present (i.e. non-zero) for a reference index, then the corresponding reference picture may be marked as “used for long-term reference” for the decoding of the current picture.

# Detailed Proposal

This section provides the changes relative to the HEVC WD (JCTVC-F803_d3).

## Sequence Parameter Set

### Summary

The proposed additions in the sequence parameter set contain two types of information:

- Granularity of the POC differences in the reference picture list description syntax. The POC granularity is provided for each temporal_id. Moreover, one set of granularities is given for reference picture lists 0 and 1, and another is given for the idle reference picture list.

- Length of the u(v)-coded “short-term” part of the POC differences in the reference picture list description syntax included in slice headers. The length, which is equivalent a maximum value, is given for each temporal_id. Moreover, one set of lengths is given for reference picture lists 0 and 1, and another is given for the idle reference picture list.

### Additions to Syntax

The following syntax is added to the sequence parameter set:

| idle_lists_present_flag | u(1) |
| --- | --- |
| for( i = 0; i <= max_temporal_layers_minus_1; i++ ) { |  |
| poc_granularity_minus1[ i ] | ue(v) |
| max_shortterm_poc_diff_minus1[ i ] | ue(v) |
| if( idle_lists_present_flag = = 1 ) { |  |
| poc_granularity_idle_idx[ i ] | ue(v) |
| max_poc_diff_idle_minus1[ i ] | ue(v) |
| } |  |
| } |  |

### Additions to Semantics

The semantics of these additions is the following:

idle_lists_present_flag equal to 0 indicates that no syntax elements relating to idle reference picture lists are present. idle_lists_present_flag equal to 1 indicates that certain syntax elements relating to idle reference picture lists are present.

poc_granularity_minus1[ i ] + 1 specifies that the POC value interval of any two consecutive pictures in output order having temporal_id equal to or smaller than i shall be j * (poc_granularity_minus1[ i ] + 1), where j is a positive integer. (poc_granularity_minus1[ i ] + 1) shall be equal to k * (poc_granularity_minus1[ i + 1 ] + 1), where k is a positive integer and i is greater than 0.

max_shortterm_poc_diff_minus1[ i ] + 1 specifies the following constraint on the pictures in reference picture list 0 and reference picture list 1. Let picI be a picture having temporal_id equal to i and absPocDiff be the absolute difference of the POC value of picI and the POC value of any picture in reference picture list 0 and reference picture list 1 of picI in units of poc_granularity_minus1[ i ] + 1. max_shortterm_poc_diff_minus1[ i ] + 1 shall be greater than or equal to the value of absPocDiff for any picI having temporal_id equal to i.

poc_granularity_idle_idx[ i ] specifies the POC difference granularity for the reference picture list description of the idle reference picture list and for temporal_id equal to i.

max_poc_diff_idle_minus1[ i ] specifies the maximum absolute POC difference that uses fixed-length coding for in the reference picture list description of the idle reference picture list and for temporal_id equal to i.

## Reference Picture List Description

### Summary

This syntax structure is used to describe a reference picture list refPicListX (RefPicList0, RefPicList1, or RefPicListIdle).

When the reference picture list description is included in a picture parameter set, there may be several instances of the structure for the same reference picture list (0, 1, or Idle), each identified by the input parameter idx.

Two modes of reference picture list description are enabled: full and subset description (ref_pic_list_description and ref_pic_list_subset_description syntax structures, respectively). In the full description, each list item of the reference picture list is described. In the subset description, the number of reference pictures excluded from the tail of another certain reference picture list is indicated.

The ref_pic_list_description( ) syntax structure also inputs

- number of included reference pictures (numRefPics),

- an indication of all POC differences have the same sign (pocDiffModeFlag),

- an indication if long-term pictures may be included in the list (longTermFlag),

- an indication of the POC difference is ue(v) or u(v) coded and the maximum value of the u(v)-coded “short-term” part of the POC difference (maxPocDiff), and

- an indication of the basis of the relative POC difference, either the POC of the current picture or a POC prediction value (pocPredFlag).

The ref_pic_list_description( )syntax structure contains two loops, where the first loop contains the “short-term” part of the POC difference for each reference picture list entry and the second loop contains the ue(v)-coded “long-term” part of the POC difference for selected reference indexes.

The ref_pic_list_subset_description( ) syntax structure inputs the idx of the reference picture list being described and baseIdx indicating the reference picture list used as the basis for describing the subset. The ref_pic_list_subset_description( ) syntax structure includes a syntax element indicating the number of pictures being reduced from the reference picture list description indicated by baseIdx.

The decoding process additionally inputs POC granularity, which is determined by the caller using the temporal_id of the current slice and the information in the sequence parameter set. The decoding process resolves the POC difference of each entry using the indicated or concluded sign, the “short-term” difference, the POC granularity, maxPocDiff, and the ue(v)-coded “long-term” difference. Pictures that are included in the second loop are marked as “used for long-term reference”, whereas other pictures indicated in the syntax structure are marked as “used for short-term reference”.

### ref_pic_list_description( ) syntax

| ref_pic_list_description( idx, numRefPics, pocDiffModeFlag, longTermFlag, maxPocDiff, pocPredFlag ) { | Descriptor |
| --- | --- |
| if( numRefPics > 0 ) { |  |
| for( i = startRefIdx; i < startRefIdx + numRefPics; i++ ) { |  |
| if( pocDiffModeFlag = = 1 ) |  |
| poc_diff_sign[ i ] | u(1) |
| if( maxPocDiff > 1 ) |  |
| poc_diff_param[ i ] | u(v) ¦ ue(v) |
| } |  |
| if( longTermFlag = = 1 ) { |  |
| num_longterm_pics | ue(v) |
| for( j = 0; j < num_longterm_pics; j++ ) { |  |
| abs_diff_ref_pic_idx | ue(v) |
| abs_diff_poc_cycle_minus1 | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

### ref_pic_list_description( ) semantics

poc_diff_sign[ i ] specifies the sign of the POC difference used in the reference picture list description process.

poc_diff_param[ i ] specifies an absolute POC difference used in the reference picture list description process. If maxPocDiff is greater than 0, poc_diff_param[ i ] is u(v) represented by Ceil( Log2( maxPocDiff ) ) bits. Otherwise, poc_diff_param[ i ] is ue(v).

num_longterm_pics specifies the number of reference pictures in the reference picture list that are marked as “used for long-term reference”.

abs_diff_ref_pic_idx specifies an absolute difference compared to a reference index prediction value.

abs_diff_poc_cycle_minus1 specifies an absolute POC difference used in the reference picture list description process.

### ref_pic_list_subset_description( ) syntax

| ref_pic_list_subset_description( idx, baseIdx ) { | Descriptor |
| --- | --- |
| if( NumberOfRefPics( baseIdx ) > 0 ) |  |
| num_ref_pics_reduced | ue(v) |
| } |  |

### ref_pic_list_subset_description( ) semantics

num_ref_pics_reduced indicates the number of reference pictures that the reference picture list indicated by idx has compared to the reference picture list indicated by baseIdx. If num_ref_pics_reduced is not present, it is inferred to be equal to 0.

### Decoding Process

Input to this process is a ref_pic_list_description( ) or ref_pic_list_subset_description( ) structure, variables idx, numRefPics, pocDiffModeFlag, maxPocDiff, pocPredFlag, and pocGranularity, and the reference picture list refPicListX (RefPicList0, RefPicList1, or RefPicListIdle) which is described.

Output of this process is the reference picture list RefPicList0, RefPicList1, or RefPicListIdle having the specified reference picture list index idx.

If idx is -1, the reference picture list description is given in a slice header and is valid for the decoding of the respective slice. If idx is greater than or equal to 0, the reference picture list description is given in a picture parameter set and is decoded when a slice header contains an index referring to it.

If the decoding process is performed for ref_pic_list_subset_description( idx, baseIdx ), this decoding process is first performed for the list identified by baseIdx (i.e., letting idx be equal to baseIdx when running the decoding process). Then the number of reference pictures in the RefPicListX is reduced by num_ref_pics_reduced by excluding reference pictures from the tail of RefPicListX. Let function NumberOfRefPics( idx ) for RefPicListX return NumberOfRefPics( baseIdx ) – num_ref_pics_reduced.

Otherwise, the following applies:

Let function NumberOfRefPics( idx ) for RefPicListX return numRefPics.

If numRefPics is greater than 0, the following ordered steps are performed for each loop entry i:

–	The variable pocDiffSign[ i ] is specified as follows:

–	If poc_diff_sign[ i ] is present, the variable pocDiffSign[ i ] is set equal to poc_diff_sign[ i ].

–	Otherwise, if the input to this process is RefPicList0 or RefPicListIdle, pocDiffSign[ i ] is set equal to 0.

–	Otherwise (the input to this process is RefPicList1), pocDiffSign[ i ] is set equal to 1.

–	If poc_diff_param[ i ] is present, the variable absPocDiff is set equal to poc_diff_param[ i ] + 1. Otherwise, the variable absPocDiff is set equal to 1.

–	The variable refPicPoc[ refPicListX, i ]  is specified by the following pseudo-code:

if( pocPredFlag  = = 0  ||  i = = 0 ) {
	if( pocDiffSign[ i ]  = = 0 )
		refPicPoc[ refPicListX, i ] = 
			PicOrderCnt – absPocDiff * pocGranularity
	else				(QQ-QQ)
		refPicPoc[ refPicListX, i ] = 
			PicOrderCnt + absPocDiff * pocGranularity
} else {
	if( pocDiffSign[ i ]  = = 0 )
		refPicPoc[ refPicListX, i ] = 
			refPicPoc[ refPicListX, i – 1] – absPocDiff * pocGranularity
	else
		refPicPoc[ refPicListX, i ] = 
			refPicPoc[ refPicListX, i – 1] + absPocDiff * pocGranularity
}

The variable cRefIdx is set equal to 0.

For each loop entry j, the following ordered steps are performed:

–	cRefIdx += abs_diff_ref_pic_idx[ j ]

–	The variable refPicPoc[ refPicListX, cRefIdx ] is updated by the following pseudo-code:

if( pocDiffSign[ cRefIdx ]  = = 0 )
	refPicPoc[ refPicListX, cRefIdx ] -= (abs_diff_poc_cycle_minus1 + 1) * maxPocDiff * pocGranularity
else					(QQ-QQ)
	refPicPoc[ refPicListX, cRefIdx ] += (abs_diff_poc_cycle_minus1 + 1) * maxPocDiff * pocGranularity

–	The picture with loop entry cRefIdx is marked as “used for long-term reference”.

All pictures in RefPicListX that are not marked as “used for long-term reference” are marked as “used for short-term reference”.

## Picture Parameter Set

### Summary

Picture parameter set is appended to include one ref_pic_lists_in_pps( ) syntax structure specified below.

The picture parameter set includes any number of reference picture list description elements for reference picture lists 0, 1, and idle. The reference picture list description elements are indexed and the index can be used to refer to them from the slice header.

### Syntax

| ref_pic_lists_pps( ) { | Descriptor |
| --- | --- |
| num_full_list_descriptions | ue(v) |
| if( num_full_list_descriptions > 0 ) { |  |
| poc_diff_mode_flag | u(1) |
| poc_pred_flag | u(1) |
| num_subset_list_descriptions | ue(v) |
| for( i = 0; i < num_full_list_descriptions; i++ ) { |  |
| num_ref_pics_l0_minus1[ i ] | ue(v) |
| ref_pic_list_description( i + num_subset_list_descriptions, // idx num_ref_pics_l0_minus[ i ] + 1, // numRefPics poc_diff_mode_flag, // pocDiffModeFlag 0, // longTermFlag 0, // maxPocDiff = = 0, thus ue(v) coding of poc_diff_mode_param poc_pred_flag // pocPredFlag ) |  |
| num_ref_pics_l1[ i ] | ue(v) |
| ref_pic_list_description( i + num_subset_list_descriptions, num_ref_pics_l1[ i ], poc_diff_mode_flag, 0, 0, poc_pred_flag ) |  |
| ref_pic_list_copy_flag | u(1) |
| } |  |
| for( i = 0; i < num_subset_list_descriptions; i++ ) { |  |
| ref_pic_list_subset_description( i + num_full_list_descriptions, (i / num_full_list_descriptions) * num_full_list_descriptions + (i % num_full_list_descriptions ) // for list 0 |  |
| ref_pic_list_subset_description( i + num_full_list_descriptions, (i / num_full_list_descriptions) * num_full_list_descriptions + (i % num_full_list_descriptions ) // for list 1 |  |
| ref_pic_list_copy_flag | u(1) |
| } |  |
| } |  |
| if( idle_lists_present_flag > 0 ) { |  |
| num_idle_lists | ue(v) |
| for( i = 0; i < num_idle_lists; i++ ) { |  |
| num_ref_pics_idle | ue(v) |
| if( num_ref_pics_idle > 0 ) { |  |
| poc_diff_mode_idle | u(1) |
| ref_pic_list_description( i, num_ref_pics_idle, poc_diff_mode_idle, 0, 0, 1 ) // for idle list |  |
| } |  |
| } |  |
| } |  |
| } |  |

### Semantics

The syntax structure may contain reference picture list description syntax structures for reference picture list 0, reference picture list 1 and the idle reference picture list.

num_full_list_descriptions indicates the number of reference picture list description structures for reference picture list 0 and 1 which do not require other reference picture list description structures in their decoding process.

poc_diff_mode_flag equal to 0 specifies that the default sign is used for POC differences indicated in the reference picture list description syntax for reference picture list 0 and 1. poc_diff_mode_flag equal to 1 specifies that the sign for each POC difference indicated in the reference picture list description syntax for reference picture list 0 and 1 is included in the reference picture list description syntax structure.

poc_pred_flag equal to 0 indicates that the POC differences indicated in the reference picture list description syntax for reference picture list 0 and 1 are relative to the POC of the current picture. poc_pred_flag equal to 1 indicates that the POC differences indicated in the reference picture list description syntax for reference picture list 0 and 1 and for reference indices greates 0 are relative to a POC prediction value.

num_subset_list_descriptions indicates the number of reference picture list description structures for reference picture list 0 and 1 which require other reference picture list description structures in their decoding process.

num_ref_pics_l0_minus1[ i ] + 1 specifies the number of reference pictures in reference picture list 0 for the reference picture list description process.

num_ref_pics_l1[ i ] specifies the number of reference pictures in reference picture list 1 for the reference picture list description process.

ref_pic_list_copy_flag[ i ] equal to 1 specifies that the initial reference picture list 0 is appended by the reference pictures in the reference picture list description for the initial reference picture list 1 and that the initial reference picture list 1 is appended by the reference pictures in the reference picture list description for the initial reference picture list 0.

num_incr_ref_pics_l0 specifies the number of additional reference pictures in reference picture list 0 for the reference picture list description process when compared to the respective base reference picture list description.

num_incr_ref_pics_l1 specifies the number of additional reference pictures in reference picture list 1 for the reference picture list description process when compared to the respective base reference picture list description.

num_idle_lists indicates the number of reference picture list description structures for the idle reference picture lists.

num_ref_pics_idle specifies the number of reference pictures in the idle reference picture list.

poc_diff_mode_idle specifies whether signed or unsigned differences are provided for the idle reference picture list in the reference picture list description process.

## Slice Header

### Summary

The reference picture list modification syntax and decoded reference picture marking syntax are removed as unnecessary. Instead, the slice header may include ref_pic_lists_in_slice_header( ) syntax structure.

The reference picture lists in slice header syntax structure includes reference picture list description structures or points to reference picture list description structures included in the active picture parameter set by their index. Three reference picture list description structures may be included or pointed to:

- a reference picture list description structure for reference picture list 0, for P and B slices

- a reference picture list description structure for reference picture list 1, for B slices

- a reference picture list description structure for the idle reference picture list, for any slices

### Changes in Slice Header Syntax

| ref_pic_lists_in_slice_header( ) |  |
| --- | --- |
| ref_pic_list_modification( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |

### Syntax of Reference Picture Lists in Slice Header

| ref_pic_lists_in_slice_header( ) { | Descriptor |
| --- | --- |
| if( slice_type % 5 != 2 ) { |  |
| ref_pic_l0l1_in_pps_flag | u(1) |
| if( ref_pic_l0l1_in_pps_flag = = 0 ) { |  |
| num_ref_pics_l0_minus1 | ue(v) |
| ref_pic_list_description( -1, // idx num_ref_pics_l0_minus1 + 1, // numRefPics poc_diff_mode_flag, // pocDiffModeFlag 1, // longTermFlag max_shortterm_poc_diff_minus1[ temporal_id ] + 1, // maxPocDiff 0, // pocPredFlag ) // for l0 |  |
| } else |  |
| ref_pic_list_idx | ue(v) |
| } |  |
| if( slice_type % 5 = = 1 ) { |  |
| if( ref_pic_l0l1_in_pps_flag = = 0 ) { |  |
| num_ref_pics_l1_minus1 | ue(v) |
| ref_pic_list_description( -1, // idx num_ref_pics_l1_minus1 + 1, // numRefPics poc_diff_mode_flag, // pocDiffModeFlag 1, // longTermFlag max_shortterm_poc_diff_minus1[ temporal_id ] + 1, // maxPocDiff 0, // pocPredFlag ) // for l1 |  |
| ref_pic_list_copy_flag | u(1) |
| } |  |
| } |  |
| if( idle_lists_present_flag > 0 ) { |  |
| idle_ref_pic_list_in_pps_flag | u(1) |
| if( idle_ref_pic_list_in_pps_flag = = 0 ) { |  |
| num_ref_pics_idle | ue(v) |
| if( num_ref_pics_idle > 0 ) { |  |
| poc_diff_mode_idle | u(1) |
| ref_pic_list_description( -1, num_ref_pics_idle, poc_diff_mode_idle, 1, max_poc_diff_idle_minus1[ temporal_id ] + 1, 0 ) // for idle list |  |
| } |  |
| } else |  |
| idle_ref_pic_list_idx | ue(v) |
| } |  |
| } |  |

### Semantics of Reference Picture Lists in Slice Header

The syntax structure may contain up to three reference picture list description syntax structures, where the first one is for the initial reference picture list 0 and is present for P and B slices, the second one is for the initial reference picture list 1 and is present for B slices, and the third one is for the idle reference picture list and is present if num_ref_pics_idle is greater than 0. Any one of the three reference picture list description syntax structures may also be indicated by a reference picture list index to a reference picture list description syntax structure included in the active picture parameter set.

ref_pic_l0l1_in_pps_flag equal to 0 indicates that the reference picture list description syntax structure is present for reference picture list 0 and, for B slices, reference picture list 1. ref_pic_l0l1_in_pps_flag equal to 1 indicates that the reference picture list index is provided for reference picture list 0 and, for B slices, reference picture list 1.

num_ref_pics_l0_minus1 +1 specifies the number of reference pictures in reference picture list 0 for the reference picture list description process.

ref_pic_list_idx indicates the reference picture list index for reference picture list description syntax structures specified for reference picture list 0 and for reference picture list 1 (for B slices) in the active picture parameter set

num_ref_pics_l1_minus1 +1 specifies the number of reference pictures in reference picture list 1 for the reference picture list description process.

ref_pic_list_copy_flag equal to 1 specifies that the initial reference picture list 0 is appended by the reference pictures in the reference picture list description for the initial reference picture list 1 and that the initial reference picture list 1 is appended by the reference pictures in the reference picture list description for the initial reference picture list 0.

idle_ref_pic_list_in_pps_flag equal to 0 indicates that the reference picture list description syntax structure is present for the idle reference picture list. idle_ref_pic_list_in_pps_flag equal to 1 indicates that the reference picture list index is provided for the idle reference picture list.

num_ref_pics_idle specifies the number of reference pictures in the idle reference picture list.

poc_diff_mode_idle specifies whether signed or unsigned differences are provided for the idle reference picture list in the reference picture list description process.

idle_ref_pic_list_idx indicates the reference picture list index for reference picture list description syntax structures specified for idle reference picture lists in the active picture parameter set.

## Decoding process for reference picture lists construction and reference picture marking

### Summary

The decoding process presented in this section replaces the reference picture list initialization, reference picture list modification, and decoded reference picture marking processes of HEVC WD.

In the decoding process, the indicated reference picture list description structures for reference picture list 0, 1, and idle are decoded. The temporal_id of the current slice and the POC granularity information in the sequence parameter set are used in the decoding process for the reference picture list description structure. In order to code efficiently reference picture lists resembling initial reference picture lists of HEVC WD, there is a specific ref_pic_list_copy_flag, which enables to append the indicated reference picture list 1 to the end of reference picture list 0 and vice versa.

### Specification Text

This process is invoked at the beginning of the decoding process for each P, B, or I slice.

Reference pictures are addressed through reference indices as specified in subclause [XREF]. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0. In addition, there is an idle reference picture list RefPicListIdle, which may contain zero or more reference pictures that are not used as reference for the current picture but may be used for reference in subsequent pictures in decoding order.

Each reference picture in the reference picture lists RefPicList0, RefPicList1, and RefPicListIdle as associated with the variable RefPicPoc[ lX, refIdx ], where lX is one of l0, l1, or lIdle and refIdx is the reference index within the reference picture list. The derivation of RefPicPoc[ lX, refIdx ] is specified below.

At the beginning of the decoding process for each slice, the reference picture lists are derived and reference pictures are marked as specified by the following ordered steps:

- RefPictList0, RefPictList1, and RefPicListIdle are initially empty.

- For P and B slices, the reference picture list RefPicList0 is initialised through the reference picture list description process with RefPicList0, the ref_pic_list_description( ) structure or ref_pic_list_subset_description( ) for reference picture list 0 that is either indicated or included in the slice header, and pocGranularity equal to (poc_granularity_minus1[ temporal_id ] + 1) as input.

- For B slices, the reference picture list RefPicList1 is initialised through the reference picture list description process with RefPicList1 the ref_pic_list_description( ) structure or ref_pic_list_subset_description( ) for reference picture list 1 that is either indicated or included in the slice header, and pocGranularity equal to (poc_granularity_minus1[ temporal_id ] + 1) as input.

- If the current slice is a B slice and ref_pic_list_copy_flag is equal to 1, RefPicList0 is copied to tempRefPicList0 and RefPicList1 is copied to tempRefPicList1. Then, RefPicList0 is reset to contain a concatenation of tempRefPicList0 and tempRefPicList1, and RefPicList1 is reset to contain a concatenation of tempRefPicList1 and tempRefPicList0.

- If idle_lists_present_flag is equal to 1, the following applies. pocGranularity for the idle reference picture list is set equal to poc_granularity_minus1[ temporal_id – poc_granularity_idle_idx[ temporal_id ] ] + 1. Then, RefPicListIdle is specified through reference picture list description process with RefPicListIdle, the ref_pic_list_description( ) structure for the idle reference picture list that is either indicated or included in the slice header, and the derived pocGranularity as input.

- Any picture that is in no one of the idle reference picture list, reference picture list RefPicList0, and for B slices RefPicList1 is marked as "unused for reference".

- The number of entries in the reference picture list RefPicList0 is set to the maximum of the number of entries in the reference picture list RefPicList0 and num_ref_idx_l0_active_minus1 + 1, and for B slices the number of entries in the reference picture list RefPicList1 is set to the maximum of the number of entries in the reference picture list RefPicList1 and num_ref_idx_l1_active_minus1 + 1.

NOTE – A reference picture may appear at more than one index in the reference picture lists RefPicList0 or RefPicList1.

## Miscellaneous Remarks

The proposal may be used with any picture order count type or coding mode.

# Examples

This section includes examples how to use the proposed syntax structures and elements for representing reference picture lists. The section also includes an analysis of the required number of bits to represent reference picture lists.

The required number of bits to represent reference picture lists is counted by assuming that all lists are described in the picture parameter set. The bit count is analyzed using a table which describes the loops of the ref_pic_lists_pps( ) structure and their bitcount:

- the reference list index for reference picture list 0 and 1 (idx);

- the bit count used for describing the reference picture lists 0 and 1 (bit count);

- the reference picture lists 0 and 1 (L0 and L1, respectively), indicated by relative POC value differences at POC granularity 1, where the POC values in paranthesis indicate the reference pictures, which included in the reference picture list description but excluded from the final list by num_ref_idx_lX_active_minus1 syntax element;

- the essential contents of the reference picture list description for list 0 and list 1 (L0 descr. and L1 descr., respectively), where

  - the values indicated poc_diff_param[ ] values in ref_pic_list_description( ) structure (assuming that poc_granularity_minus1[ 0 ] is set to 1 and poc_granularity_minus1[ 1 ] is set to 0); and

  - “idx X – Y ref pics” indicates that the reference picture list description is a subset relative to reference picture list description X with Y reference pictures reduced from the tail.

- the value of ref_pic_list_copy_flag (copy_flag); and

- the pseudo code and bit count breakdown of the syntax elements in the the loops of the ref_pic_lists_pps( ) structure and the containing ref_pic_list_description( ) or ref_pic_list_subset_description( ) structures.

## Random Access Common Condition

The values of the syntax elements included in the main level of the ref_pic_lists_pps( ) structure and their bitcount is as follows:

- poc_diff_mode_flag = 0 (1b)

- poc_pred_flag = 1 (1b)

- num_full_list_descriptions = 8 (7b)

- num_subset_list_descriptions = 4 (5b)

| idx | bit count | L0 | L1 | L0 descr. | L1 descr. | copy_flag | Pseudo code and bit count breakdown |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 23 | -1 -3 | 1 3 (5 7) | 0 1 | 0 1 1 1 | 0 | num_ref_pics_l0_minus1 = 1 (3b) poc_diff_param[0] = 0 (1b) poc_diff_param[0] = 1 (3b) num_ref_pics_l1 = 4 (5b) poc_diff_param[0] = 0 (1b) poc_diff_param[1] = 1 (3b) poc_diff_param[2] = 1 (3b) poc_diff_param[2] = 1 (3b) ref_pic_list_copy_flag = 0 (1b) |
| 1 | 15 | -8 -10 -12 -14 | -8 -10 -12 -14 | 3 0 0 0 | - | 1 | num_ref_pics_l0_minus1 = 3 (5b) poc_diff_param[0] = 3 (5b) poc_diff_param[1] = 0 (1b) poc_diff_param[2] = 0 (1b) poc_diff_param[3] = 0 (1b) num_ref_pics_l1 = 0 (1b) ref_pic_list_copy_flag = 1 (1b) |
| 2 | 14 | -4 -6 | 4 -4 | 1 0 | 1 | 1 | num_ref_pics_l0_minus1 = 1 (3b) poc_diff_param[0] = 1 (3b) poc_diff_param[1] = 0 (1b) num_ref_pics_l1 = 1 (3b) poc_diff_param[0] = 1 (3b) ref_pic_list_copy_flag = 1 (1b) |
| 3 | 13 | -2 -4 | 2 6 | 0 0 | 0 1 | 0 | num_ref_pics_l0_minus1 = 1 (3b) poc_diff_param[0] = 0 (1b) poc_diff_param[1] = 0 (1b) num_ref_pics_l1 = 2 (3b) poc_diff_param[0] = 0 (1b) poc_diff_param[1] = 1 (3b) ref_pic_list_copy_flag = 0 (1b) |
| 4 | 10 | -2 -4 (-6) | 2 -2 | 0 0 0 | 0 | 1 | num_ref_pics_l0_minus1 = 2 (3b) poc_diff_param[0] = 0 (1b) poc_diff_param[1] = 0 (1b) poc_diff_param[2] = 0 (1b) num_ref_pics_l1 = 1 (3b) poc_diff_param[0] = 0 (1b) ref_pic_list_copy_flag = 1 (1b) |
| 5 | 20 | -1 -3 | 1 3 (5) | 0 1 | 0 1 1 | 0 | num_ref_pics_l0_minus1 = 1 (3b) poc_diff_param[0] = 0 (1b) poc_diff_param[1] = 1 (3b) num_ref_pics_l1 = 3 (5b) poc_diff_param[0] = 0 (1b) poc_diff_param[1] = 1 (3b) poc_diff_param[2] = 1 (3b) ref_pic_list_copy_flag = 0 (1b) |
| 6 | 15 | -1 -3 | 1 3 | 0 1 | 0 1 | 0 | num_ref_pics_l0_minus1 = 1 (3b) poc_diff_param[0] = 0 (1b) poc_diff_param[1] = 1 (3b) num_ref_pics_l1 = 2 (3b) poc_diff_param[0] = 0 (1b) poc_diff_param[1] = 1 (3b) ref_pic_list_copy_flag = 0 (1b) |
| 7 | 12 | -1 -3 | 1 -1 | 0 1 | 0 | 1 | num_ref_pics_l0_minus1 = 1 (3b) poc_diff_param[0] = 0 (1b) poc_diff_param[1] = 1 (3b) num_ref_pics_l1 = 1 (3b) poc_diff_param[0] = 0 (1b) ref_pic_list_copy_flag = 1 (1b) |
| 8 | 5 | -1 1 | 1 3 (5 7) | idx 0 -1 ref pic | idx 0 | 1 | num_ref_pics_reduced = 1 // l0 (3b) num_ref_pics_reduced = 0 // l1 (1b) ref_pic_list_copy_flag = 1 (1b) |
| 9 | 6 | -8 | -8 | idx 1 -3 ref pic | idx 1 | 1 | num_ref_pics_reduced = 3 // l0 (5b) ref_pic_list_copy_flag = 1 (1b) |
| 10 | 5 | -4 4 | 4 -4 | idx 2 -1 ref pic | idx 2 | 1 | num_ref_pics_reduced = 1 // l0 (3b) num_ref_pics_reduced = 0 // l1 (1b) ref_pic_list_copy_flag = 1 (1b) |
| 11 | 5 | -2 2 | 2 6 | idx 3 -1 ref pic | idx 3 | 1 | num_ref_pics_reduced = 1 // l0 (3b) num_ref_pics_reduced = 0 // l1 (1b) ref_pic_list_copy_flag = 1 (1b) |
| total | 143 |  |  |  |  |  |  |

By summing up the values above, the bit count used for reference picture list description in PPS is 157 bits.

## Low Delay Common Condition

The values of the syntax elements included in the main level of the ref_pic_lists_pps( ) structure and their bitcount is as follows:

- poc_diff_mode_flag = 0 (1b)

- poc_pred_flag = 1 (1b)

- num_full_list_descriptions = 4 (5b)

- num_subset_list_descriptions = 9 (7b)

The following table describes the loops of the ref_pic_lists_pps( ) structure and their bitcount:

- the reference list index for reference picture list 0 and 1 (idx);

- the bit count used for describing the reference picture lists 0 and 1 (bit count);

- the reference picture lists 0 and 1 (L0 and L1, respectively), indicated by relative POC value differences at POC granularity 1, where the POC values in paranthesis indicate the reference pictures, which included in the reference picture list description but excluded from the final list by num_ref_idx_lX_active_minus1 syntax element;

- the essential contents of the reference picture list description for list 0 and list 1 (L0 descr. and L1 descr., respectively), where

  - the values indicated poc_diff_param[ ] values in ref_pic_list_description( ) structure (assuming that poc_granularity_minus1[ 0 ] is set to 1 and poc_granularity_minus1[ 1 ] is set to 0); and

  - “idx X – Y ref pics” indicates that the reference picture list description is a subset relative to reference picture list description X with Y reference pictures reduced from the tail.

- the value of ref_pic_list_copy_flag (copy_flag); and

- the pseudo code and bit count breakdown of the syntax elements in the the loops of the ref_pic_lists_pps( ) structure and the containing ref_pic_list_description( ) or ref_pic_list_subset_description( ) structures.

| idx | bit count | L0 | L1 | L0 descr. | L1 descr. | copy_flag | Pseudo code and bit count breakdown |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 23 | -1 -5 -9 -13 | -1 -5 -9 -13 | 0 3 3 3 | - | 1 | num_ref_pics_l0_minus1 = 4 (5b) poc_diff_param[0] = 0 (1b) poc_diff_param[1] = 3 (5b) poc_diff_param[2] = 3 (5b) poc_diff_param[3] = 3 (5b) num_ref_pics_l1 = 0 (1b) ref_pic_list_copy_flag = 1 (1b) |
| 1 | 19 | -1 -2 -6 -10 | -1 -2 -6 -10 | 0 0 3 3 | - | 1 | num_ref_pics_l0_minus1 = 4 (5b) poc_diff_param[0] = 0 (1b) poc_diff_param[1] = 0 (1b) poc_diff_param[2] = 3 (5b) poc_diff_param[3] = 3 (5b) num_ref_pics_l1 = 0 (1b) ref_pic_list_copy_flag = 1 (1b) |
| 2 | 19 | -1 -3 -7 -11 | 1 -3 -7 -11 | 0 1 3 2 | - | 1 | num_ref_pics_l0_minus1 = 4 (5b) poc_diff_param[0] = 0 (1b) poc_diff_param[1] = 1 (3b) poc_diff_param[2] = 3 (5b) poc_diff_param[3] = 2 (3b) num_ref_pics_l1 = 0 (1b) ref_pic_list_copy_flag = 1 (1b) |
| 3 | 21 | -1 -4 -8 -12 | -1 -4 -8 -12 | 0 2 3 3 | - | 1 | num_ref_pics_l0_minus1 = 4 (5b) poc_diff_param[0] = 0 (1b) poc_diff_param[1] = 2 (3b) poc_diff_param[2] = 3 (5b) poc_diff_param[3] = 3 (5b) num_ref_pics_l1 = 0 (1b) ref_pic_list_copy_flag = 1 (1b) |
| 4 | 4 | -1 -5 -9 | -1 -5 -9 | idx 0 -1 ref pic | - | 1 | num_ref_pics_reduced = 1 // l0 (3b) ref_pic_list_copy_flag = 1 (1b) |
| 5 | 4 | -1 -2 -6 | -1 -2 -6 | idx 1 -1 ref pic | - | 1 | num_ref_pics_reduced = 1 // l0 (3b) ref_pic_list_copy_flag = 1 (1b) |
| 6 | 4 | -1 -3 -7 | -1 -3 -7 | idx 2 -1 ref pic | - | 1 | num_ref_pics_reduced = 1 // l0 (3b) ref_pic_list_copy_flag = 1 (1b) |
| 7 | 4 | -1 -4 -8 | -1 -4 -8 | idx 3 -1 ref pic | - | 1 | num_ref_pics_reduced = 1 // l0 (3b) ref_pic_list_copy_flag = 1 (1b) |
| 8 | 4 | -1 -5 | -1 -5 | idx 4 -1 ref pic | - | 1 | num_ref_pics_reduced = 1 // l0 (3b) ref_pic_list_copy_flag = 1 (1b) |
| 9 | 4 | -1 -2 | -1 -2 | idx 5 -1 ref pic | - | 1 | num_ref_pics_reduced = 1 // l0 (3b) ref_pic_list_copy_flag = 1 (1b) |
| 10 | 4 | -1 -3 | -1 -3 | idx 6 -1 ref pic | - | 1 | num_ref_pics_reduced = 1 // l0 (3b) ref_pic_list_copy_flag = 1 (1b) |
| 11 | 4 | -1 -4 | -1 -4 | idx 7 -1 ref pic | - | 1 | num_ref_pics_reduced = 1 // l0 (3b) ref_pic_list_copy_flag = 1 (1b) |
| 12 | 4 | -1 | -1 | idx 8 -1 ref pic | - | 1 | num_ref_pics_reduced = 1 // l0 (3b) ref_pic_list_copy_flag = 1 (1b) |
| total | 118 |  |  |  |  |  |  |

By summing up the values above, the bit count used for reference picture list description in PPS is 132 bits.

# Patent rights declaration(s)

Nokia Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).