| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G644 |
| --- | --- |

| Title: | Multi-level significance maps for Large Transform Units |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nguyen Nguyen, Tianying Ji, Dake He, Gaëlle Martin-Cocher, and Lin Song 185 Columbia St. W. Waterloo, ON, Canada N2L 5Z5 | Tel: Email: | +1-519-8887465 nnguyen@rim.com tiji@rim.com dhe@rim.com gmartincocher@rim.com |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

A multi-level structure to encode significance coefficient flags for large transform units of sizes greater than 8x8 is proposed to reduce decoding complexity while maintaining compression performance. To accommodate this structural change, four additional contexts for luma (2) and chroma (2) are introduced. In common test conditions against HM4.0 anchor, the multilevel structure reduces the number of bins for significant map of large transform units by 15%, 12% and 30% on average for AI-HE, RA-HE and LB-HE respectively; which represents between 3% and 4% reduction of the total number of bins decoded by BAC. The average BD-rate difference against HM4.0 is within 0.1%. Tests are currently being run with RDOQ off and the results will be updated.

# Proposal

## Multiple Level Significance Maps

The significance map of a TU (transform unit) indicates the positions of non-zero coefficients in the TU.  This contribution proposes a two –level structure of significance maps: level zero (L0) and level one (L1).

L1 significance map: Large TUs (of sizes 16x16 and 32x32) are evenly partitioned into non-overlapping 4x4 and 8x8 coefficient groups, respectively. Therefore, one 16x16 TU is partitioned into 16 coefficient groups, and one 32x32 TU is partitioned into 16 coefficient groups. L1 significant map indicates the positions of coefficient groups that do not have nonzero coefficient. Accordingly, for each of the coefficient groups, a binary valued significant_coeffgroup_flag is set to 0 indicating that the coefficient group does not contain non-zero coefficients.  In the following, we use raster scan order to index and identify coefficient groups and transform coefficients.  For example, in a TU, we have the following coefficient groups:

- 1      2     3

4      5      6     7

8      9    10    11

12   13   14    15

L0 significance map: An L0 significance map for a large TU is represented by binary-valued significant_coeff_flags, as defined in the current working draft of the HEVC standard, JCTVC-F803 [2].

The advantage of using the proposed two-level significance map coding method is that we no longer need to code the significance flags of transform coefficients that are in a coefficient group with significant_coeffgroup_flag set to 0 (these coefficients can be inferred to be 0 by the decoder if it has the L1 significance map). Indeed, our experimental results show that the number of bins decoded is decreased substantially for large format sequences, with minimal impact on coding efficiency.

## Context modeling

Two contexts for luma component and two contexts for chroma components are introduced to encode significant_coeffgroup_flags.  As a result, the total number of contexts is increased by 4.

The context of a significant_coeffgroup_flag can be 0 or 1, and is determined as follows. Let x denote the current significant_coeffgroup_flag to be encoded, let a denote its immediate neighbor to the right, and let b denote its immediate neighbor below, as shown below.

x a

b

Then the context for x is equal to max{a, b}. Note that when a (or b, respectively) is not available when x is at the TU’s right (or bottom, respectively) boundary, a (or b, respectively) is assumed to be 0.

Further note since the significant_coeffgroup_flag for the group including the last position is always 1 and thus does not need to be encoded. In addition, there are two special cases where the significant_coeffgroup_flag is assumed to be 1 and does not need to be encoded.

Case 1: The coefficient group is the first group including DC.

Case 2: a=b=1, i.e.,

x 1

1

The context derivation for significant_coeff_flag remains unchanged from the current working draft, JCTVC-F803 [2].

# Encoding and decoding process

For a large TU containing nonzero coefficients, significant_coeffgroup_flags are encoded and decoded after last_significant_coeff_x and last_significant_coeff_y, and before significant_coeff_flags. After decoding significant_coeffgroup_flags, the significant_coeff_flags in a group with significant_coeffgroup_flag = 0 are all known to be zeros, and only those significant_coeff_flags in groups with significant_coeffgroup_flag =1 need to be decoded.

To illustrate how to use the two-level structure and significant_coeffgroup_flags, we use an example of a 16x16 TU. This example is taken from RaceHorses 240p sequence with QP=27.

Below are the quantized transform coefficients in the TU, where

- The last significant coefficient is labeled red.

- The zero coefficients after the last position are in light gray.

- The coefficients groups 3, 7, 12, 13, and 14 contain coefficient positions before the last significant coefficient position and have no nonzero coefficients. These coefficient groups are colored in green and have their significant_coeffgroup_flags set to 0.

- The remaining coefficient groups have their significant_coeffgroup_flags set to 1.

---------------------------------------------------------

|  5  2  0  1 |  1  0  0  0 |  1  1  1  0 |  0  0  0  0 |

|  5  0  2  1 |  2  0  1  1 |  0  2  1  0 |  0  0  0  0 |

|  0  1  0  0 |  1  0  1  1 |  0  1  1  1 |  0  0  0  0 |

|  0  0  0  1 |  1  1  1  1 |  0  0  1  0 |  0  0  0  0 |

---------------------------------------------------------

|  1  1  0  0 |  1  0  1  0 |  0  1  0  1 |  0  0  0  0 |

|  2  1  0  1 |  0  0  1  0 |  1  0  0  1 |  0  0  0  0 |

|  0  0  1  0 |  0  0  1  0 |  1  0  0  0 |  0  0  0  0 |

|  1  0  0  1 |  1  0  1  0 |  0  0  0  0 |  0  0  0  0 |

---------------------------------------------------------

|  0  1  1  1 |  0  1  0  1 |  1  1  1  0 |  0  1  0  0 |

|  0  0  1  0 |  0  1  1  1 |  0  0  0  1 |  0  0  0  0 |

|  1  0  0  1 |  0  0  0  0 |  1  1  0  0 |  1  0  0  0 |

|  0  0  1  0 |  0  0  0  0 |  1  1  0  0 |  0  0  0  0 |

---------------------------------------------------------

|  0  0  0  0 |  0  0  0  0 |  0  0  0  0 |  0  0  0  0 |

|  0  0  0  0 |  0  0  0  0 |  0  0  0  0 |  0  0  0  0 |

|  0  0  0  0 |  0  0  0  0 |  0  0  0  0 |  0  0  0  0 |

|  0  0  0  0 |  0  0  0  0 |  0  0  0  0 |  0  0  0  0 |

---------------------------------------------------------

In this example, we see that all those 69 positions labeled green have significant_coeff_flags to be encoded in HM4.0 anchor. In the two-level structure, the encoding of these 69 significant_coeff_flags can be saved and instead we encode 10 singificant_coeffgroup_flags instead (labeled in green and yellow), leading to the reduction of 59 bins.

1	1	1	0

1	1	1	0

1	1	1	1

0	0	0	0

# Results

In this section, we report experimental results of the proposal (integrated into HM4.0) against HM4.0 anchor in common test conditions [4].

Tables 4.1 and 4.2 below compare the number of bins (decoded by BAC) in our proposal against HM4.0 anchor. An entry in the column entitled “Total” is calculated as follows:

(Total bins in the proposal – Total bins in HM4.0 anchor)/ Total bins in HM4.0 anchor x 100%

An entry in the column entitled “Sig” is calculated only for bins decoded by BAC in the proposal and HM4.0 anchor for significance maps. An entry in the column entitled “4, 8” is calculated for bins decoded by BAC in the proposal and HM4.0 anchor for significance maps of small TUs of sizes 4x4 and 8x8. Finally an entry in the column entitled “16, 32” is  calculated for bins decoded by BAC in the proposal and HM4.0 anchor for significance maps of large TUs of sizes 8x8 and 32x32.

Table 4.1: Numbers of bins decoded by BAC in the proposal against those in HM 4.0 anchor

|  | All Intra HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total | Sig | 4,8 | 16, 32 |
| Class A | -2.4% | -6.0% | -0.2% | -7.4% |
| Class B | -3.9% | -9.7% | -0.3% | -24.5% |
| Class C | -2.2% | -7.1% | -0.3% | -24.8% |
| Class D | -1.7% | -5.5% | -0.3% | -21.9% |
| Class E | -4.0% | -13.2% | -0.6% | -18.9% |
| Class F |  |  |  |  |
| Overall | -3.1% | -8.1% | -0.3% | -14.6% |

|  | Random Access HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total | Sig | 4,8 | 16, 32 |
| Class A | -2.9% | -6.0% | -0.4% | -6.4% |
| Class B | -4.2% | -9.9% | 0.6% | -29.8% |
| Class C | -4.0% | -13.2% | -0.5% | -36.3% |
| Class D | -3.2% | -10.7% | 0.1% | -41.8% |
| Class E |  |  |  |  |
| Class F |  |  |  |  |
| Overall | -3.4% | -7.8% | 0.3% | -11.7% |

|  | Low delay B HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total | Sig | 4,8 | 16, 32 |
| Class A |  |  |  |  |
| Class B | -4.2% | -9.3% | 0.7% | -28.9% |
| Class C | -4.8% | -14.2% | -0.4% | -36.8% |
| Class D | -2.5% | -7.7% | 0.2% | -36.3% |
| Class E | -4.7% | -14.0% | -2.7% | -20.1% |
| Class F |  |  |  |  |
| Overall | -4.2% | -10.1% | 0.4% | -29.6% |

Table 4.2: Numbers of bins decoded by BAC in the proposal against those in HM 4.0 anchor with RDOQ off

|  | All Intra HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total | Sig | 4,8 | 16, 32 |
| Class A | -4.3% | -9.8% | 0.2% | -12.0% |
| Class B | -6.1% | -12.9% | 0.4% | -23.6% |
| Class C | -2.6% | -7.4% | -0.3% | -23.6% |
| Class D | -2.1% | -6.2% | -0.1% | -20.8% |
| Class E | -4.8% | -13.7% | -0.6% | -18.9% |
| Class F |  |  |  |  |
| Overall | -4.9% | -11.2% | 0.2% | -17.9% |

|  | Random Access HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total | Sig | 4,8 | 16, 32 |
| Class A | -3.6% | -7.5% | 0.7% | -8.2% |
| Class B | -7.5% | -19.3% | 1.3% | -33.2% |
| Class C | -4.8% | -15.1% | 0.6% | -41.0% |
| Class D | -2.6% | -8.6% | 0.9% | -38.4% |
| Class E |  |  |  |  |
| Class F |  |  |  |  |
| Overall | -4.6% | -10.6% | 0.9% | -13.6% |

|  | Low delay B HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total | Sig | 4,8 | 16, 32 |
| Class A |  |  |  |  |
| Class B | -9.2% | -20.9% | 2.1% | -33.6% |
| Class C | -6.1% | -16.5% | 1.1% | -40.1% |
| Class D | -2.8% | -8.4% | 1.1% | -39.2% |
| Class E | -5.5% | -15.9% | -0.1% | -24.0% |
| Class F |  |  |  |  |
| Overall | -7.7% | -18.9% | 1.6% | -34.1% |

Table 4.3 below shows that the BD-rates of the proposal against HM-4.0 anchor (a negative value indicates gain; and a positive value indicates loss).

Table 4.3: BD-rate vs. HM-4.0 with RDOQ on

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | -0.5% | -0.3% |
| Class B | -0.1% | -0.6% | -0.6% |
| Class C | 0.0% | -0.2% | -0.3% |
| Class D | 0.0% | -0.3% | -0.4% |
| Class E | 0.0% | -0.5% | -0.4% |
| Class F |  |  |  |
| Overall | 0.0% | -0.4% | -0.4% |
|  | 0.0% | -0.4% | -0.4% |
| Enc Time[%] |  |  |  |
| Dec Time[%] | 99.79% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.1% | -0.1% | -0.4% |
| Class B | -0.1% | -0.3% | -0.3% |
| Class C | 0.0% | -0.2% | -0.2% |
| Class D | 0.0% | -0.2% | 0.0% |
| Class E |  |  |  |
| Class F |  |  |  |
| Overall | -0.1% | -0.2% | -0.3% |
|  | -0.1% | -0.2% | -0.3% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | 99.73% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.5% | -0.5% |
| Class C | 0.2% | -0.2% | -0.4% |
| Class D | 0.1% | 0.3% | 0.1% |
| Class E | 0.1% | -0.6% | -0.1% |
| Class F |  |  |  |
| Overall | 0.1% | -0.2% | -0.3% |
|  | 0.1% | -0.2% | -0.3% |
| Enc Time[%] |  |  |  |
| Dec Time[%] | 99.83% |  |  |

Table 4.4 below shows that the BD-rate performance of the proposal against HM-4.0 anchor with RDOQ off (a negative value indicates gain, and a positive value indicates loss).

Table 4.4: BD-rate vs. HM-4.0 with RDOQ off

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% |
| Class B | -0.1% | -0.2% | -0.2% |
| Class C | -0.1% | -0.1% | -0.1% |
| Class D | 0.0% | -0.2% | -0.1% |
| Class E | -0.1% | -0.2% | -0.3% |
| Overall | 0.0% | -0.1% | -0.1% |

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.1% | 0.0% | -0.1% |
| Class B | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.1% | -0.1% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% |
| Overall | #VALUE! | #VALUE! | #VALUE! |

|  | Low delay B HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.2% | 0.2% | 0.1% |
| Class D | 0.1% | -0.2% | 0.3% |
| Class E | 0.1% | -0.9% | 0.3% |
| Overall | #VALUE! | #VALUE! | #VALUE! |

Note: For the RDOQ off RA-HE and LB-HE tests, the result for one class B sequence is not available yet. This will be updated later.

To evaluate the value of these results, consider [3], which is adopted into the current working draft [2]. The proposal in [3] increases the number of bypass bins by on average, between 1% and 4% (note that bypass bins are still part of the bitstream).  In comparison, this proposal decreases the number of bins decoded by BAC by on average, between 3% and 4% and allows the BAC decoder (or the BAC encoder, respectively) to skip decoding (or encoding, respectively) the bins altogether, all while having minimal impact on coding efficiency.

# Change to the HEVC text spec.

## Proposed changes to “7.4.10 Residual coding CABAC semantics

(The proposed change goes in front of the section of significant_coeff_flag[ xC ][ yC ]) Note: in significant_coeff_flag semantic, block should be changed to unit.

significant_coeffgroup_flag[ xCG ][ yCG ] specifies for the coefficient group position ( xCG, yCG ), 0 <= xCG, yCG < 4, within the current 16x16 or 32x32 transform unit whether the corresponding coefficient group at location ( xCG, yCG ) has non-zero significant coefficient flags as follows. (A coefficient group at location (xCG, yCG) consists of 16 transform coefficients at locations {(xC, yC): 4*xCG <= xC < 4*(xCG+1), 4*yCG <= yC < 4*(yCG+1)}  within a 16x16 transform unit or consists of 64 transform coefficients at locations {(xC, yC): 8*xCG <= xC < 8*(xCG+1), 8*yCG <= yC < 8*(yCG+1)}  within a 32x32 transform unit. One 16x16 transform unit has 16 4x4 coefficient groups, and one 32x32 transform unit has 16 8x8 coefficient groups)

- If significant_coeffgroup_flag[ xCG ][ yCG ] is equal to 0, the number of nonzero significant coefficient flags in the coefficient group at location ( xCG, yCG ) is set equal to 0;

- Otherwise (significant_coeffgroup_flag[ xCG ][ yCG ] is equal to 1), the number of nonzero significant coefficient flags  in the coefficient group at location ( xCG, yCG ) is non-zero, except for the following special cases:

  - The significant_coeffgroup_flag[ 0 ][ 0 ] at the first coefficient group position (0, 0) is inferred to be equal to 1;

  - The significant_coeffgroup_flag[xCG][yCG], 0 <= xCG, yCG < 3, at the coefficient group position ( xCG, yCG ) is inferred to be equal to 1 if   significant_coeffgroup_flag[xCG][yCG+1] = 1 and 
significant_coeffgroup_flag[xCG+1][yCG] = 1.

Note that when significant_coeffgroup_flag[ xCG ][ yCG ] is not present, it is inferred to be equal to 0.

significant_coeffgroup_flag[ xCG ][ yCG ] doesn’t apply to 4x4 and 8x8 transform units.

## Proposed changes to “7.3.10 Residual Coding CABAC Syntax”

| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| last_significant_coeff_x | ae(v) |
| last_significant_coeff_y | ae(v) |
| numCoeff = 0 |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 1 ] |  |
| if (log2TrafoSize > 3) { |  |
| log2CoeffGroupSize = log2trafoSize-2 |  |
| numCoeffGroup = 0 |  |
| for (xCG = 0; xCG < 4; xCG++) |  |
| for (yCG = 0; yCG < 4; yCG++) |  |
| numNonZeroCG [ xCG ][ yCG ] = 0 |  |
| } |  |
| while( ( xC != last_significant_coeff_x ) ¦¦ ( yC != last_significant_coeff_y ) ) { |  |
| numCoeff++ |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ numCoeff ][ 1 ] |  |
| if (log2TrafoSize > 3) { |  |
| if ( ( xC % (1<< log2CoeffGroupSize) == 0 ) && ( yC % (1 << log2CoeffGroupSize) == 0 ) ) |  |
| numCoeffGroup++ |  |
| } |  |
| } |  |
| if (log2TrafoSize > 3) { |  |
| xCGLast = last_significant_coeff_x / (1 << log2CoeffGroupSize) |  |
| yCGLast = last_significant_coeff_y / (1 << log2CoeffGroupSize) |  |
| significant _coeffgroup_flag[ xCGLast ][ yCGLast ] = 1 |  |
| significant_coeffgroup_flag[ 0 ][ 0 ] = 1 |  |
| for( n = numCoeffGroup; n > 0; n-- ) { |  |
| xCG = ScanOrder[ 0 ][ 0 ][ scanIdx ][ n ][ 0] |  |
| yCG = ScanOrder[ 0 ][ 0 ][ scanIdx ][ n ][ 1] |  |
| if (!(xCG == xCGLast && yCG == yCGLast)) |  |
| significant_coeffgroup_flag[ xCG][ yCG ] | ae(v) |
| } |  |
| } |  |
| for( n = numCoeff − 1; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ scanIdx ][ n ][ 1 ] |  |
| If ( log2TrafoSize > 3) { |  |
| xCG = xC >>(log2CoeffGroupSize) |  |
| yCG = yC >>(log2CoeffGroupSize) |  |
| if( significant_coeffgroup_flag[ xCG ][ yCG ] ) { |  |
| if( ( xC % (1<< log2CoeffGroupSize) == 0 ) && ( yC % (1 << log2CoeffGroupSize) == 0 ) && (numNonZeroCG[ xCG ][ yCG ] == 0) && n != 0 ) |  |
| significant_coeff_flag[ xC ][ yC ] = 1 |  |
| else { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| if( significant_coeff_flag[ xC ][ yC]) |  |
| numNonZeroCG[ xCG ][ yCG ]++ |  |
| } |  |
| } |  |
| else { |  |
| significant_coeff_flag[xC][yC] = 0 |  |
| } |  |
| } else |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| } |  |
| numLastSubset = numCoeff >> 4 |  |
| for( i = numLastSubset − 1; i >= 0; i− − ) { |  |
| offset = i << 4 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset` ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n− − ) { |  |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| xC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ log2TrafoSize − 2 ][ 0 ][ n + offset` ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = 0 |  |
| } |  |
| } |  |
| } |  |

## Proposed changes to “9.3.3.1.1.x Derivation of ctxIdxInc for the syntax element significant_subgroup_flag”

Inputs to this process are the current coefficient group scan position ( xCG , yCG ), and the previously decoded bins of the syntax element significant_coeffgroup_flag.

Output of this process is ctxIdxInc.

The variable sigCtx depends on the current position ( xCG, yCG ), and two previously decoded bins of the syntax element significant_coeffgroup_flag. For the derivation of sigCtx, the following applies.

- If xCG is equal to 3, and yCG is equal to 3, sigCtx is derived as follows.

sigCtx  = 44

- If xCG is less than 3, and yCG is equal to 3, sigCtx is derived as follows.

sigCtx  = 44 + significant_group_flag[ xCG+ 1 ][ yCG ]

- If xCG is equal to 3, and yCG is less than 3, sigCtx is derived as follows.

sigCtx  = 44 + significant_group_flag[ xCG ][ yCG + 1 ]

- If xCG is less than 3, and yCG is less than 3, sigCtx is derived as follows.

sigCtx  =  44 + max{significant_group_flag[ xCG + 1 ][ yCG ], significant_group_flag[ xCG ][ yCG + 1 ]}

The context index increment ctxIdxInc is derived using the color component index cIdx, the transform block size log2TrafoSize and sigCtx as follows.

- If cIdx is equal to 0, ctxIdxInc is derived as follows.

ctxIdxInc  =  ( ( 5  −  log2TrafoSize )  *  16 )  +  sigCtx

- Otherwise (cIdx is greater than 0), ctxIdxInc is derived as follows.

ctxIdxInc  =  64  +  ( ( 4  −  log2TrafoSize )  *  16 )  +  sigCtx

# References

[1] D. Marpe, H. Schwarz, and T. Wiegand, “Context-based adaptive binary arithmetic coding in the h.264/AVC video compression standard,” IEEE Transactions on Circuits and Systems for Video Technology, 13(7):620–636, Jul. 2003.

[2] B. Bross, W-J Han, J-R Ohm, G. J. Sullivan, and T. Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 6th Meeting, Torino, July 2011.

[3] T. Nguyen, T. Winken, D. Marpe et al., “Reduced-complexity entropy coding of transform coefficient levels using a combination of VLC and PIPE”, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 4th Meeting, Daegu, January 2011.

[4] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 6th Meeting, Torino, July 2011.

# Patent rights declaration(s)

Research In Motion Ltd may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).