| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G646 |
| --- | --- |

| Title: | Non-CE6a: Crosscheck of JCTVC-G244 - Luma-based chroma prediction - Model correction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi | Tel: Email: | +1 214 480 1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution presents results of cross-verification of JCTVC-G244 which proposes the use of a set of thresholds to determine when the alpha values in LM mode is not reliable so that alternate alpha values could be used.  The thresholds and alternate alpha information (index) are transmitted in slice header. The PSNR/bit-rate numbers obtained were compared to those provided by proponents and a match was reported to be observed. The Y, U, V BD-Rate savings compared to HM-4.0 are respectively: AI-HE: 0.0%, 3.9%, 4.3%, AI-LC: 0.0, 4.2%, 4.2%. The Y, U, V BD-Rate savings compared to HM-4.0 with alpha bug-fix are respectively: AI-HE: 0.0%, 3.5%, 3.8%, AI-LC: 0.0, 3.8%, 3.8%.

The setting of JCTVC-G244 tested included sub-sampling of reference samples in calculation of alpha (JCTVC-F233 and JCTVC-F241) so that only 8 neighboring samples are used independent of block size compared to HM-4.0 which uses 8 for 4x4, 16 for 8x8 and 32 for 16x16.

# Simulation results

Encoder/decoder simulations were run on Linux cluster. Table below presents a summary of BD-Rate and encoder/decoder run-times in comparison with HM 4.0 anchors. The differences in encoder and decoder times could be attributed to the heterogeneous nature of compute cluster used for simulations.

|  | TI cross-check |  |  | Proponent |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rate |  |  | BD-Rate |  |  |
|  | Y | U | V | Y | U | V |
| AI HE | 0.0% | -3.9% | -4.3% | 0.0% | -3.9% | -4.3% |
| AI LC | 0.0% | -4.2% | -4.2% | 0.0% | -4.2% | -4.2% |

| TI cross-check | Proponent | TI cross-check | Proponent |
| --- | --- | --- | --- |
| Enc Time | Enc Time | Dec Time | Dec Time |
| 99% | 100.2% | 99% | 99.0% |
| 99% | 100.6% | 99% | 101.6% |

# Discussions

- The complexity increase in decoder is comparison with thresholds during calculation of alpha. This happens only once per block and could be viewed as not being significant.

- In terms of complexity decrease, this tool enables a higher degree of sub-sampling of reference samples compared to JCTVC-G129 so that only 8 reference samples are used for any block size. This is very useful since alpha calculation consumes significant area in LM mode.

- WD text has been provided by proponents.