| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, Switzerland, 21-30 November, 2011 | Document: JCTVC-G650 WG11 Number: m22217 |
| --- | --- |

| Title: | CE8 Subtest a: Cross-check of Tool 2 (JCTVC-G609) Directional feature calculation on subset of pixels |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Email: | wlai@sta.samsung.com ffernandes@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This report presents Samsung’s cross-check results for CE8 Subtest a Tool 2 (JCTVC-G609), on directional feature calculation on subset of pixels. Four cases were tested in this report: (1) Use center 4 pixels, (2) Use 8 pixels on the center two rows, (3) Use 8 pixels on the center two rows + shape-dependent BA mode, and a supplementary set “Use center 4 pixels + shape-dependent BA mode.

Rate-PSNR numbers match exactly to the proponent’s results. As for software study, due to the coupling of BA modes with filter shapes, there is no additional syntax, and there are the same numbers of “classification + shape combinations” to be evaluated at encoder side, as compared to HM4.0.

# Cross-check results

Summary of the results are shown in the tables below and more detailed results can be found in the accompanying Excel sheet. The simulations were run in the 64-bit Linux cluster environment.

(1)	NEW_BA_SUB 4 (use center 4 pixels)

|  | CE8a Tool 2 results by the proponent ( JCTVC-G609 [1] ) |  |  |  | Cross-check results by Samsung |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LDB | LDP | IO | RA | LDB | LDP |
| BD-rate | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc. Time | 99% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Dec. time | 99% | 99% | 100% | 99% | 100% | 102% | 99% | 101% |

(2)	NEW_BA_SUB 8 (use pixels on the center two rows)

|  | CE8a Tool 2 results by the proponent ( JCTVC-G609 [1] ) |  |  |  | Cross-check results by Samsung |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LDB | LDP | IO | RA | LDB | LDP |
| BD-rate | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 |
| Enc. Time | 100% | 101% | 101% | 99% | 100% | 100% | 100% | 100% |
| Dec. time | 102% | 102% | 102% | 102% | 101% | 101% | 101% | 102% |

(3)	NEW_BA_SUB 8 (use pixels on the center two rows)

NEW_BA_MOD 3 (shape-dependent BA)

|  | CE8a Tool 2 results by the proponent ( JCTVC-G609 [1] ) |  |  |  | Cross-check results by Samsung |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LDB | LDP | IO | RA | LDB | LDP |
| BD-rate | -0.1 | -0.2 | -0.2 | -0.4 | -0.1 | -0.2 | -0.2 | -0.4 |
| Enc. Time | 101% | 101% | 102% | 100% | 101% | 100% | 100% | 100% |
| Dec. time | 100% | 100% | 99% | 99% | 101% | 101% | 101% | 103% |

Supplementary data:	NEW_BA_SUB 4 (use center 4 pixels)

NEW_BA_MOD 3 (shape-dependent BA)

|  | CE8a Tool 2 results by the proponent ( JCTVC-G609 [1] ) |  |  |  | Cross-check results by Samsung |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LDB | LDP | IO | RA | LDB | LDP |
| BD-rate |  |  |  |  | 0.0 | -0.1 | -0.1 | -0.1 |
| Enc. Time |  |  |  |  | 101% | 100% | 100% | 100% |
| Dec. time |  |  |  |  | 101% | 102% | 101% | 101% |

# Software study

- There is no additional syntax, since the BA computation is directly coupled with the filter shape: Decoder can perform the corresponding BA computation based on the decoded filter shape.

- The shape-dependent BA process at the encoder is as the following:

- BA classification using diagonal features, with snowflake-5x5 filter design (Evaluation 1).

- BA classification using horizontal/vertical features, with cross-11x5 filter design (Evaluation 2).

- Select from 1 and 2 which provides better rate-distortion efficiency, and use the selected shape together with RA to design filters (Evaluation 3).

(As a comparison, HM4.0 also requires 3 filter design evaluations as below [2].)

- BA using horizontal/vertical features, with snowflake-5x5 filter design (Evaluation 1).

- The same BA classification as in 1, now with cross-11x5 filter design (Evaluation 2).

- Select from 1 and 2 which provides better rate-distortion efficiency, and use the selected shape together with RA to design filters (Evaluation 3).

# References

- “CE8 Subtest a, Tool 2: Directional feature calculation on subset of pixels”, JCTVC-G609, Geneva, Switzerland, Nov. 2011

- “CE8 Subtest 4: ALF using vertical-size 5 filters with up to 9 coefficients”, JCTVC-F303, Torino, Italy, Jul. 2011

# Patent rights declaration(s)

Samsung may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).