| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, Switzerland, 21-30 November, 2011 | Document: JCTVC-G652 WG11 Number: m22219 |
| --- | --- |

| Title: | CE8 Subtest d: Cross-check of Tool 3 (JCTVC-G130): Chroma ALF with reduced vertical filter size, proposed by TI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Email: | wlai@sta.samsung.com ffernandes@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This report presents Samsung’s cross-check results for CE8 Sebtest d Tool 3 (JCTVC-G130), chroma ALF with reduced vertical filter size, proposed by TI. Rate-PSNR numbers match exactly to the proponent’s results. The 1% or 2% enc / dec time increases (compared to HM40 anchor) in the cross-checker’s measurements are within the normal variations of Linux cluster based simulations.

# Cross-check results

Summary of the results are shown in the table below and more detailed results can be found in the accompanying Excel sheet. The simulations were run in the 64-bit Linux cluster environment.

|  | CE8d Tool 3 results by the proponent TI ( JCTVC-G130 [1] ) |  |  |  | Cross-check results by Samsung |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LD | LDP | IO | RA | LD | LDP |
| BD-rate U | 0.4 | 0.8 | 0.7 | 0.7 | 0.4 | 0.8 | 0.7 | 0.7 |
| BD-rate V | 0.5 | 0.5 | 0.5 | 0.4 | 0.5 | 0.5 | 0.5 | 0.4 |
| Enc. Time | 104% | 100% | 101% | 100% | 101% | 100% | 100% | 100% |
| Dec. time | 105% | 101% | 102% | 100% | 100% | 101% | 100% | 102% |
| Vert. taps (chroma) | 3 |  |  |  | 3 |  |  |  |
| Max# of coeff.(chroma) | 9 |  |  |  | 9 |  |  |  |

# Findings in software study

- The code changes are consistent with the two vertical-size-3 filters as proposed, with 9 coefficients.

# References

- “CE8 Subtest d Tool 3: chroma ALF with reduced vertical filter size”, JCTVC-G130, Geneva, Switzerland, Nov. 2011

# Patent rights declaration(s)

Samsung may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).