| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G662 |
| --- | --- |

| Title: | Cross-check CE4-subtest2.3: Fine Granularity QP |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Cross-check] |  |  |
| Author(s) or Contact(s): | Xiang Yu Jing Wang Da-ke He | Email: | xiayu@rim.com wangj@rim.com dhe@rim.com |
| Source: | [Research In Motion Ltd., Canada] |  |  |

_____________________________

# Abstract

This document reports xcheck results of CE4 subtest 2.3 Test-C. Our BD-rate numbers match with the proponent reported. Overall, the FineQP scheme provides 0.5%, 0.5%, 0.4%, and 0.5% rate saving over the encoder-only-scheme QP-rounding in term of average BD-rate calculated based on (6*Ypsnr+Upsnr+Vpsnr)/8 for RAhe, RAlc, LDhe, and LDloc respectively.

# Implementation Analysis

To be included.

# Simulation results

The average BD-rate is calculated based on (6*Ypsnr+Upsnr+Vpsnr)/8, as suggested on the HEVC reflector.

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A | 0.0% | -3.6% | -3.7% | -0.4% | 0.2% | -4.7% | -5.2% | -0.4% |
| Class B | -0.2% | -2.5% | -1.8% | -0.5% | 0.1% | -4.0% | -3.6% | -0.5% |
| Class C | -0.4% | -2.8% | -2.5% | -0.8% | -0.1% | -4.1% | -3.7% | -0.8% |
| Class D | -0.2% | -1.2% | -1.4% | -0.4% | 0.0% | -2.2% | -1.9% | -0.3% |
| Class E |  |  |  |  |  |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.2% | -2.5% | -2.3% | -0.5% | 0.0% | -3.8% | -3.6% | -0.5% |
|  | -0.1% | -2.7% | -2.5% | -0.4% | 0.2% | -3.9% | -3.7% | -0.4% |
| Enc Time[%] | 102% |  |  |  | 105% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 100% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y | U | V | Average | Y | U | V | Average |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.0% | -2.6% | -2.3% | -0.4% | 0.4% | -5.8% | -6.8% | -0.7% |
| Class C | -0.5% | -2.9% | -2.6% | -0.9% | 0.1% | -5.0% | -4.8% | -0.8% |
| Class D | 0.0% | -0.6% | -1.1% | -0.2% | 0.3% | -3.9% | -3.5% | -0.4% |
| Class E | 0.0% | 0.4% | 0.5% | 0.1% | 0.4% | -1.2% | -1.3% | 0.1% |
| Class F | #NAME? | #NAME? | #NAME? | #NAME? | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.1% | -1.6% | -1.6% | -0.4% | 0.3% | -4.3% | -4.5% | -0.5% |
|  | -0.1% | -1.8% | -1.8% | -0.4% | 0.3% | -4.5% | -4.7% | -0.5% |
| Enc Time[%] | 101% |  |  |  | 104% |  |  |  |
| Dec Time[%] | 104% |  |  |  | 95% |  |  |  |

Please note the enc/dec time is not accurate as they were generated on different platforms.

# Patent rights declaration(s)

Research In Motion Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).