| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G664r1 |
| --- | --- |

| Title: | A Lossless Coding Solution for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wen Gao (1) Minqiang Jiang (1) Ye He (1) Jin Song (2) Haoping Yu (1) (1). Huawei Technologies (USA) 2330 Central Expressway Santa Clara, CA 95050 (2). Huawei Technologies Co., LTD Section B, Bantian, Longgang District Huawei Base, Shenzhen, China | Tel: Email: | +1 408 330 4816 wen.gao@huawei.com Minqiang.Jiang@huawei.com Ye.sc.He@huawei.com songjin@huawei.com haoping.yu@huawei.com |
| Source: | Huawei Technologies (USA) |  |  |

_____________________________

# Abstract

This contribution proposes a lossless coding solution that only involves a few modifications to the current HEVC WD. To achieve lossless coding in both intra and inter coding operations, the transform and quantization modules are bypassed. Due to the nature of lossless coding, the existing intra predictions are extended to pixel based prediction (DPCM), taking into account that there is no transform applied to prediction residuals. Furthermore, since the statistical properties of prediction residuals are quite different from those of transform coefficients, the CABAC coding of intra prediction residuals are also modified accordingly.

No additional flag is introduced in this proposal, the lossless coding mode is signaled by QPY=0. As a result, no change to the HEVC syntax specification is needed, and the lossless mode can be applied to the entire picture or to individual CUs conveniently.

# Introduction

From the latest version of "Vision, Applications and Requirements for High Efficiency Video Coding (HEVC)", MPEG-N11872 [1], possible applications for HEVC standards will include “remote video presentation and remote computer graphics display”. The same document also states that “Subjective visually lossless compression shall be supported. Lossless compression should be supported”.  To address these requirements, we propose true lossless coding tools in this contribution.

First to achieve lossless coding, the transform was bypassed together with the quantization process in the encoder and decoder, which is mainly due to the fact that the existing HEVC transforms are not reversible in general. Without transform operation, pixel based intra predictions become possible. In this contribution, we propose to extend the existing unified directional intra (UDI) prediction to DPCM intra prediction. In this approach, the internal pixels of a PU are predicted from its neighboring pixels, not from the pixels in the neighboring PUs. Second, without transform, prediction residuals become the direct input to the CABAC modules. Since statistical characteristics of prediction residuals and transform coefficients are quite different, the binarization method and the coding of the signs of prediction residuals in the CABAC module are modified accordingly.

To avoid the potential negative impact to the general compression efficiency and to minimize the necessary changes to the HEVC specification, we propose not to add any additional flag and instead to use QPY=0 to indicate the lossless coding mode. Using this method, the lossless coding mode can be conveniently applied to the entire picture or at CU level. Our test results showed that the proposed lossless coding solution performs better, with less number of total compressed bits and a perfect fidelity identical to the source video, than the current HEVC existing solution at QPy=0.

In the following sections, the above modifications to the existing HEVC WD will be described in details.

# Transform and quantization bypass

To achieve lossless coding, transform and quantization modules are bypassed in HM4.0, as shown in Figure 1.

Figure 1 – Lossless Coding Mode

Since the de-block filter and SAO and ALF will change pixels values, they are all disabled in the proposed lossless mode. Furthermore, since internal bit depth increase (IBDI) doesn’t provide any gain in coding efficiency for the lossless case, it is set always to be equal to the input bit depth.

# Intra prediction

## DPCM – Extension of unified directional intra prediction

The proposed DPCM intra prediction is an extension of the existing unified directional intra (UDI) prediction. The set of intra prediction angles for a PU with different sizes stays the same. The method to interpolate reference pixels to obtain predicted pixels stays the same except the set of reference pixels are changed depending on the intra prediction angle as well as the position of the pixel to be predicted. Thus there is no syntax modification to HEVC WD. However, the decoding process for intra prediction needs to be modified.

We will use an example to illustrate the DPCM intra prediction method. In Figure 3, the 4x4 PU to be predicted is represented by Z(i, j), for i=0,…,3 and j=0,…,3; P(I, J) denotes the prediction version of Z(I, J) for i=0,…,3 and j=0,…,3. The pixels from neighboring top and left PU are X(i) and Y(i) for i=-1,…7, respectively, where X(-1)=Y(-1).

Figure 3 - An example of 4x4 prediction unit (PU)

Figure 4 - Intra prediction angle definition

First define the prediction angle as the angle between the line segment BA and CA in Figure 3, and the set of intra prediction angles defined in the HEVC WD is shown in Figure 4.  For illustration purpose, we describe the DPCM intra prediction when the angle is in [-45, 45]. When the prediction angle is in the range of [-135, -45), we can transpose the PU and all the neighboring pixels and use the same prediction method described as follows.

- If prediction angle is 0, i.e., vertical intra prediction is used, the 0-th row of the PU is predicted from its top neighboring PU as P(0, J) = X(J) for J=0..,3 while others rows in PU are predicted from its row above, i.e., P(I, J)=Z(I-1, J) for I=1,2,3 and J =0,..3.

- If prediction angle is in (0, 45], the 0-th row and 3-th column of the PU are predicted using the method of the current HEVC WD. To predict other pixels in k-th rows for k=1,2,3, i.e., Z(k, j) for j=0,1,2, the detailed computation of the prediction samples is the same as the one used to predict the samples in the 0-th row, except that the reference pixels are changed to the original pixels in the (k-1)-th row, i.e., X(i) is re-defined as X(i)=Z(k-1, i) for i=0,…3.

- If prediction angle is in [-45, 0), the 0-th row and 0-th column of the PU are predicted using the method of the current HEVC WD. To predict other pixels in k-th rows for k=1,2,3, i.e., Z(k, j) for j=1,2,3, the detailed computation of the prediction samples is the same as the one used to predict the samples in the 0-th row, except that the reference pixels are changed to the original pixels in the (k-1)-th row, i.e., X(i) is re-defined as X(i)=Z(k-1, i) for i=0,…3.

## Modified DC mode

For lossless coding, DC mode is replaced with the following local adaptive predictor

P(X)=min(A,B)if C≥max(A,B)max(A,B)if C≤min(A,B)

Where X represented the pixel to be predicted, A, B and C represents its neighboring pixels, as shown in Figure 5.

| C | B | D |
| --- | --- | --- |
| A | X |  |

Figure 5 - Pixel X and its neighbors

# CABAC coding of prediction residuals

Without transform, prediction residuals become the direct input to the CABAC modules. Since the statistical characteristics of prediction residuals are quite different from that of transform coefficients, the binarization method and context modeling of sign of prediction residuals are modified accordingly.

## Binarization of luma prediction residuals

In HEVC WD, a combination of truncated Golomb-Rice (TR) code and zero-th order Exponential Golomb code (EG0) is used to binarize the transform coefficient absolute levels. For illustration purpose, we first briefly describe the existing binarization scheme in HEVC WD, which consists of 3 parts as shown in Figure 5.

Figure 5 - Binarization of the absolute transform coefficient levels in HM4.0

- The absolute transform coefficient levels, 1 and 2, are binarized using a unary code as bin string “0”, “10” respectively. If the absolute transform coefficient level is greater than or equal to 3, the value 3 is first binarized as the bin string “11” indicating the maximum value for the first part of the binarization,

- If absolute transform coefficients greater than or equal to 3 are coded (i.e., if a bin string equal to '11' is coded), the value of the absolute transform coefficient level minus 3, up to a specific limit, denoted as cTRMax, is coded using TR codes, where all bins of TR code-words are coded in bypass mode. The limit, i.e., cTRMax, is a variable and depends on the Golomb-Rice parameter denoted as cRiceParam. In HEVC WD, the limit is specified in Table 1.

Table 1 – Specification of TRMax depending on cRiceParam

| cRiceParam | cTRMax |
| --- | --- |
| 0 | 7 |
| 1 | 20 |
| 2 | 42 |
| 3 | 70 |

The parameter cRiceParam is adapted based on the following formula:

kt+1=0valuet∈[0,1]∧kt<11valuet∈[2,3]∧kt<22valuet∈[4,nkt]∧kt33valuetnktkt4

where kt+1 denotes the cRiceParam for the next coding stage, kt the current cRiceParam, and valuet denotes the current value of absolute transform coefficient level minus 3.

- If the absolute transform coefficient level minus 3 is larger than the maximum value that can be coded with the TR code, i.e., cTRMax, the value of 3 and (cTRMax+1) are subtracted from the absolute transform coefficient level. The resulting value is then coded with the EG0 code in bypass mode.

Note that the above binarization method was originally designed for coding quantized transform coefficients, which are normally small values. However, in lossless coding, large prediction residuals may appear with relatively higher frequency, compared to transform coefficients. Taking this characteristic into consideration, a different binarization method is proposed as below:

- First the cRiceParam is expanded from [0,3] to [0,5], with the following parameters:

Table 2– Specification of cTRMax depending on cRiceParam

| cRiceParam | cTRMax |
| --- | --- |
| 0 | 7 |
| 1 | 20 |
| 2 | 42 |
| 3 | 70 |
| 4 | 142 |
| 5 | 252 |

- The method to adapt cRiceParam is also modified. The following lookup table is utilized. Based on the table, the next cRiceParam kt+1 is determined based on the current cRiceParam kt and the current value of the absolute prediction residual level minus 3, i.e.,valuet. To limit the size of the lookup table, valuetis limited to maximum value of 15.

Table 3 – Adaptation of cRiceParam

|  |  | min(valuet , 15) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| kt | 0 | 0 | 0 | 1 | 1 | 2 | 2 | 2 | 3 | 3 | 3 | 4 | 4 | 4 | 5 | 5 | 5 |
|  | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 3 | 3 | 3 | 4 | 4 | 4 | 5 | 5 | 5 | 5 |
|  | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 |
|  | 3 | 3 | 3 | 3 | 3 | 3 | 4 | 4 | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 |
|  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 5 | 5 | 5 | 5 | 5 | 5 |
|  | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 |

- If the absolute prediction residual level minus 3 is larger than cTRMax, instead of EG0 code, a 5-th order Exponential-Golomb code, i.e., EG5 code, is used to binarize the value of (absolute prediction residual level – 3 – cTRMax – 1)

## Context model of signs of luma and chroma prediction residuals

In HM4.0, signs of Luma and Chroma prediction residuals are coded using bypass mode with the assumption that these signs are I.I.D and equal probable. We observe there is a significant correlation between the signs in a coding block in the lossless mode.

A context model is created for sign of Luma, Chroma-U and Chroma-V prediction residuals, respectively. There is only one single context for each context model. Thus the conditional probability is only changed adaptively based on the encoded signs.

# Prediction Residual Scanning

## Mode dependent residual scanning (MDRS) for intra coding

In HM4.0, mode dependent coefficient scanning (MDCS) is used in intra coding. There are three scanning patterns, i.e., ZIG-ZAG Scan with index 0, Horizontal Scan with index 1 and Vertical Scan with index 2. The main idea of MDCS is that for relatively small transform unit (TU), e.g. an 8x8 or 4x4 TU, when the intra prediction is in vertical direction, the transformed coefficients will often appear in top horizontal region. Thus a horizontal scan will help reduce the total number of bits to encode a significant map and a last position of a transform block. Based on this idea, a lookup table, shown in Table 4, is adopted in current HEVC WD and HM4.0 software in which the scanning direction is chosen based on intra prediction mode and the size of TU.

Table 4 – MDCS Mapping Table in HM4.0

| Mode TU | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32x32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 16x16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8x8 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |
| 4x4 | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 |

The intra prediction mode is defined in Figure 6.

Figure 6 - Intra prediction mode directions

In the proposed lossless coding, intra predicted residuals do not show the same behavior as transformed coefficients. Instead, it is observed that for relatively small transform unit (TU), e.g. an 8x8 or 4x4 TU, when the intra prediction is in vertical direction, the residual will often appear in vertical direction. Thus a vertical scan will often result in better performance. Similarly when the intra prediction is in horizontal direction, a horizontal scan will often be better. Based on this observation, we propose the following mode dependent residual scan (MDRS), shown in the lookup Table 5.

Table 5 – MDRS Mapping Table for Intra Prediction Residuals

| Mode TU | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32x32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 16x16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8x8 | 2 | 1 | 0 | 0 | 2 | 2 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 |
| 4x4 | 2 | 1 | 0 | 0 | 2 | 2 | 0 | 1 | 1 | 0 | 0 | 2 | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 |

## Multiple start scanning positions for inter coding

Unlike the transform coefficients which usually concentrate at the upper left corner of the block, the inter prediction residuals may appear in random positions. This MSCAN tool chooses different starting scanning position based on rate distortion optimization (RDO) process and rescans the prediction residuals before the regular Zigzag scan for coding of significant map for only inter-coded blocks in current HEVC WD. The proposed MSCAN tool will improve the coding efficiency for inter coded blocks.

In this contribution, the following three starting position indexes are proposed for three color components, i.e., Luma, Chorma U and Chroma V:

- ScanIdxY: Starting position index for Luma component scanning

- ScanIdxU: Starting position index for Chroma U component scanning

- ScanIdxV: Starting position index for Chroma V component scanning

The definition of the scanning starting position index is as following:

- Index = 0: scan starting from the top left corner (as in current HEVC WD), i.e., scan the first line from left to right, then scan the second line from left to right, …, scan the last line from left to right.

- Index =1: scan starting from the top right corner, i.e., scan the first line from right to left, then scan the second line from right to left, …, scan the last line from right to left.

- Index =2: scan starting from the lower left corner, i.e., scan the last line from left to right, then scan the second last line from left to right, …, scan the first line from left to right.

- Index =3: scan starting from the lower right corner, i.e., scan the last line from right to left, then scan the second last line from right to left, …, scan the first line from right to left.

Note that this scanning operation is operated before the regular Zigzag coefficient scan for inter coded prediction residuals. Alternatively, we can combine MSCAN and regular Zigzag scan operations in our description of MSCAN tool. From this point of view, we have proposed four different Zigzag scanning patterns for coding of significant map of inter-coded residuals. The four different Zigzag patterns are described in Figure 7.

# Index = 0               Index = 1

# Index =2                          Index=3

Figure 7 – Illustration of the four different Zigzag Scan pattern

Due to the introduction of the multiple starting positions for scanning of inter prediction residuals, residual coding CABAC syntax is modified as Table 6. In this table, the variable, LosslessCodingFlag, indicates whether the current coding block is coded in lossless mode. This variable is determined based on the QPY value of the current coding unit (CU).  If QPY = 0, LosslessCodingFlag is set to 1 for current CU. Otherwise, it is set to 0.

Table 6 – Modified Syntax Table for Residual Coding

| residual_coding_cabac( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| If (PreMode != MODE_INTRA) { |  |
| If (cIdx ==0) && (LosslessCodingFlag==1) |  |
| ScanIdxY | ae(v) |
| If (cIdx ==1) && (LosslessCodingFlag==1) |  |
| ScanIdxU | ae(v) |
| If (cIdx ==2) && (LosslessCodingFlag==1) |  |
| ScanIdxV | ae(v) |
| } |  |
| last_significant_coeff_x | ae(v) |
| last_significant_coeff_y | ae(v) |
| n = 0 |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n ][ 1 ] |  |
| while( ( xC != last_significant_coeff_x ) ¦¦ ( yC != last_significant_coeff_y ) ) { |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| n++ |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n ][ 1 ] |  |
| } |  |
| numSubsets = max( 1, ( 1 << ( log2TrafoSize << 1 ) ) >> ( cIdx > 0 ? 6 : 4 )) |  |
| for( i = 0; i < numSubsets; i++ ) { |  |
| offset = i << 4 |  |
| xS = ScanOrder[ log2TrafoSize − 2 ][ 0 ][ i ][ 0 ] << 2 |  |
| yS = ScanOrder[ log2TrafoSize − 2 ][ 0 ][ i ][ 1 ] << 2 |  |
| for( n = 0; n < 16; n++ ) { |  |
| xOffset = n – (n >> 2 ) << 2 |  |
| yOffset = n >> 2 |  |
| if( significant_coeff_flag[ xS + xOffset ][ yS + yOffset ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| } |  |
| for( n = 0; n < 16; n++ ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) { |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| } |  |
| } |  |
| for( n = 0; n < 16; n++ ) { |  |
| xOffset = n – (n >> 2 ) << 2 |  |
| yOffset = n >> 2 |  |
| if( significant_coeff_flag[ xS + xOffset ][ yS + yOffset ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = 0 |  |
| } |  |
| } |  |
| } |  |

# Mechanism to indicate the lossless coding mode

To allow decoder to decode HEVC bit stream correctly, a mechanism to indicate lossless coding is required. In this contribution, we propose to indicate lossless coding of a coding unit (CU) by setting its quantization parameter (QPY) equal 0. For those CUs using lossless coding mode, the internal bit depth is set to the input bit depth and the de-blocking filter, SAO and ALF are all disabled.

We use two examples to illustrate the usage of this indication mechanism. The main motivation of using the existing quantization parameter (QPY = 0) to indicate the lossless mode is that no new syntax element is introduced to the bit stream, and as a result, the coding performance of HEVC for the lossy mode would not be impacted or compromised by the addition of this lossless coding mode. In addition, using QPY=0 also brings us the flexibility in the sense that this lossless mode can be used not only for the entire picture or a slice but also for the individual CUs as well.

In the first example, to indicate the lossless coding for a video picture, we can set the following syntax elements in the bit stream

- Set cu_qp_delta_enabled_flag=0 in sequence parameter set (SPS)

- Set pic_init_qp_minus26=-26 in picture parameter set (PPS)

- Set slice_qp_delta=0 in all non-lightweight slice headers associated with the video picture

Based on this setting, QPY=0 will be used for all CUs in the video picture thus indicating lossless coding for the whole picture.

In another example, to lossless compress any individual CUs in a video picture whenever necessary, we can set following syntax elements in the bit stream.

- Set cu_qp_delta_enabled_flag=1

- Set pic_init_qp_minus26 as usual for lossy coding

- Set slice_qp_delta as usual for losing coding

- Set cu_qp_delta in a CU to a value such that the derived QPY equal to 0 thus indicating lossless coding for the CU.

Based on this setting, QPY is set to zero for those CUs using lossless coding mode.

# Test results

To test the performance of the proposed lossless coding tools, we integrate all the related changes mentioned above in the HM4.0 SDIP software used in CE6. All the sequences in Class A to F are tested for both AI-HE and LB-HE setting. The common test conditions are modified to take into account the characteristics of lossless coding. The modifications are as following:

- Set internal bit depth as input bit depth

- Disable De-block filter, SAO and ALF

- Set RDOQ=0

Besides the lossless coding results, test results at QP=0, RDOQ=0, and the rest of common test conditions unchanged are also provided as anchors. An early skip method is used to speed up the simulation in low delay case. The summary of the test results is included in Table 7.

Table 7 – Summary of test results for lossless coding

|  | AI_HE |  |  | AI_HE_SDIP |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | bit-rate saving | compression ratio |  | bit-rate saving |  |
|  | anchor | proposed |  | anchor | proposed |  |  |
| Class A | 2.14 | 2.43 | -12.02% | 2.15 | 2.57 | -11.89% |  |
| Class B | 1.93 | 2.17 | -11.06% | 1.95 | 2.20 | -11.07% |  |
| Class C | 1.79 | 2.07 | -13.63% | 1.80 | 2.08 | -13.44% |  |
| Class D | 1.71 | 1.97 | -13.65% | 1.72 | 1.98 | -13.35% |  |
| Class E | 2.71 | 3.21 | -15.41% | 2.73 | 3.22 | -15.29% |  |
| Class F | 3.88 | 5.42 | -28.43% | 4.02 | 5.49 | -26.90% |  |
| Overall | 2.51 | 2.86 | -9.71% | 2.54 | 2.88 | -9.35% |  |
| Enc Time[%] | 84% |  |  | 84% |  |  |  |
| Dec Time[%] | 95% |  |  | 81% |  |  |  |
|  | LB_HE |  |  | LB_HE_SDIP |  |  | 7 Zip |
|  | compression ratio |  | bit-rate saving | compression ratio |  | bit-rate saving | Compression Ratio |
|  | anchor | proposed |  | anchor | proposed |  |  |
| Class A |  |  |  |  |  |  |  |
| Class B | 2.18 | 2.39 | -8.68% | 2.21 | 2.41 | -8.73% | 1.81 |
| Class C | 2.32 | 2.58 | -10.19% | 2.32 | 2.58 | -10.19% | 1.69 |
| Class D | 2.48 | 2.70 | -8.75% | 2.48 | 2.70 | -8.76% | 1.70 |
| Class E | 3.78 | 4.17 | -9.40% | 3.78 | 4.17 | -9.40% | 2.49 |
| Class F | 51.11 | 55.21 | -6.58% | 52.20 | 55.58 | -5.99% | 42.06 |
| Overall | 12.29 | 13.32 | -7.24% | 12.52 | 13.40 | -7.15% | 8.62 |
| Enc Time[%] | 126% |  |  | 123% |  |  |  |
| Dec Time[%] | 77% |  |  | 70% |  |  |  |

Note that the compression ratio in the above table is defined as the ratio between the data size of raw video sequence and the total bit count of the compressed bit stream.7-Zip is a lossless compression tool used for file compression. The anchor encoder in AI_HE and LB_HE is HM4.0. The anchor encoder in AI_HE_SDIP and LB_HE_SDIP is HM4.0 SDIP software. The two 10-bit sequences are not included in the computation. The simulation time is not reliable since they are running on different machines.

From the above results, we can make the following observation: (1) In AI_HE case, the average bit saving is 9.71%. The average bit saving for class F sequences is much higher, i.e., 28.43%. The performance results of AI_HE_SDIP case are very similar to that of HE_AI case; (2) In LB_HE case, the average bit saving is smaller, i.e., 7.24%. The average bit saving for class F sequences is 6.58%, much smaller than that in AI_HE case. It’s reasonable that all of the proposed tools except MSCAN only address the intra coding, not inter coding. Again, the performance results of LB_HE_SDIP case is similar to that of LB_HE case; (3) In term of compression ratio, the performance of LB_HE coding outperforms that of 7-Zip significantly; (4) In LB_HE and LB_HE_SDIP cases, the proposed Lossless encoder runs slower than the respective anchor encoder. The main reason is due to the extra RDO process in MSCAN to determine the best start scanning position in inter coding.

As shown in the test results, the proposed lossless coding outperforms the HM with QP=0, which provides justification to the proposed signaling approach of using QPY=0 in a CU to indicate lossless coding.

# Conclusion

The proposed lossless coding solution provides satisfactory compression performance with minimum changes to the HEVC WD. We suggest conducting further tests in CE.

# Patent rights declaration(s)

Huawei Technologies (USA) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Vision, Applications and Requirements for High Efficiency Video Coding (HEVC), ISO/IEC JTC1/SC29/WG11/N11872, January 2011, Daegu, KR.

- Frank Bossen, Common test conditions and software reference configurations, JCT-VC Document, JCTVC-F900, Torino, July 2011.