| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G666 |
| --- | --- |

| Title: | 1D- DCT based frequency domain adaptive loop filter (FD-ALF) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | *Jeongyeon Lim (SK telecom) #Ju Ock Lee (Sejong Univ.) #Hae-Kwang Kim (Sejong Univ.) #Joo-Hee Moon (Sejong Univ.) #Sungwon Lim (Sejong Univ.) *9-1 Sunae-Dong, Bungdang-gu, Seongnam City, Kyeonggi-Do, 463-784, Korea #914, Choongmu-Gwan, Sejong University, 98 Gunja-Dong, Gwangjin-Gu, Seoul, 143-747, Korea | Email: | jeongyeon@sk.com jolee@sju.ac.kr hkkim@sejong.ac.kr jhmoon@sejong.ac.kr sw823@sju.ac.kr |
| Source: | SK telecom and Sejong Univ. |  |  |

_____________________________

# Abstract

A frequency domain adaptive loop filtering (FD- ALF) method on the basis of 1D DCT is proposed in this document. The purpose of this contribution is reducing computational complexity of ALF while minimizing coding efficiency loss. The basic scheme of FD-ALF follows the current ALF in HM4.0 except that the filtering is applied in 1D DCT frequency domain. FD-ALF is adaptively applied on RA or BA classification mode and can be controlled on CU unit basis just as same way as the existing ALF. DCT domain filtering is processed by multiplication operation of DCT domain filter to the DCT transformed reconstructed picture after SAO rather than the convolution operation of current pixel based ALF filter. Different 1D DCT filters are characterized by its tap size (8 tap, 16 tap), its direction (horizontal or vertical) and the coefficients of each of 1D DCT filters. The coefficients of 1D DCT filters are obtained by MSE (Mean Square Error) optimization method between the original picture and the reconstructed picture.

Using common conditions, the average bit rate reduction is +1.2% for Y components for high efficiency AI, +2.2% for Y components for high efficiency RA, +1.8% for Y components for high efficiency LDB, and +2.8% for Y components for high efficiency LDP. Encoder Time is 83% for high efficiency AI, 97% for high efficiency RA, 98% for high efficiency LDB, and 96% for high efficiency LDP. Decoder Time is 94% for high efficiency AI, 91% for high efficiency RA, 94% for high efficiency LDB, and 92% for high efficiency LDP.

# Introduction

In HM 4.0, ALF using two shapes, star 5x5 shapes and cross 11x5, was adopted [1]. The main idea of ALF is that optimized wiener filter are calculated to reduce the mean square error between the original picture and the reconstructed picture. Therefore significant coding efficiency is obtained by high quality reference picture resulting from ALF operation. However, the demerit of ALF is that the complexities of encoder and decoder are greatly increased due to using convolution calculation of the adaptive loop filter.

In this contribution, a 1D DCT based frequency domain ALF is presented where the filtering is applied using some multiplication operation and addition operation rather than convolution operation. The goal of this method is reducing complexity of ALF while minimizing the coding performance.

The basic scheme of FD-ALF follows the current ALF in HM4.0 except that the filtering is applied in 1D DCT frequency domain. FD-ALF is adaptively applied on RA or BA classification mode and can be controlled on CU unit basis just as same way as the existing ALF. DCT domain filtering is processed by multiplication operation of DCT domain filter to the DCT transformed reconstructed picture after SAO rather than the convolution operation of current pixel based ALF filter. Different 1D DCT filters are characterized by its tap size (8 tap, 16 tap), its direction (horizontal or vertical) and the coefficients of each of 1D DCT filters. The coefficients of 1D DCT filters are obtained by MSE (Mean Square Error) optimization method between the original picture and the reconstructed picture.

Section 2 explains the detailed process of the proposed method with its complexity analysis. Section 3 shows experimental results. Section 4 concludes this paper.

# FD-ALF (Frequency Domain based ALF)

While the existing ALF applied to pixel unit, the proposed FD-ALF is applied to 1D DCT block unit. FD-ALF filter is adaptively applied by RA or BA classification mode and can be controlled in CU unit as exactly same way as the existing ALF. Different 1D DCT filters are characterized by its tap size (8 tap, 16 tap), its direction (horizontal or vertical) and the coefficients of each of 1D DCT filters. So the types of filters in the proposed method are four types of 1x8, 1x16, 8x1 and 16x1 DCT filters. Apply FD-ALF with BA mode and RA mode and select best one in terms of RD cost as the mode for slice.

First, BA mode is calculated. The original picture and the reconstructed pictures are divided into blocks for BA mode. Block division of the proposed method for variance in BA mode is different from the existing one. For each type of DCT filters (1xM or Mx1), the block size of the variance calculation is decided as MXM rather than 4x4 in HM4.0. The ALF in HM 4.0 calculated one variance for each 4x4 block. The proposed method calculate one variance for each MxM block. M is same as the 1D DCT filter tap size. The MxM block’s variance is average of 4x4 blocks in MxM block. For example, if M is 8 (8x1 or 1x8 DCT cases), the variance is calculated as Figure 1. The variances v0, v1, v2 and v3 of each 4x4 block is calculated. The variance V of the MxM block is calculated by the average of v0, v1, v2 and v3.

Figure 1 Block based variance for 1x8 and 8x1 DCT case

The categorization of each MxM block on the basis of variance is same as HM4.0. For each MxM block, filter direction (1xM or Mx1) is obtained as the one that gives the smallest Sum of Squared Error between the transformed original block and the transformed reconstructed block. After that the direction is decided, filter coefficients (weighting factors) of the decided filter type that gives the most similar filtered reconstructed blocks to the original blocks of the BA category in terms of SSE in DCT domain are calculated. Two succeeding categories are tested for merging by selecting small value between the sum of SSE in DCT domain for each category and the SSE with one merged category with one filter. For merged filter, filter coefficients are recalculated by the method for calculating the filter coefficients for one category.

Secondly, RA mode is calculated. Region division for RA mode is same as the one in HM4.0 For the RA case, for each filter type, each region of the original and reconstructed frames is divided into filtering blocks of its 1D DCT filter shape size and each filtering blocks of the original and reconstructed frames are DCT transformed. The filtering type (its size and direction) of each region is decided as the filter type that gives the smallest Sum of Squared Error between the transformed original region and the transformed reconstructed region. After deciding filter type for a region, filter coefficients (weigting factors) of the decided filter type that gives the most similar filtered reconstructed picture to the original region in terms of SSE in DCT domain are calculated. Two succeeding regions are tested for merging by selecting small value between the sum of SSE in DCT domain for each region and the SSE with one merged region with one filter. For merged filter, filter coefficients are recalculated by the method for calculating the filter coefficients for one region.

## Principle of FD-ALF

The DCT coefficient for original block (p) and reconstructed block (q) can be represented as follow:

(1)

where  is the pixel value vector in the original block, k is the block belongs to category K in the picture ,and  means 1xM or Mx1 discrete cosine transform.

The DCT coefficient for filtered block can be represented as follow:

(2)

The equation (1) can be also described as in (3) by using DCT frequency domain filtering by multiplication form instead of convolution form in pixel domain.

(3)

where  is the weighting factors (filtering coefficient).

The optimal weighting factors (filter coefficients) for each category are calculated by the following formula (4). For category K, calculates the Sum of Squared Error (SSE) of all the blocks belong to the category is calculated. In order to find optimal weighting factor which gives minimum Sum of Squared Error (SSE) for all the blocks belong to category K in picture as in (4):

(4)

The partial derivative for each weighting factors will be set to zero as follow:

(5)

Weighting factor are decided as follow:

(6)

As shown in (3) and (6), in the transform domain, adaptive loop filtering is possible by using weighting factors which are simply calculated by coefficients of the reconstructed and original picture.

## Encoding algorithm

The encoding algorithm consists of the following steps.

- Transform original picture and reconstructed picture

- Decide luma filter types and filter coefficients in the BA classification mode

- Decide luma filter coefficients in the RA classification mode using decided luma filter type by BA classification mode

- Decide luma classification mode BA or RA mode based on RD cost

- Decide and ALF control map including the depth of CU splitting for the designed filter

For chroma filtering, the half size (M/2) filter is applied. Filter coefficients are recalculated for the chroma using the same method as used for the luma. For smoothing between TU boundaries, FD-ALF filter starts from (-V,-H) position in padded picture. It processed both of original picture and reconstructed picture. H and V set 2.

Figure 2 Represent filtered picture area

In Figure 2, LCU’ is LCU of filtered picture.

### Decision merging filter set for each classification category

The proposed method can be merged using the following equation (7).

(7)

If category 0 merges with category 1, the SSE for merging is calculated as follows:

(8).

The following equation (8) is used for the weighting factors in merged category:

### Derive filter coefficients and filtering reconstructed picture

The filtering reconstructed picture derives by the weighting factors on transform domain derived as follow:

(9)

where  is MxM 1D-DCT matrix in HM 4.0 that position is ,  is MxM inverse 1D-DCT matrix in HM 4.0. (9) modify for converting the weighting factors for decreasing complexity of filtering. That is in order to make filtered reconstruction picture, need just reconstructed picture and converting the weighting factors. Therefore decoder does not need transform. Decoder makes the filter converted weighting factors from transform domain to spatial domain. For example, if filter tap size (M) is 4, as follow:

can be represent 4x4 matrix  to modifying (9).

(10)

, , , ,  and  consists of  and , as follow:

(10) can be modify same as (11).

(11)

, , ,  and  is modified from , , , ,  and , as follow:

, ,  and  is consists of , as follow:

After generate , , ,  and , encoder and decoder can make to reconstruct 4x1 or 1x4 pixels by calculating , , , , , . The reconstructed 4x1 or 1x4 pixels need 6 multipliers. That is 1.5 multipliers for one pixel. 8x1, 1x8, 16x1 and 1x16 can be modification same way as 4x1 and 1x4.

### Predicted coding for weighting factors

Encoder codes weighting factors as filter coefficients in the bitstream. The weighting factors are coded by prediction. There are two prediction modes. One of the prediction methods is as follow:

(12)

where  is coded value for filter coefficients,  indicates the position of filter coefficient,  indicates the set of filters. For the case of  equals to 0, the coefficient predicted by . The  sets 8. The other prediction method is as follow:

(13)

For the first filter of the filter set for the current picture the first prediction method is used. For the following filters the first prediction method or second prediction method can be selected and its signaled in the bitstream.

### Derive chroma filter and chroma filtering reconstructed picture

Chroma filter tap decide by luma filter tap. Encoder decide the direction of the filter tap. If luma use 8 filter tap, the chroma use 1x4 or 4x1 filter tap. It is one filter set for whole chroma pixels.

The chroma filter coefficients are coded by using (12).

The chroma filtering reconstructed picture can make same as luma filtering reconstructed picture by (10).

## Analysis complexity of filtering

Table 1 is one pixel’s number of multiplier and addition for star 5x5 and cross 11x5 in HM 4.0.

|  | star 5x5 | cross 11x5 |
| --- | --- | --- |
|  | 8 | 9 |
|  | 15 | 17 |

Table 1 One pixel’s number of multiplier and addition for star 5x5 and cross 11x5 in ALF

Table 2 is one pixel’s number of multiplier and summation for filter tap 4, 8 and 16 in proposed method.

|  | 4 | 8 | 16 |
| --- | --- | --- | --- |
|  | 1.5 | 2.75 | 5.375 |
|  | 3.5 | 5.25 | 8.125 |

Table 2 One pixel’s number of multiplier and addition for filter tap 4. 8, and 16 in FD-ALF

# Simulation Results

Simulation results for all high efficiency configure are shown on Table 3. M is filter size.

Table 3 Simulation results for all high efficiency configure

|  | All Intra HE (M=8) |  |  | All Intra HE (M=16) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 2.2% | 1.2% | 0.8% | 2.2% | 1.2% | 0.8% |
| Class B | 1.3% | 1.8% | 1.3% | 1.3% | 1.8% | 1.3% |
| Class C | 1.1% | 2.6% | 2.7% | 1.1% | 2.6% | 2.7% |
| Class D | 0.5% | 2.4% | 2.7% | 0.5% | 2.4% | 2.7% |
| Class E | 1.9% | 2.8% | 4.3% | 1.9% | 2.8% | 4.3% |
| Class F | 0.6% | 1.7% | 1.4% | 0.6% | 1.7% | 1.4% |
| Overall | 1.2% | 2.0% | 2.1% | 1.2% | 2.0% | 2.1% |
|  | 1.2% | 2.0% | 2.1% | 1.2% | 2.0% | 2.1% |
| Enc Time[%] | 83% |  |  | 83% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Random Access HE (M=8) |  |  | Random Access HE (M=16) |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 3.7% | 5.4% | 5.0% | 3.3% | 4.6% | 4.2% |
| Class B | 2.7% | 3.1% | 1.9% | 2.6% | 3.1% | 2.0% |
| Class C | 1.7% | 1.7% | 1.8% | 1.6% | 1.3% | 1.6% |
| Class D | 1.7% | 1.4% | 1.6% | 1.6% | 1.2% | 1.5% |
| Class E |  |  |  |  |  |  |
| Class F | 0.9% | 1.6% | 1.4% | 1.0% | 1.4% | 1.1% |
| Overall | 2.2% | 2.6% | 2.3% | 2.1% | 2.3% | 2.1% |
|  | 2.2% | 2.6% | 2.3% | 2.1% | 2.3% | 2.1% |
| Enc Time[%] | 97% |  |  | 96% |  |  |
| Dec Time[%] | 91% |  |  | 91% |  |  |
|  | Low delay B HE (M=8) |  |  | Low delay B HE (M=16) |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2.2% | 2.5% | 2.1% | 2.0% | 2.5% | 1.8% |
| Class C | 1.7% | 2.3% | 2.1% | 1.4% | 1.6% | 1.8% |
| Class D | 0.9% | 1.9% | 2.1% | 0.8% | 2.2% | 2.1% |
| Class E | 3.3% | 1.9% | 3.3% | 3.0% | 2.3% | 2.8% |
| Class F | 1.3% | 1.5% | 1.3% | 1.1% | 1.5% | 1.1% |
| Overall | 1.8% | 2.0% | 2.1% | 1.6% | 2.0% | 1.9% |
|  | 1.8% | 2.1% | 2.2% | 1.6% | 2.1% | 2.0% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 94% |  |  | 94% |  |  |
|  | Low delay P HE (M=8) |  |  | Low delay P HE (M=16) |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 4.5% | 3.4% | 3.1% | 4.0% | 3.2% | 2.8% |
| Class C | 2.2% | 1.7% | 2.0% | 2.1% | 1.3% | 1.5% |
| Class D | 1.0% | 1.5% | 1.2% | 1.0% | 1.0% | 1.3% |
| Class E | 5.1% | 3.7% | 4.0% | 4.8% | 3.9% | 4.4% |
| Class F | 1.4% | 1.5% | 1.0% | 1.5% | 1.4% | 0.8% |
| Overall | 2.8% | 2.4% | 2.2% | 2.6% | 2.1% | 2.1% |
|  | 2.8% | 2.4% | 2.2% | 2.7% | 2.1% | 2.1% |
| Enc Time[%] | 96% |  |  | 96% |  |  |
| Dec Time[%] | 92% |  |  | 93% |  |  |

Table 4 shows the case that M is 8 or 16.

Table 4 Simulation results the case that M = 8 or M = 16

|  | All Intra HE (M=8 or M=16) |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 2.0% | 1.2% | 0.7% |
| Class B | 1.3% | 1.7% | 1.3% |
| Class C | 1.1% | 2.4% | 2.5% |
| Class D | 0.5% | 2.3% | 2.6% |
| Class E | 1.8% | 2.7% | 4.2% |
| Class F | 0.6% | 1.7% | 1.4% |
| Overall | 1.2% | 1.9% | 2.0% |
|  | 1.2% | 1.9% | 2.0% |
| Enc Time[%] | 84% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Random Access HE (M=8 or M=16) |  |  |
|  | Y | U | V |
| Class A | 3.3% | 4.6% | 4.3% |
| Class B | 2.6% | 3.0% | 2.0% |
| Class C | 1.6% | 1.3% | 1.7% |
| Class D | 1.7% | 1.2% | 1.4% |
| Class E |  |  |  |
| Class F | 0.8% | 1.4% | 1.2% |
| Overall | 2.0% | 2.3% | 2.1% |
|  | 2.0% | 2.3% | 2.1% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 92% |  |  |
|  | Low delay B HE (M=8 or M=16) |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 2.0% | 2.5% | 1.8% |
| Class C | 1.5% | 1.9% | 2.1% |
| Class D | 0.8% | 1.6% | 2.0% |
| Class E | 3.0% | 1.9% | 3.3% |
| Class F | 1.1% | 1.5% | 1.3% |
| Overall | 1.6% | 1.9% | 2.0% |
|  | 1.6% | 2.0% | 2.1% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 96% |  |  |
|  | Low delay P HE (M=8 or M=16) |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 4.0% | 3.3% | 2.8% |
| Class C | 2.1% | 1.5% | 1.6% |
| Class D | 1.0% | 1.1% | 1.7% |
| Class E | 4.7% | 3.8% | 4.9% |
| Class F | 1.3% | 1.6% | 1.3% |
| Overall | 2.6% | 2.2% | 2.3% |
|  | 2.6% | 2.3% | 2.4% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 93% |  |  |

# Conclusions

In this proposal, the main idea of the proposed method is that adaptive loop filtering is performed on the 1D-DCT transform domain by weighting the coefficients. The proposed method reduces decoding time in average 5% while loss of coding efficiency by in average around 1.8%. It is recommend that this 1D FD-ALF be considered to be adopted in HEVC through CE for further improvement.

# Syntax & Semantics

## Syntax

### Adaptive loop filter parameter syntax

| alf_param() { | C | Descriptor |
| --- | --- | --- |
| adaptive_loop_filter_flag | 2 | u(1) |
| if ( adaptive_loop_filter_flag ) { |  |  |
| alf_region_adaptation_flag | 2 | u(1) |
| alf_realfiltNo | 2 | ue(v) |
| alf_no_filters_minus1 | 2 | ue(v) |
| if (alf_no_filters_minus1 == 1) |  |  |
| alf_start_second_filter | 2 | ue(v) |
| else if (alf_no_filters_minus1 > 1) { |  |  |
| for (i=1; i< 16; i++) |  |  |
| alf_filter_pattern[i] | 2 | u(1) |
| } |  |  |
| for (i=0; i< AlfNumFilters; i++) |  |  |
| alf_filter_1D_pattern[i] | 2 | u(1) |
| if (AlfNumFilters > 1){ |  |  |
| for (i=1; i< AlfNumFilters; i++) |  |  |
| alf_pred_method[i] | 2 | u(1) |
| } |  |  |
| alf_min_kstart_minus1 | 2 | ue(v) |
| for (i=0; i < AlfCodedLengthLuma; i++) |  |  |
| alf_golomb_index_bit[i] | 2 | u(1) |
| for (i=0; i< AlfNumFilters; i++) |  |  |
| for (j=0; j< AlfCodedLengthLuma; j++) { |  |  |
| alf_coeff_luma[i][j] |  | ge(v) |
| alf_coeff_sign_luma[i][j] |  | u(1) |
| } |  |  |
| alf_chroma_idc | 2 | ue(v) |
| if ( alf_chroma_idc ) { |  |  |
| alf_chroma_filter_1D_pattern | 2 | u(1) |
| for( i = 0; i< AlfCodedLengthChroma; i++ ) |  |  |
| alf_coeff_chroma[i] |  | se(v) |
| } |  |  |
| } |  |  |
| } |  |  |

### Adaptive loop filter coding unit control parameter syntax

| alf_cu_control_param() { | C | Descriptor |
| --- | --- | --- |
| if( adaptive_loop_filter_flag ) { |  |  |
| alf_cu_control_flag | 2 | u(1) |
| if( alf_cu_control_flag ) { |  |  |
| alf_cu_control_max_depth | 2 | ue(v) |
| alf_length_cu_control_info | 2 | se(v) |
| for( i = 0; i < NumAlfCuFlag; i++ ) |  |  |
| alf_cu_flag[ i ] | 2 | u(1) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |

## Semantics

alf_filter_1D_pattern[ i ] specifies whether the i-th filter in the filter set is vertical or horizontal direction. The number of filter sets AlfNumFilters is derived by alf_filter_pattern.

alf_pred_mthod [ i ] specifies whether the each filters coefficients are coded in a predictive way. The number of filter sets AlfNumFilters is derived by alf_filter_pattern. If alf_pred_method[i] is 0, it means intra prediction. If afl_pred_method[i] is 1, it means inter prediction. If i is 0, alf_pred_method[0] sets 0.

alf_golomb_index_bit[ i ] specifies the i-th difference in order k of k-th order exponential golomb code for the different groups of the luma filter coefficients. Note that there are several groups of the luma filter coefficients where each group may have different order k.

alf_coeff_sign_luma[ i ][ j ] specifies the i-th filter coefficient sign of j-th filter set for the adaptive loop filtering process for luma samples.

alf_chroma_filter_1D_pattern specifies whether the filter for chroma is vertical or horizontal direction.

# References

- JCTVC-F303, “CE8 Subtest 4: ALF using vertical-size 5 filters with up to 9 coefficients,” 6th JCT-VC Meeting, Torino, July. 2011.

- JCTVC-F700, “Common test conditions and software reference configurations,” 6th JCT-VC meeting, Torino, July. 2011.

# Patent rights declaration(s)

SK telecom and Sejong University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).