| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G669 M22237 |
| --- | --- |

| Title: | Delayed Duplicate I-Frame for Video Conferencing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ranga Ramanujam Srinivasan Chaitanya Ghone Mihir Mody Minhua Zhou | Tel: Email: Tel: Email: | ramanujam@ti.com csghone@ti.com mihir@ti.com zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution presents a technique to reduce the peak data bandwidth on constant bandwidth network (resulting packet loss) because of I/IDR-Frames in a video-conferencing use-case or other real-time multimedia exchange use-cases. Delayed Duplicate I-Frame (DDI-Frame) is an error resiliency tool to enable reliable transmission on constant bit-rate network for bursty IDR/I frame traffic. DDI-Frame is identical to I/IDR-Frame except having additional P-Frame generated from the same input frame. The DDI bit-stream is not sent real-time, unlike P frame which is sent in real time to the decoder end.  Bit-stream will have later reference (after N frames) to the DDI-Frame. This relaxes the transmission time requirement by factor of N for given DDI frame and smooth out bit-rate peaks which essentially cause latency. With this modification, it is claimed that peak bit-rate requirement because of I-Frame reduces by more than 3 times with a marginal increase in computation and bit-rate. The measured DDI-frame bit-rate overhead is around 3% for an I-frame period of 100 frames in simulation, the real overhead should be much smaller because the I-frame period used in the application is 3600 frames (or every 2 minutes).  The proposed change is applied for low-delay LC as well as HE cases and not applicable for Random-Access and All-Intra cases.

# Introduction

Video Conferencing and other real-time multimedia exchange use-cases requires I/IDR-Frame at regular long intervals to keep all end-points refreshed and avoid error propagation because of network packet-loss and to use as long-term reference frame in case of frame-loss. I/IDR-Frames are normally 7 times larger than P-Frames. In bandwidth constrained network settings, I/IDR-Frames results in huge data peaks on network, which are typically contant rate channel resulting in packet loss as well as increased latency of transmission. This results in bad visual experience for user. Figure 1 shows IDR Instantaneous Bandwidth requirement for typical video sequences for constant bit-rate control setting.

Figure 1: IDR Instantaneous Bandwidth requirement

# Proposed Solution

In Delayed Duplicate I-Frame the same Picture/Frame is coded as P-Frame as well as I-Frame as explain in Figure 2. The P-Frame is transmitted in real-time to decode end, whereas the I-Frame (termed as DDI-Frame) is sent in parts in the background with relaxed real-time constraints by factor of N. DDI-Frame is added as Long-Term reference frame is referred to by a distant P-Frame and this distance is referred to as DDI Delay (N) in this proposal. On the decode-side DDI-Frame is received and decoded before the referring P-Frame is decoded and DDI-Frame is never displayed. In a lossy network this DDI-Frame can be sent using reliable protocol as there is a relaxed real-time constraint. This involves one additional frame processing on encode and decode-side for every DDI-Frame transmitted.

P

P

P

I

P

P

P1

P2

I

P

P

P

P11

P10

P9

…

Original

Proposal

DDI Frame

Refers to DDI Frame

Delay

Figure 2: Delayed Duplicate I-Frame Illustrative Diagram

# Changes in Standard

Following are the changes required in the standard,

- New Slice-type defined specifically for DDI-Frames, named ‘D-Slice’. This by characteristics is I/IDR-Frame except that it is sent non-real time and referred to by a distant P-Frame

- 1 bit flag as part of slice header to indicate ‘referring to DDI-Frame’. This bit has to be set to 1 by P-Frame referring to DDI-Frame, for other frames have to set to 0

1 bit per slice before entropy coding is the syntax cost for adding this tool.

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed. Applicable groups for this proposal are,

- Low Delay – P Low Complexity

- Low Delay – P High Efficiency

- Low Delay – B Low Complexity

- Low Delay – B High Efficiency

The test configuration changes for low-delay configurations when compared to the base settings are listed in Table 1 below.

| Parameter | Base Value | Value for DDI |
| --- | --- | --- |
| RateGOPSize | 4 | 3 |
| NumOfReference | 4 | 3 |
| UseNewRefSetting | 1 | 3 |
| IntraPeriod | -1 | 3 |

Table 1: Test Parameter Changes

Two new flags are added specifically for DDI, DDI_PIC_Enabled and DDI_PIC_Delay.

Three different variants of DDI along with HM4.0 are run, the variants are mentioned in Table 2 below.

| Method | DDI_PIC_Enabled | DDI_PIC_Delay |
| --- | --- | --- |
| Original 4.0 | 0 | Ignored |
| DDI Delay 5 | 1 | 5 |
| DDI Delay 10 | 1 | 10 |
| DDI Delay 15 | 1 | 15 |

Table 2: Variants of DDI

# Experimental Results

The simulation results using HM4.0 are summarized in the table 3 to show effect of DDI Delay (N) on bit-rate increase. This simulation is done for Intra-Frame frequency of 100. Video-Conferencing typically uses Intra-Frame frequency of 3600 (or every 2 minutes), which will give much better result compared to simulation result shown in Table 3.

| DDI Delay (N) | DDI_PIC_Enabled | DDI_PIC_Delay | B - HE | B - LC | P - HE | B - LC |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 1 | 5 | 2.2% | 2.2% | 2.2% | 2.2% |
| 10 | 1 | 10 | 2.9% | 2.8% | 2.8% | 2.8% |
| 15 | 1 | 15 | 3.3% | 3.3% | 3.2% | 3.2% |

Table 3: Results for different variants of DDI

Figure 3 plots comparison between traditional I/IDR Frame only bit-stream Vs DDI-Frame with DDI delay (N) of 15. It clearly shows that peak bandwidth with DDI-Frame is reduced by 3 times when compared to traditional I/IDR Frame approach. In this example I-Frame is transmitted in 33 ms to the decoder-end whereas the DDI-Frame has 500 ms to reach decoder-end by reliable means.

Figure 3: Frame Size Vs Frame-Number Plot for I-Frame and DDI-Frame bit-streams

Our LSF cluster normally cannot produce accurate runtime. We have run the case of DDI delay = 10 with the HM4.0 anchor back to back on LSF and obtained relatively accurate runtime. The results are summarized in Table 4.

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 3.0% | 3.7% | 3.4% | 3.0% | 3.4% | 3.3% |
| Class C | 2.6% | 2.8% | 2.9% | 2.5% | 2.5% | 2.6% |
| Class D | 2.6% | 2.7% | 3.2% | 2.5% | 2.5% | 2.5% |
| Class E | 3.5% | 3.0% | 3.5% | 3.4% | 3.0% | 3.1% |
| Class F |  |  |  |  |  |  |
| Overall | 2.9% | 3.1% | 3.3% | 2.8% | 2.9% | 2.9% |
|  | 2.9% | 3.1% | 3.3% | 2.8% | 2.9% | 2.9% |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 3.0% | 3.5% | 3.4% | 2.9% | 3.3% | 3.2% |
| Class C | 2.5% | 2.6% | 2.7% | 2.5% | 2.3% | 2.4% |
| Class D | 2.5% | 2.5% | 2.5% | 2.4% | 2.4% | 2.3% |
| Class E | 3.4% | 3.1% | 3.5% | 3.4% | 2.4% | 2.9% |
| Class F |  |  |  |  |  |  |
| Overall | 2.8% | 3.0% | 3.0% | 2.8% | 2.7% | 2.7% |
|  | 2.8% | 3.0% | 3.0% | 2.8% | 2.7% | 2.8% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

Table 4. Results for DDI delay = 10. (see ddi_delay_10_time.xls)

# Conclusions

The proposed Delayed Duplicate I-Frame is aimed to reduce latency for video conferencing use case, it is recommended to conduct further study on this tool in CE.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).