| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30, Nov., 2011 | Document: JCTVC-G671 |
| --- | --- |

| Title: | CE3: Crosscheck for Qualcomm’s Adaptive MVD Resolution in JCTVC-G727 |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | *Jicheng An *Xun Guo *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 | Email: | jicheng.an@mediatek.com xun.guo@mediatek.com |  |
| Source: | MediaTek Inc. |  |  |  |

_____________________________

# Abstract

The purpose of this document is to cross-check the proposal JCTVC-G727 in CE3 Tool 11 proposed by Qualcomm. It is reported that the results match those provided by Qualcomm.

# Introduction

An adaptive MVD resolution method was proposed in JCTVC-F567 [1] and JCTVC-G727. In JCTVC-G727, MVD resolution is explicitly signaled with specific syntax based on the magnitude of MV difference (MVD).The explicit signal can indicate whether MVD is in 1/2-pel or 1/4-pel resolution or 1/4-pel and 1/8 pel resolution. A threshold signaled in slice header to determine the freedom of the adaptation.

# Experimental Results

The experiments are conducted according to the common test condition JCTVC-F900 [2]. There are two versions to be tested. In the first version (quarter version), the explicit signal indicates whether MVD is in 1/2-pel or 1/4-pel resolution. In the second version (eighth version), the explicit signal indicates whether MVD is in 1/4-pel or 1/8-pel resolution. Table 1 and Table 2 show the performance of these two versions respectively.

Table 1 Performance of quarter version

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -1.0% | -0.9% | -0.5% | -0.6% | -0.7% |
| Class B | -0.2% | -0.5% | -0.4% | -0.3% | -0.5% | -0.3% |
| Class C | -0.2% | -0.5% | -0.5% | -0.3% | -0.5% | -0.5% |
| Class D | 0.1% | -0.2% | -0.3% | 0.0% | -0.4% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | -0.6% | -0.5% | -0.3% | -0.5% | -0.4% |
|  | -0.2% | -0.6% | -0.5% | -0.3% | -0.5% | -0.4% |
| Enc Time[%] | 95% |  |  | 94% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | -5% |  |  | -5% |  |  |
| Adds[%] | -5% |  |  | -6% |  |  |
| Access[%] | -3% |  |  | -3% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 5% |  |  | 10% |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 5% |  |  | 9% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.5% | -0.5% | -0.3% | -0.2% | -0.1% |
| Class C | -0.2% | -0.2% | -0.3% | -0.3% | -0.1% | -0.4% |
| Class D | 0.1% | 0.0% | -0.1% | 0.0% | 0.2% | -0.3% |
| Class E | -0.1% | -0.2% | 0.5% | -0.3% | 0.2% | 0.2% |
| Overall | -0.1% | -0.3% | -0.2% | -0.2% | 0.0% | -0.2% |
|  | -0.1% | -0.3% | -0.2% | -0.2% | -0.1% | -0.2% |
| Enc Time[%] | 93% |  |  | 93% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | -8% |  |  | -8% |  |  |
| Adds[%] | -8% |  |  | -9% |  |  |
| Access[%] | -5% |  |  | -5% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 5% |  |  | 10% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | 5% |  |  | 9% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | -0.2% | 0.1% | 0.0% | 0.1% |
| Class D | 0.3% | 0.3% | 0.3% | 0.3% | 0.4% | -0.1% |
| Class E | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | -0.2% |
| Overall | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
|  | 0.1% | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 96% |  |  | 96% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
| Mult[%] | -8% |  |  | -8% |  |  |
| Adds[%] | -8% |  |  | -8% |  |  |
| Access[%] | -5% |  |  | -5% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 2% |  |  | 1% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | 2% |  |  | 1% |  |  |
| Gain Ave.[%] | -0.09% | -0.19% | -0.22% |  |  |  |
| Gain Min. [%] | -0.93% | -1.83% | -1.63% |  |  |  |
| Gain Max.[%] | 0.82% | 1.44% | 1.44% |  |  |  |
| Enc Time[%] | 94.53% |  |  |  |  |  |
| Dec Time[%] | 100.72% |  |  |  |  |  |
| Mult Ave.[%] | -7% |  |  |  |  |  |
| Mult Worst[%] | 0% |  |  |  |  |  |
| Adds Ave.[%] | -7% |  |  |  |  |  |
| Adds Worst[%] | 0% |  |  |  |  |  |
| Access[%] | -4% |  |  |  |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  |  |  |  |
| MemBand(2D) Max.[%] | 5% |  |  |  |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |  |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  |  |  |  |
| MemBand(1D) Max.[%] | 5% |  |  |  |  |  |
| MemBand(1D) Worst[%] | 0% |  |  |  |  |  |

Table 2 Performance of eighth version

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | -0.2% | -0.4% | 0.3% | -0.6% | -0.5% |
| Class B | -0.1% | -0.2% | -0.4% | -0.2% | -0.6% | -0.7% |
| Class C | -0.6% | -0.7% | -0.5% | -0.7% | -1.3% | -1.2% |
| Class D | -0.8% | -0.9% | -1.2% | -1.2% | -1.8% | -2.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -0.5% | -0.6% | -0.4% | -1.1% | -1.1% |
|  | -0.3% | -0.5% | -0.6% | -0.4% | -1.0% | -1.1% |
| Enc Time[%] | 119% |  |  | 121% |  |  |
| Dec Time[%] | 104% |  |  | 106% |  |  |
| Mult[%] | 18% |  |  | 18% |  |  |
| Adds[%] | 18% |  |  | 18% |  |  |
| Access[%] | 10% |  |  | 10% |  |  |
| MemBand(2D) Ave.[%] | 3% |  |  | 3% |  |  |
| MemBand(2D) Max.[%] | 10% |  |  | 11% |  |  |
| MemBand(1D) Ave.[%] | 3% |  |  | 3% |  |  |
| MemBand(1D) Max.[%] | 8% |  |  | 11% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3% | -0.3% | -0.5% | 0.3% | -1.1% | -1.2% |
| Class C | -0.2% | -0.6% | -0.5% | -0.5% | -1.6% | -2.0% |
| Class D | -0.8% | -0.6% | -1.9% | -1.3% | -2.5% | -3.6% |
| Class E | 1.3% | 0.8% | 1.0% | 1.0% | 0.0% | -0.1% |
| Overall | 0.1% | -0.2% | -0.6% | -0.2% | -1.4% | -1.8% |
|  | 0.1% | -0.3% | -0.5% | -0.2% | -1.4% | -1.7% |
| Enc Time[%] | 120% |  |  | 121% |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |
| Mult[%] | 12% |  |  | 13% |  |  |
| Adds[%] | 12% |  |  | 13% |  |  |
| Access[%] | 5% |  |  | 5% |  |  |
| MemBand(2D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(2D) Max.[%] | 12% |  |  | 6% |  |  |
| MemBand(1D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(1D) Max.[%] | 12% |  |  | 7% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.7% | -1.5% | -1.7% | -1.4% | -1.8% | -1.7% |
| Class C | -3.2% | -2.7% | -2.6% | -2.8% | -3.2% | -3.5% |
| Class D | -5.1% | -4.8% | -5.5% | -4.9% | -5.2% | -6.4% |
| Class E | -1.7% | -0.7% | -0.9% | -1.0% | -1.5% | -0.8% |
| Overall | -2.9% | -2.5% | -2.7% | -2.5% | -2.9% | -3.2% |
|  | -2.9% | -2.4% | -2.7% | -2.6% | -2.9% | -3.2% |
| Enc Time[%] | 117% |  |  | 119% |  |  |
| Dec Time[%] | 103% |  |  | 104% |  |  |
| Mult[%] | 17% |  |  | 17% |  |  |
| Adds[%] | 17% |  |  | 17% |  |  |
| Access[%] | 7% |  |  | 7% |  |  |
| MemBand(2D) Ave.[%] | 2% |  |  | 3% |  |  |
| MemBand(2D) Max.[%] | 9% |  |  | 12% |  |  |
| MemBand(1D) Ave.[%] | 2% |  |  | 3% |  |  |
| MemBand(1D) Max.[%] | 9% |  |  | 12% |  |  |
| Gain Ave.[%] | -1.05% | -1.43% | -1.66% |  |  |  |
| Gain Min. [%] | -15.59% | -13.69% | -18.73% |  |  |  |
| Gain Max.[%] | 1.70% | 2.51% | 1.59% |  |  |  |
| Enc Time[%] | 119.38% |  |  |  |  |  |
| Dec Time[%] | 104.31% |  |  |  |  |  |
| Mult Ave.[%] | 16% |  |  |  |  |  |
| Mult Worst[%] | 0% |  |  |  |  |  |
| Adds Ave.[%] | 16% |  |  |  |  |  |
| Adds Worst[%] | 0% |  |  |  |  |  |
| Access[%] | 7% |  |  |  |  |  |
| MemBand(2D) Ave.[%] | 2% |  |  |  |  |  |
| MemBand(2D) Max.[%] | 10% |  |  |  |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |  |  |  |
| MemBand(1D) Ave.[%] | 2% |  |  |  |  |  |
| MemBand(1D) Max.[%] | 10% |  |  |  |  |  |
| MemBand(1D) Worst[%] | 0% |  |  |  |  |  |

# Conclusion

The results presented by CE3 Tool 11 in JCTVC-G727 are confirmed.

# References

- W. Chien, P. Chen, X. Wang, and M. Karczewicz, “Adaptive resolution on motion vector difference”, JCTVC-F567, Torino, IT, July, 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, IT, July, 2011.