| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 Nov, 2011 | Document: JCTVC-G674 WG11 Number: |
| --- | --- |

| Title: | CE5: Sub-block coding of transform coefficients with CAVLC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Yunfei Zheng, Liwei Guo, Xianglin Wang 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: 1-858-658-5673 Email: martak@qualcomm.com zhengy@qualcomm.com liweig@qualcomm.com xianglin@qualcomm.com |  |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution is to report CE results on sub-block coding of transform coefficients using CAVLC based on proposal JCTVC-F612. According to the proposal, 16x16 and 32x32 transform coefficient blocks are divided into 4x4 sub-blocks. For the sub-block information is sent whether it has any non-zero coefficients. Simulation results show that the proposed scheme provides BD-rate savings of 0.6% for Intra, 0.2% for random access and 0.1% for low delay configuration.

# Introduction

This document presents CE results based on JCTVC-F612 on top of HM4.0. The proposed scheme aims at improving coding efficiency of transform coefficients of large blocks using CAVLC.

# Proposed Method

According to JCTVC-F612, the 16x16 and 32x32 transform coefficient blocks are divided into 4x4 sub-blocks.  After the position of the last nonzero coefficient,  lastPos,  in 16x16 or 32x32 block is sent, for each 4x4 sub-block at position (xS,yS) two conditions are checked:

- The number of coded coefficients in this sub-block is larger than 4: noCoeff[xS][yS]>4.

- The average number of non-zero coefficients in this sub-block in the previously coded blocks is less than a threshold, i.e. noCodedCoeff[xS][yS]<=subBlkThreshold.

For each sub-block for which both conditions are true a single bit is sent. The bit is equal to zero if all the transform coefficients in this sub-block are zero. Remaining coefficients in 16x16 or 32x32 block are coded using current coding method for these blocks.

Values in the variable array noCoeff[xS][yS] are initialized to 0 before encoding 16x16 or 32x32 block and calculated using value of lastPos:

for (pos=0; pos<= lastPos; pos++) {

xC= ScanOrder [log2TrafoSize−2][scanIdx][pos][0];

yC= ScanOrder [log2TrafoSize−2][scanIdx][pos][1];

noCoeff[xC/4] [yC/4]++;

}

where the scanning order array ScanOrder specifies the mapping of the scan position pos to transform coefficient position (xC, yC) within the current transform block. Variable log2TrafoSize has value of 4 for 16x16 and 5 for 32x32 blocks.

After the transform coefficient level block, transCoeffLevel, is encoded, the variable arrays noSubBlks  and noCodedCoeff are updated as follows:

for (pos=0; pos<= lastPos; pos++){

xC= ScanOrder [log2TrafoSize−2][scanIdx][pos][0];

yC= ScanOrder [log2TrafoSize−2][scanIdx][pos][1];

if (transCoeffLevel[xC][yC] != 0){

noCodedCoeff[log2TrafoSize−4][scanIdx] [xC/4][yC/4]++;

}

}

for (xS=0; xS< blkSize/4; xS++)

for (yS=0; yS< blkSize/4; yS++)

if (noCoeff[xS] [yS]>0)

noSubBlks[log2TrafoSize−4][scanIdx][xS][yS]++;

Values of noSubBlks and noCodedCoeff for sub-lock at position (xS, yS) are divided by 2 if any of them becomes larger than 16. Values of variable noSubBlks  are used to derive subBlkThreshold as specified in Table 2-1.

Table 2-1 – Derivation of subBlkThreshold from noSubBlks[xS][yS]

| noSubBlks[xS][yS] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| subBlkThreshold | 32 | 2 | 4 | 7 | 9 | 12 | 14 | 17 | 19 | 22 | 24 | 27 | 29 | 32 | 32 | 32 |

Intra luma 16x16 and 32x32 blocks are using the same mapping between index of the VLC table, vlcNum, and maximum run maxRunIdx as intra luma 4x4 and 8x8 blocks.

# Simulation Results

Simulations are performed using all three low complexity configurations[1]. Results are shown in Table 1. Detailed results can be found in the associated excel data sheet.

Table 1. Simulation results with reference to HM4.0

|  | LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| AI | -0.6 | 0.1 | 0.2 |
| RA | -0.2 | -0.1 | 0.2 |
| LB | -0.1 | -0.1 | -0.1 |

In terms of complexity, there is no obvious encoding/decoding time difference observed compared against HM4.0.

# Conclusions

Simulation results show that the sub-block coding scheme can further improve coding efficiency of transform coefficients with no complexity overhead. We recommend such a scheme to be adopted into HM.

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.

- M. Karczewicz, Y. Zheng, L. Guo, X. Wang, “Modifications to intra blocks coefficient coding with VLC”, JCTVC-F612, July 2011.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: proposed text change with sub-block coding

## Residual coding CAVLC syntax

| residual_coding_cavlc( x0, y0, log2TrafoSize, trafoDepth, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| n = ( 1 << ( log2TrafoSize << 1 ) ) >> ( cIdx > 0 ? 2 : 0 ) |  |
| last_pos_level_one | ce(v) |
| lastPos = last_pos_level_one % n |  |
| levelGreaterThanOneFlag = ( last_pos_level_one > n ) |  |
| level_and_sign( levelGreaterThanOneFlag, transCoeffLevel, x0, y0, trafoDepth, cIdx, lastPos) |  |
| if (log2TrafoSize >= 4) { |  |
| numBlocks = n>>4 |  |
| getNumCoeffs ( numCoeffs, log2TrafoSize, scanIdx, lastPos) |  |
| for ( i = 0; i < numBlocks; i++) |  |
| zeroBlk [i]= (numCodedCoeff [i] <= subBlkThreshold[noSubBlks] ) ? 1:0 |  |
| zeroBlk[getBlkIndex(log2TrafoSize, scanIdx, lastPos)] = 0 |  |
| for ( i = 0; i < numBlocks; i++) { |  |
| if (numCoeffs [ i ] > 4 && zeroBlk [i] ) { |  |
| coded_blk_flag | u(1) |
| zeroBlk[i] = coded_blk_flag |  |
| } |  |
| } |  |
| numZeroes = 0 |  |
| for ( i = 0; i <= lastPos; i++) |  |
| if( zeroBlk[getBlkIndex(log2TrafoSize, scanIdx, i)] ) |  |
| numZeroes ++ |  |
| lastPos = lastPos – numZeroes |  |
| } |  |
| runModeFlag = TRUE |  |
| n = lastPos – 1 |  |
| sumBigCoeff = 0 |  |
| switchThres = ( PredMode == MODE_INTRA && cIdx == 0)? 0 : 49 |  |
| while (runModeFlag && n >= 0) { |  |
| run_level_one | ce(v) |
| runOfZeros = run_level_one % n |  |
| levelGreaterThanOneFlag = (run_level_one > n) |  |
| trOne = (trOne = = 0 ¦ ¦ levelGreaterThanOneFlag) ? 0 : Max(4, trOne + 1) |  |
| if (log2TrafoSize >= 4) { |  |
| i = 0 |  |
| while( i < runOfZeros ) |  |
| zeroBlk[getBlkIndex(log2TrafoSize, scanIdx, n - i)] ? (i++): (n--) |  |
| } |  |
| n = n – runOfZeros |  |
| if( n >= 0 ) { |  |
| level_and_sign( levelGreaterThanOneFlag, transCoeffLevel, x0, y0, trafoDepth, cIdx, n) |  |
| if ( levelGreaterThanOneFlag ) { |  |
| sumBigCoeff = sumBigCoeff + level |  |
| if ( log2TrafoSize==2 ¦ ¦ n > switchThres ¦ ¦ sumBigCoeff > 2 ) |  |
| runModeFlag = FALSE |  |
| } |  |
| } |  |
| n-- |  |
| } |  |
| while ( n >= 0 ) { |  |
| level | ce(v) |
| if ( level > 0 ) { |  |
| sign_flag | u(1) |
| sign = 1 – 2 * sign_flag |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n ] = level * sign |  |
| } |  |
| n-- |  |
| } |  |
| } |  |

Function definition:

getBlkIndex ( log2TrafoSize, scanIdx, pos)

{

xC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx  ][ pos ][ 0 ]

yC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx  ][ pos ][ 1 ]

return( (yC>>2<< (log2TrafoSize-2)) + (xC>>2) )

}

getNumCoeffs ( numCoeffs, log2TrafoSize, scanIdx, lastPos)

{

numCoeffs[i] = 0, with i = 0…numBlocks-1

for ( i = 0; i <= lastPos; i++)

numCoeffs[ getBlkIndex(log2TrafoSize, scanIdx, i) ] ++

}