| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30, Nov., 2011 | Document: JCTVC-G675 |
| --- | --- |

| Title: | Non-CE12: Crosscheck for LGE’s Simplified Bs Calculation in JCTVC-G176 |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | *Jicheng An *Xun Guo *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 | Email: | jicheng.an@mediatek.com xun.guo@mediatek.com |  |
| Source: | MediaTek Inc. |  |  |  |

_____________________________

# Abstract

The purpose of this document is to cross-check the proposal JCTVC-G176 proposed by LGE. It is reported that the results match those provided by LGE exactly. The subjective quality is reported to be similar to HM4.0.

# Introduction

In HM4.0, the BS calculation unit is 4x4 and the filtering unit is 8x8, therefore an additional BS unification process is needed for the 8x8 block edge. In JCTVC-G176[1], the BS calculation unit is modified to 8x8, i.e., the first 4x4 vertical and horizontal edges in every 8x8 block are checked and allocated with a proper BS value which is to be used for whole 8x8 block edge.

The proposed method can reduce the BS calculation process and remove the additional unification process used in current HM4.0.

# Experimental Results

The experiments are conducted according to the common test condition JCTVC-F900 [2]. Table 1 shows the experimental results achieved by LGE’s software, which match those provided by LGE.

Table 1 Performance of simplified BS calculation

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.1% | 0.1% | 0.1% | 0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | 0.1% | 0.2% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% |
| Class D | 0.1% | 0.4% | 0.2% | 0.0% | 0.2% | -0.2% |
| Class E | 0.0% | -0.2% | 0.6% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# Conclusion

The results presented by LGE JCTVC-G176 are confirmed.

# References

- S. Park and B. Jeon, “Simplified BS calcualtion process in deblocking filter”, JCTVC-G176, Geneva, CH, Nov., 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, IT, July, 2011.