| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G682 M22250 |
| --- | --- |

| Title: | Non-CE8: Reduced number of band offsets in SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Do-Kyoung Kwon 12500 TI Blvd MS 8649 Dallas TX USA 75243 | Tel: Email: | 1-214-480-1562 wskim@ti.com d-kwon@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In the current HM-4.0 design SAO parameters are encoded into adaptation parameter set (APS), and need to be stored in a buffer until SAO process is completed for each picture. The buffer size is proportional to the number of SAO partitions and size of SAO parameters for each partition. In this contribution the number of band offsets is reduced from 16 to 8, and number of bands is increased from 2 to 4. With this scheme, the number of band offsets that a decoder needs to buffer can be effectively reduced from 16 to 8. The experimental results and subjective viewing show that the proposed scheme reduces the SAO parameter buffer size on the decoder side by half without degrading subjective and objective quality. The measured BD-rate reduction is 0.1% in LB-LC, and 0.0% in all the other configurations.

# Introduction

Sample adaptive offset (SAO) [1] improves visual quality by adding an offset to each reconstructed pixel. SAO parameters are encoded and included in a slice header. At a decoder side, SAO parameters have to be stored in a buffer after decoding slice header until SAO process is completed for each slice. The buffer size is proportional to the number of partitions and size of SAO parameters. In many implementations, the buffer has to be prepared beforehand, and need to provide enough space to store information in worst case. The worst case happens when a slice or frame is divided into maximum number of partitions, and all the partition uses maximum number of offsets.

In the current SAO design, band offset (BO) requires the maximum number of offset, which is 16. Considering edge offset (EO) only uses 4 offsets, BO seems to use too many offsets, and this causes large buffer size to store them.

In this document, we propose to reduce the number of band offset from 16 to 8. This results in reducing the buffer size to store offsets by half. Experimental results show that there is no subjective and objective quality degradation when the proposed method is applied.

# Proposed Method

Current BO design divides entire intensity level range into 2 bands as shown in Fig. 1 (a). One of the two bands is selected for a given partition, and the selected band is coded as SAO type. For each band the intensity level range is equally divided into 16 sub-bands, and one offset per sub-band is provided. This requires 16 offsets to be coded and stored in the buffer.

(a) Two bands in the current band offset design

(b) Four bands as in the proposed method

Fig. 1. Band division for band offset

Fig. 2. Extended coverage of the first and last sub-band

We propose to divide intensity level into 4 bands as shown in Fig. 1 (b). Selected band is coded as SAO type. For each band the intensity level range is divided into 8 sub-bands. Therefore, only 8 offsets need to be coded and stored in the buffer.

By increasing number of band from 2 to 4 and decreasing number of sub-band from 16 to 8, one offset can cover the same length of range in the proposed method as in the current scheme. However, one of the differences is that the coverage of one band is narrower than before. To resolve this issue, the coverage of offsets of the first and the last sub-band are extended to the pixels outside the band. This is illustrated in Fig. 2. The offset of the first sub-band covers pixels smaller than the minimum bound of the given band in addition to its own sub-band. The offset of the last sub-band covers pixels larger than the maximum bound of the given band in addition to its sub-band.

Note that the two middle bands are assigned as BO type 1 and 2, and the other bands are assigned as BO type 3 and 4, according to the usage frequency in many test sequences. Also note that the same method is applied for all color components.

# Experimental Results

The proposed solution is implemented using HM-4.0 reference code. We have performed subjective quality evaluation using the test sequences used in [2], and verified that there is no subjective quality degradation in all the sequences when the proposed method is applied.

Table 1 is the summary of the simulation results in terms of the objective quality generated using the common test conditions [3], where only the 8 band offset proposal described in Fig. 1 is tested. The simulations have used HM4.0 software with the AMP encoder speedup disabled. The simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570 64 bits Linux machines of different frequencies. On average there is 0.0% bitrate reduction in Y, and -0.2% bitrate reduction both in U and V.

Table 2 is the summary of the simulation results containing both 8 band offset proposal and sub-band extension method illustrated in Fig. 2. On average there is 0.0% bitrate reduction in Y, and -0.1% bitrate reduction both in U and V. Note that there are -0.2% and -0.3% bitrate reduction in Y in case of LB-HE and LB-LC, respectively, for Class E.

# Conclusion

A new band offset scheme has been proposed, in which the number of band is increased from 2 to 4, and number of sub-band is reduced from 16 to 8, so that the buffer size to store these offsets can be reduced by half. Experimental results show that the proposed scheme maintains both subjective and objective quality as before.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-F803, Jul. 2011.

- O. G. Sezer and M. Budagavi, “Subjective Tests on ALF and SAO”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-F320, Jul., 2011.

- F. Bossen, “Common test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-F900, Jul. 2011.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1. BD-rate results of the proposed method: 8 band offsets without sub-band extension.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.2% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class E | 0.0% | -0.3% | -0.1% | 0.0% | -0.2% | -0.1% |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 98% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 96% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.4% | -0.6% | 0.0% | -0.2% | -0.2% |
| Class B | 0.0% | -0.2% | -0.1% | 0.0% | -0.3% | -0.2% |
| Class C | 0.0% | -0.2% | 0.1% | 0.0% | -0.3% | -0.1% |
| Class D | 0.0% | -0.2% | -0.3% | 0.0% | -0.3% | -0.3% |
| Overall | 0.0% | -0.2% | -0.2% | 0.0% | -0.3% | -0.2% |
|  | 0.0% | -0.2% | -0.2% | 0.0% | -0.3% | -0.2% |
| Enc Time[%] | 95% |  |  | 98% |  |  |
| Dec Time[%] | 96% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class B | 0.0% | -0.5% | -0.4% | 0.0% | -0.3% | -0.4% |
| Class C | 0.0% | 0.0% | -0.3% | -0.1% | -0.2% | -0.3% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.3% |
| Class E | 0.0% | -0.4% | 0.2% | -0.1% | -0.2% | -0.6% |
| Overall | 0.0% | -0.2% | -0.2% | -0.1% | -0.2% | -0.4% |
|  | 0.0% | -0.3% | -0.3% | -0.1% | -0.2% | -0.4% |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | -0.3% | -0.1% |
| Class C | 0.0% | -0.1% | -0.2% | 0.0% | -0.6% | -0.2% |
| Class D | 0.1% | 0.2% | -0.6% | 0.0% | -0.6% | -0.5% |
| Class E | -0.1% | -1.4% | -0.3% | 0.0% | -0.5% | -0.6% |
| Overall | 0.0% | -0.3% | -0.4% | 0.0% | -0.5% | -0.3% |
|  | 0.0% | -0.3% | -0.5% | 0.0% | -0.5% | -0.4% |
| Enc Time[%] | 97% |  |  | 94% |  |  |
| Dec Time[%] | 98% |  |  | 93% |  |  |

Table 2. BD-rate results of the proposed method: 8 band offsets with sub-band extension.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.2% | 0.2% | 0.0% | -0.2% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 105% |  |  | 102% |  |  |
| Dec Time[%] | 104% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.3% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.2% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.3% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 96% |  |  | 99% |  |  |
| Dec Time[%] | 94% |  |  | 97% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | 0.1% | 0.0% | 0.1% | 0.2% |
| Class C | 0.0% | -0.1% | -0.3% | 0.0% | -0.2% | -0.5% |
| Class D | 0.0% | 0.2% | -0.2% | 0.0% | 0.3% | -0.3% |
| Class E | -0.2% | -0.6% | 0.8% | -0.3% | 0.0% | 0.2% |
| Overall | 0.0% | -0.2% | 0.1% | -0.1% | 0.0% | -0.1% |
|  | 0.0% | -0.2% | 0.0% | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 97% |  |  | 102% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class B | 0.0% | 0.2% | 0.3% | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | -0.4% | -0.2% |
| Class D | 0.0% | -0.1% | -0.6% | 0.0% | -0.5% | -0.7% |
| Class E | -0.2% | -0.4% | 0.4% | 0.0% | -0.3% | -0.4% |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | -0.3% | -0.3% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | -0.3% | -0.3% |
| Enc Time[%] | 100% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

# WD Text Changes (changes marked in yellow)

7.4.3.4	Sample adaptive offset parameter semantics

…

Table 7-1 – Specification of NumSaoClass

| - sao_type_idx[ cIdx ][ saoDepth ][ rx ][ ry ] | - NumSaoCategory | - Edge type (informative) |
| --- | --- | --- |
| - 0 | - 0 | - Not applied |
| - 1 | - 4 | - 1D 0-degree edge |
| - 2 | - 4 | - 1D 90-degree edge |
| - 3 | - 4 | - 1D 135-degree edge |
| - 4 | - 4 | - 1D 45-degree edge |
| - 5 | - 8 | - Central left band |
| - 6 | - 8 | - Central right band |
| - 7 | - 8 | - Side left band |
| - 8 | - 8 | - Side right band |

8.6.2.1.1	Modification process for luma and chroma samples

…

–	Otherwise, if saoTypeIdx is equal to one of the values of 5, 6, 7, and 8, the following ordered steps applies:

…

(According to 8 band offset proposal only without sub-band extension)

Table 8-2 – Specification of array bandTable according to bandIdx

| bandIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| bandTable[1][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[2][bandIdx] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[3][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandIdx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| bandTable[0][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[1][bandIdx] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[2][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| bandTable[3][bandIdx] | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |

(According to 8 band offset proposal with sub-band extension)

Table 8-3 – Specification of array bandTable according to bandIdx

| bandIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bandTable[0][bandIdx] | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| bandTable[1][bandIdx] | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| bandTable[2][bandIdx] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| bandTable[3][bandIdx] | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| bandIdx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| bandTable[0][bandIdx] | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| bandTable[1][bandIdx] | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| bandTable[2][bandIdx] | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| bandTable[3][bandIdx] | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |