| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G684 M22252 |
| --- | --- |

| Title: | Subjective Tests on ALF and SAO Using HM-4.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim Osman G. Sezer Madhukar Budagavi 12500 TI Blvd MS 8649 Dallas TX USA 75243 | Tel: Email: | 1-214-480-1562 wskim@ti.com osman@ti.com madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution presents results of informal subjective tests conducted on two loop filtering operations in HM-4.0: SAO and ALF. Test videos were presented at their actual frame rates to the viewers. Three configurations were tested using low delay B high efficiency (LB-HE) condition: HM-4.0 Anchor vs. SAO-off+ALF-off, HM-4.0 Anchor vs. SAO-off+ALF-on, and HM-4.0 Anchor vs. SAO-on+ALF-off. In HM-4.0 Anchor, both SAO and ALF are activated (SAO-on+ALF-on). Among these three configurations, SAO-on+ALF-off gives subjective quality results better than or comparable to the HM-4.0 Anchor. This contribution requests that JCT-VC conducts subjective quality test of SAO and ALF in a core experiment setting to evaluate the subjective quality gains provided by the aforementioned tools, and decide on the minimal set of configurations that provide the best subjective quality improvements, so that the optimal performance can be achieved in the final standard in terms of implementation cost and visual quality.

# Introduction

Informal subjective test are conducted on two separate loop filtering techniques in HM-4.0: Adaptive Loop Filter (ALF) and Sample Adaptive Offset (SAO) [1]. ALF seeks for a set of filters learned through Wiener-Hopf equations that tries to get the reconstructed frame close to the original one via iterative minimization of rate-distortion cost. SAO, on the other hand, applies a set of offset values representing different local geometric features by adding these values to the reconstructed pixels when that feature is observed in the filtering process. These two filtering operations are observed to provide bit savings in HM-4.0.

Similar subjective quality test was performed using HM-3.0 [2]. The main difference is that in HM-4.0 SAO is also applied to chroma components.

To better understand the subjective gains provided by SAO and ALF when they work together or separately, we have devised several experiments. Total three test configurations are evaluated compared to the anchor as in Table 1.

Table 1. Test configurations

|  | SAO | ALF |
| --- | --- | --- |
| Anchor | on | on |
| Test 1 | off | off |
| Test 2 | off | on |
| Test 3 | on | off |

Due to limited resources, six sequences are selected from HEVC committee streams, for which the loop filters provide some subjective difference at low bitrates (high QPs) when a frame-by-frame comparisons is done. The reported subjective tests in this document try to understand how these subjective differences reflect on actual viewer experience when the sequences are viewed in their original frame rates.

In the experiments, each sequence is encoded using HM-4.0 Anchor with the low delay B high efficiency (LB-HE) test condition [3]. Reconstructed streams of the anchor and one of the test configurations are presented consecutively but in randomized orders to the viewers. Note that the order of test configurations in each sequence is also randomized, so that the viewer cannot know which test configuration is under the test.

In these viewing sessions, the subjects are asked to report the video that seems better, or mark neutral if they cannot see a difference. Note that neutral selection is inserted to avoid wrong interpretation of the test due to limited number of viewers; however, it is recommended for the viewers not to select neutral when they see difference between two streams.

Since the first frame of the second video is likely to have higher PSNR values compared to the last frame of the first video, the viewers might be bias towards reporting the second video as the better one. As a remedy, the subjects are asked to compare the subjective quality of same segments in two videos, and make an overall decision.

# Test Conditions

A 46-inch LCD HDTV monitor with an approximate viewing distance of 80 inches is used.  Streams are played with original frame rates. QP values of 32 and 37 are used with six sequences, hence a total of 12 streams are shown in each sessions. The following HEVC committee streams are included in the experiments:

•	BasketballDrill_p832x480_50fps_420pl_500fr.yuv

•	RaceHorsesC_p832x480_30fps_420pl_300fr.yuv

•	vidyo1_p1280x720_60fps_420pl_600fr.yuv

•	vidyo3_p1280x720_60fps_420pl_600fr.yuv

•	vidyo4_p1280x720_60fps_420pl_600fr.yuv

•	BasketballDrive_p1920x1080_50fps_420pl_500fr.yuv

The viewing sessions are done on different days with three viewers for each test configuration. There are three test configurations used in our experiments. Each test configuration is subjectively compared against the HM-3.0 Anchor. From three viewers and 12 sequences, 36 different scores are obtained for our informal subjective tests. The viewers have three options. They can, without knowing, vote for test or anchor configuration, or they can stay neutral. The total number of neutral, test, and anchor decisions are used to compare subjective gains. Basically, higher neutral score means the test configuration looks similar to the HM-4.0 Anchor.

# Results

Table 2 provides the summary of subjective scores gathered from our experiments. In the first test configuration, both SAO and ALF are disabled. This experiment is designed to provide a baseline for the remaining tests. In other words, once SAO or ALF is separately enabled, they can be compared against this baseline test configuration. Any improvement over the baseline therefore can be detected.

In the second test configuration, SAO is disabled while ALF is enabled. It can be noticed that the score of this configuration is even worse than that of the first configuration. Especially the score of the Test case is the lowest among the configurations. This result may suggest that this configuration would not provide optimal performance in terms of the visual quality.

The third test configuration is the reverse case, where ALF is disabled and SAO is enabled. Compared to baseline experiment, the third test configuration (SAO on, ALF off) produced the highest Neutral and lowest Anchor scores. This reveals that similar preference between the anchor and the test case is observed.

Table 2. Summary of the subjective test scores conducted on ALF and SAO. Each configuration has a total of 36 observations.

| Test configuration | Number of votes |  |  |
| --- | --- | --- | --- |
|  | Anchor | Test | Neutral |
| SAO off, ALF off | 14 | 6 | 16 |
| SAO off, ALF on | 17 | 1 | 18 |
| SAO on, ALF off | 9 | 8 | 19 |

# Summary

This contribution presented results of informal subjective tests conducted on two loop filtering operations in HM-4.0: SAO and ALF. Test videos were presented at their actual frame rates to the viewers. Three configurations were tested using low delay B high efficiency (LB-HE) condition: HM-4.0 Anchor vs. SAO-off+ALF-off, HM-4.0 Anchor vs. SAO-off+ALF-on, and HM-4.0 Anchor vs. SAO-on+ALF-off. Among these three configurations, SAO-on+ALF-off provided subjective quality better than or comparable to that of the HM-4.0 Anchor.

This contribution requests that JCTVC conduct subjective quality test of SAO and ALF in a core experiment setting to evaluate the subjective gains provided by the tools, and decide on the minimal set of configurations that provide the best subjective quality improvements, so that the optimal performance can be achieved in the final standard in terms of implementation cost and visual quality.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “WD4: Working Draft 4 of High-Efficiency Video Coding,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-F803, Jul. 2011.

- O. G. Sezer and M. Budagavi, “Subjective Tests on ALF and SAO”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-F320, Jul., 2011.

- F. Bossen, “Common test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-F900, Jul. 2011.