| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, Switzerland, 21-30 Nov., 2011 | Document: JCTVC-G685 WG11 Number: m22253 |
| --- | --- |

| Title: | Selective Run-Level Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim and Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | Tel: +1-360-817-8487 asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

- In this contribution, a selective run-level coding (SRLC) technique for CAVLC is presented. The proposed technique improves the performance of CAVLC for high bit-rate applications by selectively disabling the run-level coding mode in CAVLC. It is asserted that the run-level mode requires the most computational complexity in CAVLC, so the method provides some benefit in reducing computational complexity. The primary advantage though is improved coding efficiency. For high bit-rate situations (i.e., not common conditions), it is reported that the average BD bitrate change is observed to be LD-LC: Y:-0.8%, U:-0.2%, V:-0.2%; RA-LC: Y:-0.6%, U:-0.5%, V:-0.6%. In addition, when the SRLC method is combined with fixed intra mapping mode (FIMM) proposed in JCTVC-G692, the average BD bitrate change is observed to be LD-LC: Y:-1.1%, U:-0.3%, V:-0.2%; RA-LC: Y:-0.9%, U:-0.5%, V:-0.6%.

# Introduction

The current run-level coding in CAVLC is an efficient coding method to compress consecutive zeros and the following significant coefficient together. Especially when the level of the following significant coefficient is equal to one (i.e. TrailingOne = =1), the level of the coefficient is to be skipped and only sign information is to be transmitted for the significant coefficient coding. However, current run-level coding method has been observed to be less efficient as the number of consecutive zeros and the occurrence of TrailingOne decrease. In high bitrate coding environment, the above mentioned cases are frequently occurred and a different coding technique is needed to deal with this issue.

Figure 1 Proposed SRLC Structure for CAVLC

# Selective Run-Level Coding

Selective run-level coding technique is introduced to improve coding performance in high bitrate coding by selectively employing the run-level coding mode. As shown in Figure 1, run-level flag is employed to specify whether to skip run-level mode right after encoding the last significant position and last coefficient level.

We have observed that the quantization parameter (QP) and resolution information are good indicators of ‘G’ which represents the corresponding category. This information can be determined at the decoder or calculated at the encoder and signaled to the decoder. Here, we proposed to calculate the information at the encoder and transmit the indicators in the bit-stream.

The detailed process can be described as follows;

If (QP<TH1): Q=3 (very high bitrates)

Else if (QP<TH2): Q=2 (high bitrates)

Else if (QP<TH3): Q=1(medium bitrates)

Else low bitrates: Q=0 (low bitrates), where (TH1<TH2<TH3)

Resolution information is used to classify the block into two cases (High/Low)

If (Current Image Width>=W): R=1 (High resolution)

Else (Low resolution)

G=2*Q+R (0<G<7)

Finally, 3 bits are to be sent to signal ‘G’ in slice header. As shown in Figure 2, run-level flag is determined based on block type, a block size and a trailing one (TR1). Here, TR1 represents whether or not the level of the last significant coefficient is equal to ‘1’

- Condition 1 (C1): if ((block size = =4 && TR1 = = 0) || block size ==8)

- Condition 2 (C2): if (block type = =4 && TR1= = 0)

- Condition 3 (C3): if (block size <16 && TR1 = =0)

- Condition 4 (C4): if (block size==4 && TR1==0)

Figure 2 Decision Tree for determining the run-level flag

# Results

We implemented the SRLC technology into the HM4.0 software and tested with the recommended configuration on all sequences. Table 1 below shows the summary results of SRLC method vs. HM4.0 anchor.

Table 1. Summary RD results of SRLC vs. HM4.0 Anchor (QP=7, 12, 17, 22)

|  | Low delay (B) LC |  |  |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  | Y | U | V |
| Class A |  |  |  |  | Class A | -0.2 | -0.2 | -0.3 |
| Class B | -0.5 | -0.3 | -0.3 |  | Class B | -0.4 | -0.4 | -0.5 |
| Class C | -1.1 | 0.2 | 0.2 |  | Class C | -0.8 | -0.4 | -0.5 |
| Class D | -1.4 | -0.4 | -0.4 |  | Class D | -1.2 | -0.9 | -0.9 |
| Class E | -0.1 | -0.1 | -0.1 |  | Class E |  |  |  |
| Overall | -0.8 | -0.2 | -0.2 |  | Overall | -0.6 | -0.5 | -0.6 |

The average BD bitrate change in high bitrate condition are reported to be LD-LC: Y:-0.8%, U:-0.2%, V:-0.2%; RA-LC: Y:-0.6%, U:-0.5%, V:-0.6%. Table 2 shows the summary results of combined method where SRLC and FIMM proposed in JCTVC-G692 are applied together. The average BD bitrate change in high bitrate condition is observed to be LD-LC: Y:-1.1%, U:-0.3%, V:-0.2%; RA-LC: Y:-0.9%, U:-0.5%, V:-0.6%.

Table 2. Summary RD results of Combined method vs. HM4.0 Anchor (QP=7, 12, 17, 22)

|  | Low delay (B) LC |  |  |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  | Y | U | V |
| Class A |  |  |  |  | Class A | -0.7 | -0.5 | -0.5 |
| Class B | -1.1 | -0.2 | -0.1 |  | Class B | -0.8 | -0.5 | -0.5 |
| Class C | -1.1 | 0.1 | 0.2 |  | Class C | -0.8 | -0.4 | -0.5 |
| Class D | -1.4 | -0.4 | -0.4 |  | Class D | -1.2 | -0.9 | -0.9 |
| Class E | -0.4 | -0.9 | -0.7 |  | Class E |  |  |  |
| Overall | -1.1 | -0.3 | -0.2 |  | Overall | -0.9 | -0.5 | -0.6 |

Table 3. Summary RD results of SRLC vs. HM4.0 Anchor (QP=22, 27, 32, 37)

|  | Low delay (B) LC |  |  |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  | Y | U | V |
| Class A |  |  |  |  | Class A | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 |  | Class B | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 |  | Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |  | Class D | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | -0.1 | -0.1 |  | Class E |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 |  | Overall | 0.0 | 0.0 | 0.0 |

- In the experiments, for the simplification, the TH1, TH2, TH3 are set to 16, 20, and 27, respectively. When it comes to resolution information, ‘W’ is also set to 1280 pixels.

# Conclusion

Introduced SRLC technique for CAVLC has been verified to improve the coding performance in high bitrates environment. It also provides a more advanced CAVLC coding structure for high bit-rate coding by selectively employing the run-level coding mode in CAVLC. We observe an average luminance coding gain of 0.6% and 0.8% for random access and low delay, respectively. In addition, when the SRLC method is combined with fixed intra mapping mode (FIMM) proposed in JCTVC-G692, the average BD bitrate change in luminance component is observed to be -1.1% and -0.9% for random access and low delay, respectively .

Given this level of performance and the importance of anticipating a wide variety of applications in the HEVC standard, we propose the JCT-VC adopt SRLC/Combined method into the HM.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCTVC-E700, “Common test conditions and software reference configurations,” 5th JCT-VC meeting, Geneva, Mar. 2011.