# Proposed Changes to Working Draft by adding

- An option of sending SRLC flag bits

- Run-level skip mode based on SRLC flag

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| else { |  |
| pic_order_cnt_lsb | u(v) |
| ref_pic_set_pps_flag | u(1) |
| if( ! ref_pic_set_pps_flag ) |  |
| ref_pic_set( num_ref_pic_sets ) |  |
| else |  |
| ref_pic_set_pps_idx | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } |  |
| if( !lightweight_slice_flag ) { |  |
| entropy_coding_mode_flag | u(1) |
| if(entropy_coding_mode_flag ==0 && QPY < TH) { |  |
| if(PicWidthInSamplesL < W) R =0; |  |
| else R=1; |  |
| if(QPY <Th_v) Q=3; |  |
| else if (QPY <Th_h) Q=2; |  |
| else if (QPY <Th_m) Q=1; |  |
| else Q=0; |  |
| SRLC_G = 2*Q + R |  |
| SRLC_G } | u(v) |
| } |  |

7.4.3 Slice header semantics

SRLC_G specifies the category information used for selective run-level coding mode. Each category requires different condition to determine whether or not to skip the run-level mode.

The followings are added to decoding process

9.2.4.2 Parsing process for SRLC_flag

Input to this process are bits from slice data.

Output of this process is run-level_skip flag.

The value of ‘run-level_skip_flag’ is derived as follow.

1. Obtain 3-bit value SRLC_flag according to subclause 7.3.3

2. Derive Q value and R value according to the following equations

Q = SRLC_flag/2 			                      (9-49)

R = SRLC_flag%2 			                      (9-50)

3. Based on Q and R, determine ‘run-level_skip_flag’ as follows

1. If Q is equal to 0, set run-level_skip_flag to 0

2. If Q is equal to 1, ‘run-level_skip_flag’ is determined based on R as

If (blSize = =4 && tr1= =0) flag=0

else flag=1

3. If Q is equal to 2, ‘run-level_skip_flag’ is determined based on R as

If(R= =0) {

if ((blSize = =4 && tr1= =0) || blSize = =8 ) flag=0;

else flag=1 }

else {

if( tr1= =0 && blSize<16) flag=0

else flag=1}

4. If Q is equal to 3, ‘run-level_skip_flag’ is determined based on R as

If(R= =0) {

if ((blSize = =4 && tr1= =0 ) || blSize = =8 ) flag=0

else flag=1}

else {

if(blockType= =4 && tr1= =0) flag=0

else flag=1}

NOTE- tr1 is determined based the level of the last significant coefficient.