| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G686 |
| --- | --- |

| Title: | CE13: Crosscheck report of JCTVC-G231 Test 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yunfei Zheng Xianglin Wang 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-651-9568 zhengy@qualcomm.com xianglin@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution verified the implementation and simulation results of CE13 JCTVC-G231 which is proposed by MediaTek.

# Introduction

During the 6th JCT-VC meeting, additional merge candidates are proposed to fill the empty position of merge candidate in order to solving the parsing issue. The additional merge candidates include combined bi-pred, scaled bi-pred, and zero candidates. In JCTVC-G231, some other additional merge candidates are proposed to further improve the coding performance.

# Proposed method

In CE13 JCTVC-G231 proposed to add 2 more additional candidates to replace the empty candidates in the list, which include:

1.  Truncating the MV of the first merge candidate to integer precision;

2.  Adding an offset (1 integer pixel) to the MV in the horizontal dimension of the first merge candidate.

Table 1 shows the comparison of the additional merge candidates in HM4.0 and those proposed JCTVC-G231.

Table 1 Additional merge candidates comparison

| Additional Candidates | HM4.0 | Proposed in F052 |
| --- | --- | --- |
| Cand 1 | Combined bi-pred | Combined bi-pred |
| Cand 2 | Scaled bi-pred | Scaled bi-pred |
| Cand 3 | Zero | Truncated Mv |
| Cand 4 |  | Offset Mv |
| Cand 5 |  | Zero |

# Simulation results

The software was released by MediaTek and was verified in this contribution. The proposd methods are implemented on top of HM4.0 with a merge mode decision bug fixed. The implementation is correct based our code review. It is tested under the common test condidation defined by [1]. Table 2 shows the coding performance and encoding/decoding time of the proposed method respectively. The anchor is the HM4.0 with the bug fix. The detailed results can be found in the attached excel sheet. The results are matching with that provided by MediaTek.

Table 2 Coding performance (BD rate and enc/dec time) comparison with HM4.0 with bug fix

|  | BD rate |  |  | Enc/Dec time |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec |
| RA_HE | -0.09% | -0.04% | 0.00% | 99% | 99% |
| LB_HE | -0.15% | -0.13% | -0.02% | 97% | 92% |
| LP_HE | -0.20% | -0.02% | -0.09% | 103% | 95% |
| RA_LC | -0.14% | -0.07% | -0.08% | 100% | 102% |
| LB_LC | -0.22% | -0.05% | 0.02% | 96% | 97% |
| LP_LC | -0.48% | -0.12% | -0.33% | 101% | 104% |

# Comments

As shown in Table 1, there are totally 5 categories of additional candidates. It is worth to investigate the performance of each of them in order to remove some redundant merge candidates.

The prunning process is still kept in this proposal, which need additional computations. It is also worth to investigate the performance improvement of prunning process when the new candidates are added.

# Conclusion

The implementation and coding performance of the proposal was verified. Based on the results and code analysis, comments are also provided to further improve and investigate the proposal.

# Reference

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, JCT-VC 6th Meeting: Torino, July, 2011.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).