| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G687 |
| --- | --- |

| Title: | Fast fractional motion search with adaptive searching point reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wan-Chi Siu+ Yan-Ho Kam+ Wai-Lam Hui+ Yui-Lam Chan+ Hoi-Kok Cheung+ Yu Liu* Jenny Yan Huo* | Tel: Email: | +852-27666229 enwcsiu@polyu.edu.hk +852-27666201 enyhkam@polyu.edu.hk |
| Source: | +The Hong Kong Polytechnic University *Hong Kong Applied Science and Technology Research Institute (ASTRI) |  |  |

_____________________________

# Abstract

This contribution proposes an additional option of the approach of performing fractional motion search, aiming at reducing the computational complexity of the encoder of the current HEVC reference model. According to the simulation results, the proposed technique achieves 6% reduction in computational complexity with a negligible 0.44% loss in luma BD-rate.

# Introduction

The fractional motion search in the HM4.0 encoder [1] refines the motion vector of a PU obtained through an integer motion search. This process comprises two stages: the first stage is the half-pixel search, and the second stage is the quarter-pixel search. In the half-pixel search, a motion vector is refined in the half-pixel precision. The relative positions and the indices of nine points involved in the half-pixel search are shown in Figure 1. The integer-pixel position (represented by the black circle in Figure 1) which is pointed by the unrefined motion vector and the surrounding eight half-pixel positions (represented by the white circles in Figure 1) are all tested in order to find out the point which results in a lowest cost value. In the quarter-pixel search, the motion vector is further refined in the quarter-pixel precision. The relative positions and the indices of nine points involved in the quarter-pixel search are shown in Figure 2. Similarly, the position (represented by the circle) which is pointed by the motion vector refined in the half-pixel search and the surrounding eight quarter-pixel positions (represented by the squares) are tested.

Figure 1 Relative positions and indices of 9 points involved in half-pixel search carried out by current HM encoder

Figure 2 Relative positions and indices of 9 points involved in quarter-pixel search carried out by current HM encoder

Since all the eight half-pixel points and eight quarter-pixel points are tested, the current fractional motion search in the HM encoder is most accurate at the expense of the processing speed. In this contribution, another approach of carrying out a fractional motion search agreeing with the current draft of the upcoming HEVC standard is proposed. The proposed approach performs motion searches in a step-by-step manner. Only a few adjoining points are involved in each step to continuously deduce the tendency towards the best searching point to be looked for. Since only the points along the searching path are tested, the complexity of the fractional motion search is reduced.

# Proposed Technique

The half-pixel search in this proposal comprises two steps. Instead of testing all eight half-pixel points like the original approach, the points with half-pixel indices 0, 1, 2, 3 and 4 are tested in step 1. After determining the tentative best point with the lowest cost value among these five points, some remaining half-pixel points are excluded from the search. At most two remaining half-pixel points which are adjacent to the tentative best point are tested in the next step. The indices of the new testing points in step 2, the indices of the excluded half-pixel points and the number of the excluded half-pixel points corresponding to each possible case are listed in Table 1.

Table 1 Indices of new testing points in step 2 of proposed half-pixel search, indices of excluded half-pixel points and number of excluded half-pixel points corresponding to each possibility of tentative best point in half-pixel search

| Index of tentative best point in half-pel search | Indices of new testing points in step 2 of half-pel search | Indices of excluded half-pel points | No. of excluded half-pel points |
| --- | --- | --- | --- |
| 0 |  | 5, 6, 7, 8 | 4 |
| 1 | 5, 6 | 7, 8 | 2 |
| 2 | 7, 8 | 5, 6 | 2 |
| 3 | 5, 7 | 6, 8 | 2 |
| 4 | 6, 8 | 5, 7 | 2 |

The quarter-pixel search in this proposal also comprises two steps. Rather than testing all eight quarter-pixel points like the original approach, the points required to be tested in step 1 depend on the selection of the tentative best point and the final best point in the half-pixel search. The indices of the testing points in step 1 of the quarter-pixel search, the indices of the excluded quarter-pixel points and the number of the excluded quarter-pixel points corresponding to each possible case are listed in Table 2. In step 2 of the quarter-pixel search, the points required to be tested depend on the selection of the tentative best point in the half-pixel search, the final best point in the half-pixel search and the tentative best point in the quarter-pixel search. The indices of the new testing points in step 2 of the quarter-pixel search, the indices of the further excluded quarter-pixel points and the number of the further excluded quarter-pixel points corresponding to each possible case are listed in Table 3.

Table 2 Indices of testing points in step 1 of proposed quarter-pixel search, indices of excluded quarter-pixel points and number of excluded quarter-pixel points corresponding to each possible case

| Index of tentative best point in half-pel search | Index of final best point in half-pel search | Indices of testing points in step 1 of quarter-pel search | Indices of excluded quarter-pel points | No. of excluded quarter-pel points |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0, 1, 2, 5, 6 |  | 0 |
| 1 | 1 | 0, 2, 5, 6 | 1, 3, 4 | 3 |
| 1 | 5 | 0, 6 | 1, 2, 3, 4, 5, 7 | 6 |
| 1 | 6 | 0, 5 | 1, 2, 3, 4, 6, 8 | 6 |
| 2 | 2 | 0, 1, 5, 6 | 2, 7, 8 | 3 |
| 2 | 7 | 0, 6 | 1, 2, 3, 5, 7, 8 | 6 |
| 2 | 8 | 0, 5 | 1, 2, 4, 6, 7, 8 | 6 |
| 3 | 3 | 0, 6, 1, 2 | 3, 5, 7 | 3 |
| 3 | 5 | 0, 2 | 1, 3, 4, 5, 6, 7 | 6 |
| 3 | 7 | 0, 1 | 2, 3, 5, 6, 7, 8 | 6 |
| 4 | 4 | 0, 5, 1, 2 | 4, 6, 8 | 3 |
| 4 | 6 | 0, 2 | 1, 3, 4, 5, 6, 8 | 6 |
| 4 | 8 | 0, 1 | 2, 4, 5, 6, 7, 8 | 6 |

Table 3 Indices of new testing points in step 2 of proposed quarter-pixel search, indices of further excluded quarter-pixel points and number of further excluded quarter-pixel points corresponding to each possible case

| Index of tentative best point in half-pel search | Index of final best point in half-pel search | Index of tentative best point in quarter-pel search | Indices of new testing points in step 2 of quarter-pel search | Indices of further excluded quarter-pel points | No. of further excluded quarter-pel points |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 |  | 3, 4, 7, 8 | 4 |
| 0 | 0 | 1 | 3, 4 | 7, 8 | 2 |
| 0 | 0 | 2 | 7, 8 | 3, 4 | 2 |
| 0 | 0 | 5 | 3, 7 | 4, 8 | 2 |
| 0 | 0 | 6 | 4, 8 | 3, 7 | 2 |
| 1 | 1 | 0 |  | 7, 8 | 2 |
| 1 | 1 | 2 | 7, 8 |  | 0 |
| 1 | 1 | 5 | 7 | 8 | 1 |
| 1 | 1 | 6 | 8 | 7 | 1 |
| 1 | 5 | 0 |  | 8 | 1 |
| 1 | 5 | 6 | 8 |  | 0 |
| 1 | 6 | 0 |  | 7 | 1 |
| 1 | 6 | 5 | 7 |  | 0 |
| 2 | 2 | 0 |  | 3, 4 | 2 |
| 2 | 2 | 1 | 3, 4 |  | 0 |
| 2 | 2 | 5 | 3 | 4 | 1 |
| 2 | 2 | 6 | 4 | 3 | 1 |
| 2 | 7 | 0 |  | 4 | 1 |
| 2 | 7 | 6 | 4 |  | 0 |
| 2 | 8 | 0 |  | 3 | 1 |
| 2 | 8 | 5 | 3 |  | 0 |
| 3 | 3 | 0 |  | 4, 8 | 2 |
| 3 | 3 | 6 | 4, 8 |  | 0 |
| 3 | 3 | 1 | 4 | 8 | 1 |
| 3 | 3 | 2 | 8 | 4 | 1 |
| 3 | 5 | 0 |  | 8 | 1 |
| 3 | 5 | 2 | 8 |  | 0 |
| 3 | 7 | 0 |  | 4 | 1 |
| 3 | 7 | 1 | 4 |  | 0 |
| 4 | 4 | 0 |  | 3, 7 | 2 |
| 4 | 4 | 5 | 3, 7 |  | 0 |
| 4 | 4 | 1 | 3 | 7 | 1 |
| 4 | 4 | 2 | 7 | 3 | 1 |
| 4 | 6 | 0 |  | 7 | 1 |
| 4 | 6 | 2 | 7 |  | 0 |
| 4 | 8 | 0 |  | 3 | 1 |
| 4 | 8 | 1 | 3 |  | 0 |

# Simulation Results

The proposed technique was implemented in the HM4.0 encoder [1]. Experiments with the setting suggested in JCTVC-E700 [2] were conducted to evaluate the rate-distortion performance and the coding speed of the proposed technique. The experimental results comparing the performance of the proposed technique with that of the HM4.0 anchor are listed in Table 4. Detailed results can be found in the attachment, JCTVC-G-result.xls.

Table 4 Comparison of the performance between the proposed technique and the HM4.0 anchor in terms of the changes in BD-rate and coding time

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.33 | 0.10 | -0.06 | 0.38 | 0.48 | 0.38 |
| Class B | 0.28 | 0.21 | 0.23 | 0.28 | 0.14 | 0.26 |
| Class C | 0.46 | 0.35 | 0.36 | 0.47 | 0.35 | 0.26 |
| Class D | 0.56 | 0.41 | 0.34 | 0.53 | 0.25 | 0.50 |
| Class E |  |  |  |  |  |  |
| All | 0.40 | 0.26 | 0.22 | 0.41 | 0.29 | 0.35 |
| Enc Time[%] | 94% |  |  | 94% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.22 | 0.11 | 0.24 | 0.24 | 0.21 | 0.27 |
| Class C | 0.56 | 0.32 | 0.39 | 0.38 | 0.38 | 0.05 |
| Class D | 0.49 | 0.40 | 0.44 | 0.51 | 0.61 | 0.56 |
| Class E | 0.36 | 0.44 | 0.74 | 0.23 | 0.24 | 0.29 |
| All | 0.40 | 0.30 | 0.42 | 0.34 | 0.36 | 0.29 |
| Enc Time[%] | 93% |  |  | 93% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |

|  | Low delay P |  |  | Low delay LoCo P |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.32 | 0.28 | 0.07 | 0.38 | 0.34 | 0.06 |
| Class C | 0.52 | 0.27 | 0.53 | 0.58 | 0.44 | 0.42 |
| Class D | 0.71 | 0.72 | 0.02 | 0.68 | 0.52 | 0.12 |
| Class E | 0.57 | 0.38 | 0.19 | 0.55 | 0.34 | -0.06 |
| All | 0.52 | 0.40 | 0.20 | 0.54 | 0.41 | 0.14 |
| Enc Time[%] | 94% |  |  | 94% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

# Conclusion

A fast approach of the fractional motion search in the HM encoder which adaptively reduces the number of searching points according to the deduced tendency of the best point is proposed. The computational complexity of the HM4.0 encoder is reduced by 6% by applying the proposed technique, while the loss in Y BD-rate is about 0.44%, which is negligible. The proposed technique is found to be useful for increasing the processing speed of the HM software.

# References

- http://hevc.kw.bbc.co.uk/trac/browser/jctvc-hm/tags/HM-4.0

- Frank Bossen, “Common test conditions and software reference configurations,” JCT-VC document, JCTVC-E700, Jan. 2011

# Patent rights declaration(s)

The Hong Kong Polytechnic University does not have any current or pending patent rights relating to the technology described in this contribution.