| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G688 |
| --- | --- |

| Title: | CE09: Crosscheck report of JCTVC-G052 test MRG_MVD03 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yunfei Zheng Xianglin Wang 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-651-9568 zhengy@qualcomm.com xianglin@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document reports results on cross-checking the implementation and simulation results of CE09 JCTVC-G052 test MRG_MVD03 which is proposed by JVC.

# Introduction

During the 6th JCT-VC meeting, an additional merge mode that was named merge mvd mode was proposed to improve coding efficiency. The motion vector derivation process for merge mvd mode is shared with existing merge mode and one mvd is transmitted, which construct a bi-predictive mode with less cost of the original bi-predictive mode.

# Proposed method

In merge mvd mode, bi-predictive motion compensation is always used based on merge parameter in merge mode. The syntax element mvd_ls is transmitted as mvd for merge mvd mode according to the condition of merge parameter in merge mode.

If inter_pred_mode in merge parameter is uni-prediction, the reference index of the other list which is not selected by merge parameter is set to 0 for bi-prediction in merge_mvd mode. The reference picture on which mvd_ls is added to mvp is the picture indicated by the reference index in the other list.

Otherwise if inter_pred_mode in merge parameter is bi-prediction, same reference picture as for the merge mode is used for the merge mvd mode. The reference picture on which mvd_ls is added to mvp is selected based on the distance to the current picture.

The detailed relationship between merge parameter in merge mode and parameters of motion compensation in merge mvd mode is presented in the following table.

| parameters of motion compensation in merge mvd mode | inter pred mode in merge parameter |  |  |  |
| --- | --- | --- | --- | --- |
|  | Uni-pred(L0) | Uni-pred(L1) | Bi-pred |  |
|  |  |  | abs(poc_ref_l0-cur_poc) <=abs(poc_ref_l1-cur_poc) | abs(poc_ref_l0-cur_poc) >abs(poc_ref_l1-cur_poc) |
| ref_idx_l0 | ref_idx_l0 in merge parameter | 0 | ref_idx_l0 in merge parameter | ref_idx_l0 in merge parameter |
| ref_idx_l1 | 0 | ref_idx_l1 in merge parameter | ref_idx_l1 in merge parameter | ref_idx_l1 in merge parameter |
| mvp_l0 | mvp_l0 in merge parameter | scaled mvp_l1 | mvp_l0 in merge parameter | mvp_l0 in merge parameter |
| mvp_l1 | scaled mvp_l0 | mvp_l1 in merge parameter | mvp_l1 in merge parameter | mvp_l1 in merge parameter |
| mvd_l0 | not transmitted | mvd_ls (transmitted) | not transmitted | mvd_ls (transmitted) |
| mvd_l1 | mvd_ls (transmitted) | not transmitted | mvd_ls (transmitted) | not transmitted |
| mv_l0 | mvp_l0 | mvp_l0 + mvd_l0 | mvp_l0 | mvp_l0 + mvd_l0 |
| mv_l1 | mvp_l1 + mvd_l1 | mvp_l1 | mvp_l1 + mvd_l1 | mvp_l1 |

The merge mvd mode is signaled by merge_mvd_flag that is inserted just after inter_pred_flag. CABAC context model for merge_mvd_flag is the same as for merge_flag. Merge_mvd_flag for CAVLC is transmitted when ref_idx_l0 and ref_idx_l1 are equal to 0.

For use of merge mvd mode to flexible encoding environment, three configurations will be tested. In ADD_MRG01-02, merge_idx is transmitted just after merge mvd flag. The number of the candidates is the same as merge mode. The decoding specification is identical in ADD_MRG01-02. The difference between ADD_MRG01 and ADD_MRG 02 is whether to use additional motion estimation in encoder. ADD_MRG01 uses no additional motion estimation. ADD_MRG02 uses additional motion estimation. In ADD_MRG01, mvd_ls is determined by just calculating difference for the motion vector that has been detected for AMVP mode. Motion estimation for merge mvd mode is executed as bi-pred ME for small search area. Initial search position is based on motion vector that has been calculated for AMVP mode with motion estimation in the HM4.0.

Furthermore, it will be tested not to transmit merge_idx for the merge mvd mode in ADD_MRG03. The merge mvd mode can be used only for one merge candidate indicated by merge_idx equal to 0 in the case of merge mode. merge_mvd_mode is inferred to be equal to 0.

| Test | Description | Related Doc. | Tester1 | Tester2 |
| --- | --- | --- | --- | --- |
| MRG_MVD01 | five candidates No additional ME | JCTVC-F373 | JVC Kenwood | Fujitsu |
| MRG_MVD02 | five candidates Additional ME | JCTVC-F373 | JVC Kenwood | Panasonic |
| MRG_MVD03 | one candidate (merge_idx = 0) Additional ME | JCTVC-F373 | JVC Kenwood | Qualcomm |

# Simulation results

The software was released by JVC and was verified in this contribution. The implementation is correct based our code review. It is tested under the common test conditions defined by [1]. Table 1 shows the coding performance and encoding/decoding time of the proposed method respectively. The detailed results can be found in the attached excel sheet. The results are matching with that provided by JVC.

Table 1 Coding performance (BD rate and enc/dec time) comparison with HM4.0

|  | BD rate |  |  | Enc/Dec time |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec |
| RA_HE | -0.4% | -0.5% | -0.5% | 110% | 100% |
| LB_HE | -0.7% | -0.6% | -0.6% | 106% | 101% |
| RA_LC | -0.4% | -0.3% | -0.4% | 112% | 99% |
| LB_LC | -0.7% | -0.3% | -0.4% | 108% | 100% |

# Comments

Based on our code review, we found the inter_dir is always transmitted to decoder. Actually it can be inferred from the information indicated by the merge index, which may improve the coding performance.

Based on the simulation results of the three tests, it is easy to find that the gain drops dramatically if the motion estimation is skipped. It seems that most of the gain comes from better motion estimation. The remaining gain may come from additional mode decision. Since in HM4.0, the motion estimation and inter mode decision is not using true RD cost and far from optimal, additional mode decision or better motion estimation with the similar encoding complexity increasing as the proposed mode can usually improve the coding performance. So the efficiency of this new proposed mode still needs to be further investigated.

# Conclusion

The implementation and coding performance of the proposal was verified. Based on the results and code analysis, comments are also provided to further improve and investigate the proposal.

# Reference

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, JCT-VC 6th Meeting: Torino, July, 2011.