| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G690 WG11 number: m22258 |
| --- | --- |

| Title: | Non-CE9: Some possible motion vector coding related simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yih Han Tan Chuohao Yeo Zhengguo Li 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 yhtan@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution presents three modifications that are aimed at simplifying motion vector coding. The first modification is related to the derivation of the L1 motion vector predictor (MVP) during bi-directional prediction. L0 and L1 MVP candidates are derived pair-wise such that both sets of candidates are derived from the same neighboring PU. This reportedly led to a coding loss of 0.0% and 0.1% BD-rate loss for RA and LB configurations respectively. The second modification restricts the reference index to zero during the derivation of the reference index of the temporal Merge candidate. This reportedly led to a coding loss of 0.0% and 0.1% BD-rate loss for RA and LB configurations respectively.  The third proposed modification is to remove the context for coding the MVP index when CABAC is the entropy coder, which reportedly resulted in an average of 0.0% BD-Rate loss.

# Introduction

This contribution presents three modifications aimed at simplifying motion vector coding. In HM4 [1], the motion vector predictor (MVP) candidates for the L0 and L1 directions are derived independently for bi-directional prediction. The first proposed modification derives the L0 and L1 MVP candidates pair-wise such that both predictors are always derived from the same neighboring PU. This modification aims to reduce the operations required for the derivation of the L1 MVP.

In the current design of HM4, the reference index of temporal Merge candidate is inferred from the reference indices of neighboring PUs. The second modification fixes the reference index of the temporal Merge candidate to zero. This reduces the number of operations required for deriving the predictor.

The third modification removes the context for coding the MVP index when CABAC is the entropy coder; in other words, the MVP index is coded in bypass mode.

# Proposed modifications

## Reducing operations during the derivation of L1 MVP during bi-prediction

In HM4, the MVP candidates for L0 and L1 directions are derived independently for bi-directional prediction. In the proposed approach, the L0 and L1 MVPs are derived simultaneously during bi-directional prediction. The candidate derivation and pruning process is first carried out for the L0 MVP as described in HM4.

After determining the MVP for the L0 direction given the signaled MVP index, the MVP for the L1 direction is derived from L1 motion vector of the same PU where the L0 MVP is derived. Depending on the reference index of the derived L1 predictor and the signaled L1 reference of the current PU, the motion vector predictor is scaled accordingly.

If a L1 MVP is not available at the position indicated by the decoded MVP index, e.g., the neighboring PU only has a L0 MV, the L0 MVP is scaled accordingly to obtain the L1 MVP.

In the proposed scheme, only one MVP index is sent for the decoding of a pair of L0 and L1 MVPs.

## Derivation of the reference index of temporal merge candidate

In HM4, the reference index of the temporal merge candidate is derived by checking the reference indices of neighboring PUs. In the proposed approach, the reference indices of temporal merge candidates are set to zero.

## Removing context for motion vector predictor index

In the design of HM4, up to two MVP candidates are derived. The MVP used for coding a motion vector is signaled by an MVP index.

When CABAC is used as the entropy coder, we propose to remove the context used to code the MVP index. In other words, this syntax element is coded in bypass mode (assuming an equal probability distribution), eliminating one context model and initialization entry.

# Experimental results

The implementation is on HM 4.0. Tests were conducted on the High-Efficiency and Low-Complexity settings of the Random Access and Low Delay configurations as specified in the common test conditions [2]. The results are shown in the table below.

## Reducing operations during the derivation of L1 MVP during bi-prediction

The tables below show the performance of the proposed bi-directional motion vector coding method when applied to HM4.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | 0.1% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.1% | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | 0.1% | 0.2% | 0.2% | 0.3% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | -0.1% |
| Class D | 0.1% | 0.5% | 0.4% | 0.1% | 0.6% | 0.0% |
| Class E | 0.1% | -0.4% | 0.9% | 0.0% | 0.1% | 0.1% |
| Overall | 0.1% | 0.1% | 0.3% | 0.1% | 0.3% | 0.1% |
|  | 0.1% | 0.1% | 0.3% | 0.1% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

From the results, it can be observed that the proposed method has no significant impact on coding performance in the Random Access configurations (~0.0%) and impacts coding performance in the Low Delay configurations by ~0.1%.

## Derivation of the reference index of temporal merge candidate

Next, we show the effect of setting the reference indices of temporal merge candidates to zero in HM4:

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.3% | 0.6% | 0.1% | 0.7% | 0.8% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.4% | 0.3% |
| Class D | 0.3% | 0.8% | 0.4% | 0.2% | 0.8% | 0.3% |
| Class E | 0.0% | 0.1% | 0.5% | 0.1% | 1.0% | 0.6% |
| Overall | 0.1% | 0.3% | 0.4% | 0.1% | 0.7% | 0.5% |
|  | 0.1% | 0.3% | 0.4% | 0.1% | 0.7% | 0.6% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |

From the results, it can be observed that the proposed method has no significant impact on coding performance in the Random Access configurations (~0.0%) and impacts coding performance in the Low Delay configurations by ~0.1%.

## Removing context for motion vector predictor index

The table below shows the results for coding MVP index in bypass mode, i.e., removing the context for MVP index.

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% |
| Class D | 0.1% | 0.0% | -0.1% |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 97% |  |  |

|  | Low delay B HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.5% | -0.1% |
| Class E | 0.1% | 0.0% | 0.9% |
| Overall | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 96% |  |  |

From the results, no significant coding loss is observed.

# Conclusions

In this contribution, we have presented several possible methods of simplifying motion coding. Coding simulations on HM4 shows no significant coding loss. We recommend adopting the modifications into the next HEVC test model.

# Reference

[1] B. Bross et al., “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCTVC-F803, Torino, Italy, Jul. 2011.

[2] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-F900, Torino, Italy, Jul. 2011.

# Patent rights declaration(s)

Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).