| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G690 WG11 number: m22258 |
| --- | --- |

| Title: | Non-CE9: Some possible motion vector coding related simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yih Han Tan Chuohao Yeo Zhengguo Li 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 yhtan@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution presents several modifications that are aimed at simplifying motion vector coding. The first modification is related to the derivation of the L1 motion vector predictor (MVP) during bi-directional prediction. L0 and L1 MVP candidates are derived pair-wise such that both sets of candidates are derived from the same neighboring PU. This reportedly led to a coding loss of 0.0% and 0.1% BD-rate loss for RA and LB configurations respectively. The second modification restricts the reference index to zero during the derivation of the reference index of the temporal Merge candidate. This reportedly led to a coding loss of 0.0% and 0.1% BD-rate loss for RA and LB configurations respectively.  The third proposed modification is to remove the context for coding the MVP index when CABAC is the entropy coder, which reportedly resulted in an average of 0.0% BD-Rate loss. The last two modifications are related to the addition and pruning process of ‘combined’ Merge candidates. The modified process limits the number of combined candidates to five and uses a simplified pruning process. When applied together, these two modifications result in an average of 0.0% BD-Rate loss in all configurations.

# Introduction

This contribution presents three modifications aimed at simplifying motion vector coding. In HM4 [1], the motion vector predictor (MVP) candidates for the L0 and L1 directions are derived independently for bi-directional prediction. The first proposed modification derives the L0 and L1 MVP candidates pair-wise such that both predictors are always derived from the same neighboring PU. This modification aims to reduce the operations required for the derivation of the L1 MVP.

In the current design of HM4, the reference index of temporal Merge candidate is inferred from the reference indices of neighboring PUs. The second modification fixes the reference index of the temporal Merge candidate to zero. This reduces the number of operations required for deriving the predictor.

The third modification removes the context for coding the MVP index when CABAC is the entropy coder; in other words, the MVP index is coded in bypass mode.

The last two modifications are related to the addition and pruning process of ‘combined’ Merge candidates, limiting the number of combined candidates to five and using a simplified pruning process.

# Proposed modifications

## Reducing operations during the derivation of L1 MVP during bi-prediction

In HM4, the MVP candidates for L0 and L1 directions are derived independently for bi-directional prediction. In the proposed approach, the L0 and L1 MVPs are derived simultaneously during bi-directional prediction. The candidate derivation and pruning process is first carried out for the L0 MVP as described in HM4.

After determining the MVP for the L0 direction given the signaled MVP index, the MVP for the L1 direction is derived from L1 motion vector of the same PU where the L0 MVP is derived. Depending on the reference index of the derived L1 predictor and the signaled L1 reference of the current PU, the motion vector predictor is scaled accordingly.

If a L1 MVP is not available at the position indicated by the decoded MVP index, e.g., the neighboring PU only has a L0 MV, the L0 MVP is scaled accordingly to obtain the L1 MVP.

In the proposed scheme, only one MVP index is sent for the decoding of a pair of L0 and L1 MVPs.

## Derivation of the reference index of temporal merge candidate

In HM4, the reference index of the temporal merge candidate is derived by checking the reference indices of neighboring PUs. In the proposed approach, the reference indices of temporal merge candidates are set to zero.

## Removing context for motion vector predictor index

In the design of HM4, up to two MVP candidates are derived. The MVP used for coding a motion vector is signaled by an MVP index.

When CABAC is used as the entropy coder, we propose to remove the context used to code the MVP index. In other words, this syntax element is coded in bypass mode (assuming an equal probability distribution), eliminating one context model and initialization entry.

## Limiting the number of ‘combined’ merge candidates

In the current design of HM4, after the first round of candidate addition, ‘combined’, ‘non-scaled’ and ‘zero’ Merge candidates can be added to the list if the number of candidates is less than five.

In experiments done in CE13, it was found that the total number of pruning during the second round of candidate addition can be limited to five without a coding performance drop. During the addition of combined candidates, however, up to twelve combinations may have to be constructed.

In this part of the proposal, different limits are placed on the maximum number of combinations constructed.  It was found that the maximum can be reduced from twelve to five without coding performance drop.

## Simplified pruning during addition of ‘combined’ merge candidates

Experiments were done in CE13 to limit the number of pruning required during the second round of candidate addition. In this part of the proposal, a simplified checking process on each combination is carried out in place of the current pruning process. Consider the case where a combination is to be constructed from the pair of bi-directional motion vector, MVa and MVb . MVa(0) and MVa(1) is the L0 and L1 motion vector of MVa respectively and refa(0) and refa(1) is reference index of MVa(0) and MVa(1) respectively. The combination involving MVa and MVb is not allowed if

- MVa(0)== MVb(0) && refa(0)== refb(0) || MVa(1)== MVb(1) && refa(1)== refb(1).

That is, a combination is allowed only if both L0 and L1 components of the two potential combination candidates are different. With the proposed check, the O(n) duplicate check process after the addition of each potential combined candidate can be removed.

# Experimental results

The implementation is on HM 4.0. Tests were conducted on the High-Efficiency and Low-Complexity settings of the Random Access and Low Delay configurations as specified in the common test conditions [2]. The results are shown in the table below.

## Reducing operations during the derivation of L1 MVP during bi-prediction

The tables below show the performance of the proposed bi-directional motion vector coding method when applied to HM4.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | 0.1% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.1% | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class D | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | 0.1% | 0.2% | 0.2% | 0.3% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | -0.1% |
| Class D | 0.1% | 0.5% | 0.4% | 0.1% | 0.6% | 0.0% |
| Class E | 0.1% | -0.4% | 0.9% | 0.0% | 0.1% | 0.1% |
| Overall | 0.1% | 0.1% | 0.3% | 0.1% | 0.3% | 0.1% |
|  | 0.1% | 0.1% | 0.3% | 0.1% | 0.2% | 0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

From the results, it can be observed that the proposed method has no significant impact on coding performance in the Random Access configurations (~0.0%) and impacts coding performance in the Low Delay configurations by ~0.1%.

## Derivation of the reference index of temporal merge candidate

Next, we show the effect of setting the reference indices of temporal merge candidates to zero in HM4:

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.3% | 0.6% | 0.1% | 0.7% | 0.8% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.4% | 0.3% |
| Class D | 0.3% | 0.8% | 0.4% | 0.2% | 0.8% | 0.3% |
| Class E | 0.0% | 0.1% | 0.5% | 0.1% | 1.0% | 0.6% |
| Overall | 0.1% | 0.3% | 0.4% | 0.1% | 0.7% | 0.5% |
|  | 0.1% | 0.3% | 0.4% | 0.1% | 0.7% | 0.6% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |

From the results, it can be observed that the proposed method has no significant impact on coding performance in the Random Access configurations (~0.0%) and impacts coding performance in the Low Delay configurations by ~0.1%.

## Removing context for motion vector predictor index

The table below shows the results for coding MVP index in bypass mode, i.e., removing the context for MVP index.

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% |
| Class D | 0.1% | 0.0% | -0.1% |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 97% |  |  |

|  | Low delay B HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.5% | -0.1% |
| Class E | 0.1% | 0.0% | 0.9% |
| Overall | 0.0% | 0.1% | 0.1% |
|  | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 96% |  |  |

From the results, no significant coding loss is observed.

## Limiting the number of ‘combined’ merge candidates

The tables below show the results when the maximum number of combined candidates is limited to 2.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |
| Class C | 0.1% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.4% | 0.3% | 0.4% | 0.2% | 0.5% |
| Class C | 0.3% | 0.3% | 0.1% | 0.3% | 0.4% | 0.3% |
| Class D | 0.3% | 0.1% | 0.1% | 0.3% | 0.2% | 0.4% |
| Class E | 0.2% | 0.8% | 1.1% | 0.4% | 0.4% | 0.2% |
| Overall | 0.2% | 0.4% | 0.3% | 0.3% | 0.3% | 0.3% |
|  | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% | 0.3% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 96% |  |  | 95% |  |  |

The tables below show the results when the maximum number of combined candidates is limited to 5.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | 0.0% | 0.1% | 0.2% | 0.0% |
| Class C | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.1% |
| Class E | 0.0% | -0.5% | 0.4% | 0.0% | -0.1% | -0.1% |
| Overall | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |

From the results, no significant coding loss is observed when the maximum number of combinations is limited to 5.

## Simplified pruning during addition of ‘combined’ merge candidates

The table below shows the results when the simplified pruning process is used during the addition of combined Merge candidates.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.2% | -0.3% | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | -0.1% |
| Class D | 0.0% | 0.1% | 0.2% | 0.1% | 0.0% | 0.2% |
| Class E | 0.0% | 0.6% | 0.3% | 0.1% | 0.4% | -0.1% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  |

From the results, no significant coding loss is observed when the simplified pruning process is used.

The table below shows the results when the simplified pruning process is used during the addition of combined Merge candidates in conjunction with the proposed modification in section 2.4.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.1% | 0.2% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 101% |  |  |

|  | Low delay B HE |  |  | Low delay B LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | -0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| Class C | 0.0% | 0.2% | 0.1% | 0.1% | 0.1% | -0.2% |
| Class D | 0.0% | 0.5% | 0.1% | 0.1% | 0.1% | 0.2% |
| Class E | 0.1% | 0.4% | 0.7% | 0.0% | 0.5% | -0.3% |
| Overall | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.0% |
|  | 0.0% | 0.2% | 0.2% | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  |

Again, no significant coding loss is observed.

- Conclusions

In this contribution, we have presented several possible methods of simplifying motion coding. Coding simulations on HM4 shows no significant coding loss. We recommend adopting the modifications into the next HEVC test model.

# Reference

[1] B. Bross et al., “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCTVC-F803, Torino, Italy, Jul. 2011.

[2] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-F900, Torino, Italy, Jul. 2011.

# Patent rights declaration(s)

Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix – Proposed Text

##### 8.4.2.1.4 Derivation process for combined bi-predictive merging candidates

Inputs of this process are

- a merging candidate list mergeCandList,

- reference indices refIdxL0N and refIdxL1N of every candidate N being in mergeCandList,

- prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N being in mergeCandList,

- motion vectors mvL0N and mvL1N of every candidate N being in mergeCandList,

- the number of elements numMergeCand within mergeCandList,

- the number of elements numOrigMergeCand within the mergeCandList after the spatial and temporal merge candidate derivation process,

- the maximum number of elements maxNumMergeCand within the mergeCandList.

Outputs of this process are

- the merging candidate list mergeCandList,

- the number of elements numMergeCand within mergeCandList.

- reference indices refIdxL0combCandk and refIdxL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

- prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

- motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added in mergeCandList during the invokation of this process,

The function RefPicOrderCnt( refidx, LX ) specifies the value of PicOrderCnt of the picture that is the reference picture RefPicListX[ refidx ] of the current picture with X being 0 or 1. PicOrderCnt of the reference picture shall be maintained until the picture is marked as “non-existing”. The variable PicOrderCnt specifies the PicOrderCnt of the current picture.

[Ed. (BB): PicOrderCnt(.) needs to be defined. Currently the Decoding process for picture order count is not described]

When numOrigMergeCand is greater than 1 and less than maxNumMergeCand, the variable numInputMergeCand is set to numMergeCand, the variables combIdx and combCnt are set to 0, the variable combStop is set to false and the following steps are repeated until combStop is equal to true.

- The variables l0CandIdx and l1CandIdx are derived using combIdx as specified in Table 8-1.

- The following assignments are made with l0Cand being the candidate at position l0CandIdx and l1Cand being the candidate at position l1CandIdx in the merging candidate list mergeCandList ( l0Cand = mergeCandList[ l0CandIdx ] , l1Cand = mergeCandList[ l1CandIdx ] ).

- When all of the following conditions are true,

    - predFlagL0l0Cand = = 1

    - predFlagL1l1Cand = = 1

    - ! ( refIdxL0l0Cand = = refIdxL0l1Cand && L0l0Cand = = L0l1Cand | |

- refIdxL1l0Cand = = refIdxL1l1Cand && L1l0Cand = = L1l1Cand )

    - RefPicOrderCnt( refIdxL0l0Cand, L0 ) != RefPicOrderCnt( refIdxL1l1Cand, L1 ) | | mvL0l0Cand != mvL1l1Cand

the following applies.

    - When no candidate in mergeCandList has the same list0 motion vector and reference index as l0Cand and the same list1 motion vector and reference index as l1Cand, the The candidate combCandk with k equal to ( numMergeCand − numInputMergeCand ) is added at the end of mergeCandList ( mergeCandList[ numMergeCand ] = combCandk ) and the reference indices, the prediction list utilization flags and the motion vectors of combCandk are dervied as follows and numMergeCand is incremented by 1.

refIdxL0combCandk = refIdxL0l0Cand     (8-1)

refIdxL1combCandk = refIdxL1l1Cand     (8-2)

predFlagL0combCandk = 1                         (8-3)

predFlagL1combCandk = 1                         (8-4)

mvL0combCandk[ 0 ] = mvL0l0Cand[ 0 ]  (8-5)

mvL0combCandk[ 1 ] = mvL0l0Cand[ 1 ]  (8-6)

mvL1combCandk[ 0 ] = mvL1l1Cand[ 0 ]  (8-7)

mvL1combCandk[ 1 ] = mvL1l1Cand[ 1 ]  (8-8)

numMergeCand = numMergeCand + 1      (8-9)

    - The variable combCnt is incremented by 1.

- The variable combIdx is incremented by 1.

- When combIdx is equal to 5 ( numOrigMergeCand * ( numOrigMergeCand − 1 ) ) or numMergeCand is equal to maxNumMergeCand or combCnt is equal to 5, combStop is set to true.

Table 8-1 – Specification of l0CandIdx and l1CandIdx

| combIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| l0CandIdx | 0 | 1 | 0 | 2 | 1 | 2 | 0 | 3 | 1 | 3 | 2 | 3 |
| l1CandIdx | 1 | 0 | 2 | 0 | 2 | 1 | 3 | 0 | 3 | 1 | 3 | 2 |