| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G691 |
| --- | --- |

| Title: | Non-CE8: Combination of CE8.a.1 and CE8.a.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 Takashi Watanabe1 Takeshi Chujoh1 Ching-Yeh Chen2 Chih-Ming Fu2 Chia-Yang Tsai2 Yu-Wen Huang2 Shawmin Lei2 Marta Karczewicz3 In Suk Chong3 Tomonori Ikai4 Andrew Segall4 Tomoyuki Yamamoto4 Yukinobu Yasugi 4 | Email: | tomoo.yamakage@toshiba.co.jp takashi39.watanabe@toshiba.co.jp takeshi.chujoh@toshiba.co.jp chingyeh.chen@mediatek.com chihming.fu@mediatek.com chiayang.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com martak@qualcomm.com ichong@qualcomm.com ikai.tomohiro@sharp.co.jp asegall@sharplabs.com yamamoto.tomoyuki@sharp.co.jp yasugi.yukinobu@sharp.co.jp |
| Source: | TOSHIBA Corporation1, MediaTek Inc.2, Qualcomm3 and Sharp4 |  |  |

_____________________________

# Abstract

This evaluates a combination of CE8.a.1 and CE8.a.2. The coding efficiency is improved by is 0.2 %, 0.3 %, 0.3% and 0.4 % in HE-AI, RA, LB, and LP.

#### Introduction

In this meeting, we have two proposals in CE8 subset a (i.e., Tool 1 and Tool 2), which are inherenently orthogonal. So we provide combined results of those two proposals.

# Description

## 2D merge

The proposed “2-D mergeable syntax” encodes filter index of each class directly. Figure 2 shows an example of indexes. The classes sharing a same index are regarded as a group (or merged redgion) where corresponding coefficients set is sent.

Figure 2: 2D Flexible class representation

## BA mode computations

- BA mode computation with 8 points

This BA mode computation requires to access pixels of 6x4 windows with 1.25 additions/pixel.

## Filter shape dependent BA mode computations

Depending on the filter shapes, BA mode direction computation can be changed. Cross shape inherently follows vertical/horizontal (BA)  direction. And star shape can follow either vertical/horizontal or diagonal direction. Therefore we tied filter shapes with directionality.

# Changes in Working Draft Text

- 7.3.3.5	Adaptive loop filter parameter syntax

| alf_param() { | C | Descriptor |
| --- | --- | --- |
| adaptive_loop_filter_flag | 2 | u(1) |
| if ( adaptive_loop_filter_flag ) { |  |  |
| alf_region_adaptation_flag | 2 | u(v) |
| alf_filter_shape | 2 | u(1) |
| alf_no_filters_minus1 | 2 | ue(v) |
| for (i=1; i < AlfNumRegions; i++) { |  |  |
| if (alf_no_filters_minus1 == 1) { |  |  |
| alf_region_idx[ i ] |  | u(1) |
| } else if (alf_no_filters_minus1 > 1) { |  |  |
| if( remain_fixed_regions > 0 ) { |  |  |
| prev_alf_region_pred_flag[ i ] |  | u(1) |
| if( prev_alf_region_pred_flag[ i ] ) |  |  |
| rem_alf_region_pred_idx[ i ] |  | ue(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| if (AlfNumFilters > 1) |  |  |
| alf_pred_method | 2 | u(1) |
| alf_min_kstart_minus1 | 2 | ue(v) |
| for (i=0; i < AlfMaxDepth; i++) |  |  |
| alf_golomb_index_bit[i] | 2 | u(1) |

alf_region_idx[ i ] specifies the filter index for adaptive loop filter.

prev_alf_region_pred_flag [ i ] and rem_alf_region_pred_idx [ i ] specify the filter index for adaptive loop filter . When prev_alf_region_pred_flag[i ] is equal to 0, the alf_region_idx [ i ] will be inferred from a index of the neighboring classes.

an array AlfRegionIdx is derived as follows. Initially,

AlfRegionIdx[0] is set to be 0

a value alf_curr_filters is initialized to be 1.

For each i in the range of 1 to AlfNumRegions -1, the following steps is processed,

If alf_no_filters_minus1 is 0, AlfRegionIdx[i] is set to be 0.

Otherwise if alf_no_filters_minus1 is 1, AlfRegionIdx[i] is set to be alf_region_idx[i]

Otherwise if alf_no_filters_minus 1 is larger than 1, AlfRegionIdx[i] is derived the following steps

a value of alf_remain_regions is derived as AlfNumRegions - i

a value of alf_remain_filters is derived as AlfNumFilters - alf_curr_filters

a value of remain_fixed_region is derived as alf_remain_regions - alf_remain_filters

if remain_fixed_region is 0, AlfRegionIdx[i] is inferred to be a value of alf_curr_filters

Otherwise if prev_alf_region_pred_flag[i] is 1, AlfRegionIdx[i] is inferred to a predicted value mpm_region_idx[i].

Otherwise if rem_alf_region_pred_idx[i] is less than mpm_region_idx[i], AlfRegionIdx[i] is set to be prev_alf_region_pred_flag[i].

Otherwise AlfRegionIdx[i] is set to be rem_alf_region_pred_idx[i].

If AlfRegionIdx[i] is equal to alf_curr_filters, alf_curr_filters is incremented with alf_curr_filters + 1.

mpm_region_idx[i] is derived as follows,

If alf_remain_regions is larger than alf_remain_filters >> 1, mpm_region_idx[i] is derived as alf_curr_filters.

Otherwise if (i % 5) is 0 and alf_region_adaptation_flag is 0, mpm_region_idx[i] is derived as AlfRegionIdx[i-5]

Otherwise mpm_region_idx[i] is derived as AlfRegionIdx[i-1]

- 8.6.6.6	Derivation process for filter index array for luma samples

…

…

- When alf_region_adaptation_flag is equal to 1, the following ordered steps apply.

- The variables xIdx and yIdx are derived as

regionTab[16] = { 0, 1, 4, 5, 15, 2, 3, 6, 14, 11, 10, 7, 13, 12, 9, 8 }	(8-1)
	offset = 1 << (Log2MaxCUSize – 1) 		(8-2)
	xInterval = ( ( ( PicWidthInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2	(8-3)
	yInterval = ( ( ( PicHeightInSamplesL + offset ) >> Log2MaxCUSize ) + 1 ) >> 2	(8-4)
	xIdx = Min( 3, Floor( ( xC + x ) / ( xInterval << Log2MaxCUSize ) ) )	(8-5)
	yIdx = Min( 3, Floor( ( yC + y ) / ( yInterval << Log2MaxCUSize ) ) )	(8-6)

- The filter index fIdx[ x, y ] with x, y = 0..(nS)-1 is derived as

fIdx[ x ][ y ] = regionTab[ ( yIdx << 2 ) + xIdx ]		(8-7)

- Otherwise (alf_region_flag is equal to 0), the following ordered steps apply.

- The variables varTempH[ x ][ y ], varTempV[ x ][ y ] and varTemp1[ x ][ y ] with x, y = 0,2,4,...(nS)-1is derived as

// for filter shape dependent BA mode

if alf_filter_shape is equal to 0,

varTempH[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x-1, yC+y-1 ] – s’’[ xC+x+1, yC+y+1 ] |	(8-8)
		varTempV[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x+1, yC+y-1 ] – s’’[ xC+x-1, yC+y+1 ] |	(8-9)

otherwise

varTempH[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x-1, yC+y ] – s’’[ xC+x+1, yC+y ] |	(8-10)
		varTempV[ x ][ y ] = | ( s’’[ xC+x, yC+y ] << 1 ) – s’’[ xC+x, yC+y-1 ] – s’’[ xC+x, yC+y+1 ] |	(8-11)

- The variables varTemp3[ x, y ], varTempH1[ x, y ] and varTempV1[ x, y ] with x, y = 0..( (nS) – 1 )>>2 are derived as

//  for 4 points

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 1, 2 	(8-12)
	varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i, j = 1, 2 	(8-13)
 	varTemp3[ x ][ y ] = (varTempH1[ x ][ y ] + varTempV1[ x ][ y ]) >> 2	(8-14)

//  for 8 points

varTempH1[ x ][ y ] = ij varTempH[ (x << 2 ) + i ][ (y << 2) + j ] with i = 0..3, j = 1, 2 	(8-15)
	varTempV1[ x ][ y ] = ij varTempV[ (x << 2 ) + i ][ (y << 2) + j ] with i = 0..3, j = 1, 2 	(8-16)
 	varTemp3[ x ][ y ] = (varTempH1[ x ][ y ] + varTempV1[ x ][ y ]) >> 3	(8-17)

# Simulation results

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% |
| Class B | -0.1% | 0.0% | 0.0% |
| Class C | -0.2% | 0.0% | 0.0% |
| Class D | -0.2% | 0.1% | 0.1% |
| Class E | -0.2% | 0.0% | 0.0% |
| Overall | -0.2% | 0.0% | 0.0% |
|  | -0.2% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.2% | 0.0% | 0.0% |
| Class B | -0.3% | 0.0% | -0.1% |
| Class C | -0.3% | 0.0% | 0.1% |
| Class D | -0.3% | 0.2% | 0.2% |
| Class E |  |  |  |
| Overall | -0.3% | 0.0% | 0.0% |
|  | -0.3% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.2% | 0.0% | -0.1% |
| Class C | -0.3% | 0.2% | -0.1% |
| Class D | -0.3% | 0.2% | 0.2% |
| Class E | -0.4% | 0.8% | 0.3% |
| Overall | -0.3% | 0.2% | 0.1% |
|  | -0.3% | 0.2% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.4% | 0.1% | 0.1% |
| Class C | -0.4% | -0.1% | 0.0% |
| Class D | -0.4% | 0.1% | -0.4% |
| Class E | -0.7% | 0.0% | 0.1% |
| Overall | -0.4% | 0.0% | -0.1% |
|  | -0.4% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusions

In this proposal, the coding efficiency is improved by adopting combination of CE8.a.1. and CE8.a.2. The coding efficiency is improved by is 0.2 %, 0.3 %, 0.3% and 0.4 % in HE-AI, RA, LB, and LP.

# Patent rights declaration(s)

TOSHIBA Corporation, MediaTek Inc., Sharp Corporation, and Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).