| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, Switzerland, 21-30 Nov., 2011 | Document: JCTVC-G692 WG11 Number: m22260 |
| --- | --- |

| Title: | Fixed Intra Mapping Mode for Run-Level Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim and Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | Tel: +1-360-817-8487 asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

- In this contribution, a fixed intra mapping mode (FIMM) technique for CAVLC is introduced. The proposed technique improves the performance of CAVLC for high bit-rate applications by selectively switching between the current run-level coding mode and fixed intra run-level coding mode in CAVLC. For high bit-rate situations (i.e., not common conditions), it is reported that the average BD bitrate changes are observed to be Y:-0.6%, U:-0.5%, V:-0.4%; RA-LC: Y:-0.5%, U:-0.3%, V:-0.3%.

# Introduction

The run-level coding in CAVLC is an efficient coding method to compress consecutive zeros and the following significant coefficient together. In the run-level coding, (run, lev) symbol is first encoded to indicate whether the level of the following significant coefficient is equal to ‘1.’ Here, (run, lev) pair is mapped into a codeword index based on block type (intra or inter). In other words, different mapping methods are required between inter block and intra block because of the statistical differences. However, in high bitrate coding environment, statistics of the residual data between intra and inter become similar. So, in some cases, intra mapping method can be more efficient to deal with inter block case.

Figure 1 Proposed Fixed Intra Mapping Mode

# Fixed Intra Mapping Mode

Fixed intra mapping mode run-level coding technique is introduced to improve coding performance in high bitrate coding by selectively switching between the current run-level coding mode and fixed intra run-level coding mode. As shown in Figure 1, fixed intra mapping flag is employed to specify which mapping mode to use between the conventional mapping mode and fixed intra mapping mode.

We have also observed that the quantization parameter (QP) and resolution information are good indicators of ‘G’ which represents the corresponding category. This information can be determined at the decoder or calculated at the encoder and signaled to the decoder. Here, we proposed to calculate the information at the encoder and transmit the indicators in the bit-stream.

The detailed process can be described as follows;

If (QP<TH1): Q=3 (very high bitrates)

Else if (QP<TH2): Q=2 (high bitrates)

Else if (QP<TH3): Q=1(medium bitrates)

Else low bitrates: Q=0 (low bitrates), where (TH1<TH2<TH3)

Resolution information is used to classify the block into two cases (High/Low)

If (Current Image Width>=W): R=1 (High resolution)

Else (Low resolution)

G=2*Q+R (0<G<7)

Finally, 3 bits are to be sent to signal ‘G’ in slice header. As shown in Figure 2, fixed intra mapping mode flag is determined based on block type and a trailing one (TR1). Here, TR1 represents whether or not the level of the last significant coefficient is equal to ‘1’

Figure 2 Decision Tree for determining fixed intra mapping mode

# Results

We implemented the FIMM technology into the HM4.0 software and tested with the recommended configuration on all sequences. Table 1 below shows the summary results of FIMM method vs. HM4.0 anchor.

Table 1. Summary RD results of FIMM vs. HM4.0 Anchor (QP=7, 12, 17, 22)

|  | Low delay (B) LC |  |  |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  | Y | U | V |
| Class A |  |  |  |  | Class A | -0.6 | -0.4 | -0.4 |
| Class B | -1.0 | -0.4 | -0.3 |  | Class B | -0.6 | -0.4 | -0.4 |
| Class C | -0.5 | -0.3 | -0.3 |  | Class C | -0.3 | -0.2 | -0.2 |
| Class D | -0.5 | -0.5 | -0.4 |  | Class D | -0.4 | -0.3 | -0.3 |
| Class E | -0.5 | -0.9 | -0.9 |  | Class E |  |  |  |
| Overall | -0.6 | -0.5 | -0.4 |  | Overall | -0.5 | -0.3 | -0.3 |

The average BD bitrate changes in high bitrate condition are reported to be LD-LC: Y:-0.6%, U:-0.5%, V:-0.4%; RA-LC: Y:-0.5%, U:-0.3%, V:-0.3%.

Table 2. Summary RD results of FIMM vs. HM4.0 Anchor (QP=22, 27, 32, 37)

|  | Low delay (B) LC |  |  |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  |  | Y | U | V |
| Class A |  |  |  |  | Class A | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 |  | Class B | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 |  | Class C | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 |  | Class D | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 |  | Class E |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 |  | Overall | 0.0 | 0.0 | 0.0 |

- From above results, it can be observed that FIMM method provides coding gain;-0.5%/-0.3%/-0.3% for random access and -0.6%/-0.5%/-0.4% for low delay and high quality. In the experiments, for the simplification, the TH1, TH2, TH3 are set to 16, 20, and 27, respectively. When it comes to resolution information, ‘W’ is also set to 1280 pixels.

# Conclusion

Introduced FIMM technique for CAVLC has been verified to improve the coding performance in high bitrates environment by selectively switching between the current run-level coding mode and fixed intra run-level coding mode. We observe an average luminance coding gain of 0.5% and 0.6% for random access and low delay, respectively.

Given this level of performance and the importance of anticipating a wide variety of applications in the HEVC standard, we propose the JCT-VC adopt FIMM method into the HM.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCTVC-E700, “Common test conditions and software reference configurations,” 5th JCT-VC meeting, Geneva, Mar. 2011.