# Proposed Changes to Working Draft by adding

- An option of sending SRLC flag bits

- Fixed intra mapping mode based on SRLC flag

### Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| if( IdrPicFlag ) { |  |
| idr_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |
| else { |  |
| pic_order_cnt_lsb | u(v) |
| ref_pic_set_pps_flag | u(1) |
| if( ! ref_pic_set_pps_flag ) |  |
| ref_pic_set( num_ref_pic_sets ) |  |
| else |  |
| ref_pic_set_pps_idx | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } |  |
| if( !lightweight_slice_flag ) { |  |
| entropy_coding_mode_flag | u(1) |
| if(entropy_coding_mode_flag ==0 && QPY < TH) { |  |
| if(PicWidthInSamplesL < W) R =0; |  |
| else R=1; |  |
| if(QPY <Th_v) Q=3; |  |
| else if (QPY <Th_h) Q=2; |  |
| else if (QPY <Th_m) Q=1; |  |
| else Q=0; |  |
| SRLC_G = 2*Q + R |  |
| SRLC_G } | u(v) |
| } |  |

7.4.3 Slice header semantics

SRLC_G specifies the category information used for fixed intra mapping mode. Each category requires different condition to determine ‘fixed_intra_mapping_mode_flag’.

The followings are added to decoding process

9.2.4.2 Parsing process for SRLC_G

This process is invoked when entropy_coding_mode_flag is equal to 0 for parsing syntax element SRLC_G in subclause 7.7.3.

Inputs to this process are bits from slice data, SRLC_G.

Output of this process is fixed_intra_mapping_mode_flag.

The value of fixed_intra_mapping_mode_flag is derived as follow.

1. Obtain a 3-bit value SRLC_G according to subclause 7.3.3

2. Derive Q value and R value according to the following equations

Q = SRLC_flag/2 			                      (9-xx)

R = SRLC_flag%2 			                      (9-xx)

3. Based on Q and R, determine ‘fixed_intra_mapping_mode_flag’

1. If Q<3, set fixed_intra_mapping_mode_flag to 0

2. Otherwise, fixed_intra_mapping_mode_flag is determined based on ‘R’ as follows

if(R= =0) {

if ( tr1= =0 ) flag=1;

else flag=0;}

else {

if( blockType>1) flag=1;

else flag=0;}}

NOTE- tr1 is determined based the level of the last significant coefficient.

#### Parsing process for run_level_one

Inputs to this process are bits from slice data, the scan position n of the previous non-zero transform coefficient in inverse scan order, and the variable TrOne.

Outputs of this process are the syntax element run_level_one and the variable TrOne.

The value of run_level_one is derived as follows.

- The variable N is set equal to (1 << log2TrafoSize) >> (cIdx > 0 ? 2 : 0).

- The variable blockType is derived as

blockType = (cIdx == 0 ? (PredMode==MODE_INTRA ? 0 : slice_type + 1) + (N > 8 ? 5 : 2) : cIdx – 1) 											(9-1)

- The variable tableIdx is derived as

tableIdx = (blockType = = 2 | |  blockType = = 5) ? (N <= 8 ? 0 : 1) : (N <= 8 ? 2 : 3) 	(9-2)

- The variable maxRun is set equal to n.

- The variable maxRunIdx is set equal to Min(28, maxRun).

- The variable vlcNum is derived from tableIdx and maxRunIdx as shown in Error: Reference source not found.

- The parsing process described in subclause Error: Reference source not found is invoked with vlcNum as input and the variable codeNum as output. If blockType is equal to 2 or blockType is equal to 5, the value of run_level_one is derived as follows.

  - If N is equal to 4, the variable largeOnePos is derived from trOne and maxRunIdx as shown in Error: Reference source not found.

  - Otherwise (N is greater than 4), the variable largeOnePos is derived from trOne and maxRunIdx as shown in Error: Reference source not found.

  - The variables levelGreaterThanOneFlag and runOfZeros are derived as follows.

if ( largeOnePos > 0 ) {
		if( codeNum < min(largeOnePos, maxRunIdx + 2 ) ) {
			levelGreaterThanOneFlag = 0
			runOfZeros = codeNum
		} else if( codeNum < (maxRunIdx << 1 ) + 4 – largeOnePos ) {
			if(( codeNum + largeOnePos ) & 1 ) {
				levelGreaterThanOneFlag = 0
				runOfZeros = (codeNum + largeOnePos - 1) >> 1
			} else {
				levelGreaterThanOneFlag = 1
				runOfZeros = (codeNum - largeOnePos)>>1	(9-3)
			}
		} else {
			levelGreaterThanOneFlag = 1
			runOfZeros = codeNum - maxRunIdx – 2
		}
	} else {
		if( codeNum & 1) {
			levelGreaterThanOneFlag = 0 
			runOfZeros = (codeNum - 1)>>1
		} else {
			runOfZeros = codeNum >> 1
			levelGreaterThanOneFlag = ((codeNum >> 1) <= maxRunIdx)?1:0
		}
	}

  - The syntax element run_level_one is derived as

run_level_one = levelGreaterThanOneFlag ? runOfZeros + maxRun : runOfZeros	(9-4)

- Otherwise (Fixed_intra_mapping_flag is equal to 0 and blockType is not equal to 2 and blockType is not equal to 5), the value of run_level_one is derived as follows.

  - The variable levelGreaterThanOneFlag is derived as

levelGreaterThanOneFlag  = codeNum <= maxRun + 1 ? 0 : 1	(9-5)

  - If maxRun is less than 28, the variable runOfZeros is derived from maxRun and codeNum as shown in Error: Reference source not found.

  - Otherwise (maxRun is not less than 28), the variable runOfZeros is derived as

runOfZeros = codeNum <= maxRun + 1 ? codeNum : codeNum – maxRun – 2	(9-6)

  - The syntax element run_level_one is derived as

run_level_one = levelGreaterThanOneFlag ? runOfZeros + n : runOfZeros	(9-7)

- The variable trOne is derived as

trOne = (trOne = = 0 | | levelGreaterThanOneFlag) ? 0 : Max(4, trOne + 1) 	(9-8)