| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G696 |
| --- | --- |

| Title: | CE3: Fixed interpolation filter tests by Motorola Mobility |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Koohyar Minoo David Baylon Limin Wang Ajay Luthra 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-2521 kminoo@motorola.com +1-858-404-2285 david.baylon@motorola.com +1-858-404-3893 limin.wang@motorola.com +1-858-404-3470 aluthra@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This document reports the results of Motorola Mobility’s interpolation filters for HEVC. The simulations were conducted using HM4.0 software with Motorola Mobility’s modifications. Four sets of fixed interpolation filters are tested. Compared with the current interpolation filter in HM4.0, the proposed 6-tap half-pel filter with 7-tap quarter-pel filter scheme with 13/64 offset achieves 0.0%, 0.3%, 0.1%, 0.2%, -0.3% and -0.4% bitrate differences in RAHE, RALC, LBHE, LBLC, LPHE and LPLC settings; the proposed 6-tap half-pel filter with 7-tap quarter-pel filter scheme with 3/16 offset achieves 0.4%, 0.9%, 0.3%, 0.5%, -0.4% and -1.2% bitrate differences in RAHE, RALC, LBHE, LBLC, LPHE and LPLC settings; the proposed 6-tap half-pel filter with 7-tap quarter-pel filter scheme with 15/64 offset achieves -0.1%, 0.0%, 0.1%, 0.2%, 0.3% and -0.3% bitrate differences in RAHE, RALC, LBHE, LBLC, LPHE and LPLC settings; the proposed 8-tap half-pel filter with 8-tap quarter-pel filter scheme with 3/16 offset achieves -0.1%, -0.3%, 0.1%, -0.4%, -0.8% and -0.3% bitrate differences in RAHE, RALC, LBHE, LBLC, LPHE and LPLC settings. Cross-check will be provided by Samsung. The attached spreadsheet contains detailed data of the results.

# Introduction

The target of this document is to verify the performance of Motorola Mobility’s 6-bit interpolation filter for HEVC. The simulations were conducted using HM4.0 software [1] with Motorola Mobility’s modifications. Cross-check will be provided by Samsung [2].

Fig. 1 illustrates the positions of the resulting half-pel and quarter-pel pixels between full-pel pixels along a pixel line within an image, where L3, L2, L1, L0, R0, R1, R2, and R3 are full-pel pixels, H is a half-pel pixel between L0 and R0, FL is a sub-pel pixel between L0 and H, and FR is a sub-pel pixel between H and R0.

Fig. 1 Sub-Pel Positions

# Proposed sub-pel interpolation filters

Table 1 summarizes the Luma filter coefficients of the proposed four filter sets. All the four Luma filter sets use the same Chroma filter set which is summarized in Table 2.

Table 1 Sub-pel pixel Luma interpolation filter coefficients

|  | Position | Coefficients |
| --- | --- | --- |
| 6H+7Q (13/64) | FL | {-1, 4, -10, 61, 13, -4, 1, 0,} |
|  | H | {0, 2, -9, 39, 39, -9, 2, 0,} |
|  | FR | {0, 1, -4, 13, 61, -10, 4, -1,} |
| 6H+7Q (3/16) | FL | {-1, 3, -8, 60, 13, -4, 1, 0,} |
|  | H | {0, 2, -9, 39, 39, -9, 2, 0,} |
|  | FR | {0, 1, -4, 13, 60, -8, 3, -1,} |
| 6H+7Q (15/64) | FL | {-2, 5, -11, 60, 15, -4, 1, 0,} |
|  | H | {0, 2, -9, 39, 39, -9, 2, 0,} |
|  | FR | {0, 1, -4, 15, 60, -11, 5, -2,} |
| 8H+8Q (3/16) | FL | {-1, 3, -8, 60, 14, -6, 3, -1,} |
|  | H | {-1, 4, -11, 40, 40, -11, 4, -1,} |
|  | FR | {-1, 3, -6, 14, 60, -8, 3, -1,} |

Table 2 Sub-pel pixel Chroma interpolation filter coefficients

| Position | Coefficients |
| --- | --- |
| 1/8 | {-2, 61, 6, -1, } |
| 1/4 | {-4, 54, 16, -2,} |
| 3/8 | {-5, 44, 29, -4,} |
| 1/2 | {-4, 36, 36, -4,} |
| 5/8 | {-4, 29, 44, -5,} |
| 3/4 | {-2, 16, 54, -4,} |
| 7/8 | {-1, 6, 61, -2,} |

# Simulation environment

## Software

The software used to run the simulations is HM4.0. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [3]. YUV file output during the decoding was disabled when collecting the decoding time statistics.

## Hardware

All the simulation experiments for 6H+7Q with 13/64 offset, 6H+7Q with 3/16 offset and 8H+8Q with 3/16 offset were conducted on the PCs with identical settings, i.e., Microsoft Windows 7 64bits, Intel Xeon 3.2GHz with 14G memory. Therefore, both the encoding time and decoding time comparisons are reliable. The simulation experiments for 6H+7Q with 15/64 offset were conducted on the PCs with different settings, so the encoder time and decoding time comparisons are not provided.

# Simulation Results

The experimental results of the rate-distortion performance, speed comparison and complexity analysis are summarized in Table 3-6. Detailed results are included in the attachments (JCTVC-G696(6H7Q 13_64).xls, JCTVC-G696(6H7Q 3_16).xls, JCTVC-G696(6H7Q 15_64).xls, JCTVC-G696(8H8Q 3_16).xls).

Table 3 Rate-distortion performance, speed comparison and complexity analysis of 6H7Q with 13/64 offset

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.2% | -0.1% | 0.1% | 0.2% | 0.1% |
| Class B | -0.2% | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% |
| Class C | 0.2% | 0.3% | 0.2% | 0.2% | 0.2% | 0.3% |
| Class D | 0.3% | 0.2% | 0.3% | 0.6% | 0.4% | 0.4% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.3% | 0.2% | 0.2% |
|  | 0.0% | 0.1% | 0.1% | 0.3% | 0.2% | 0.2% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |
| Mult[%] | -5% |  |  | -7% |  |  |
| Adds[%] | -8% |  |  | -9% |  |  |
| Access[%] | 2% |  |  | 2% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 1% |  |  | 5% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 2% |  |  | 5% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -0.1% | 0.1% | 0.0% | -0.1% | -0.2% |
| Class C | 0.2% | 0.4% | 0.4% | -0.1% | 0.6% | 0.3% |
| Class D | 1.2% | 1.0% | 1.1% | 0.9% | 1.0% | 0.9% |
| Class E | -0.7% | -0.2% | -0.1% | 0.2% | 0.3% | -0.4% |
| Overall | 0.1% | 0.3% | 0.4% | 0.2% | 0.4% | 0.2% |
|  | 0.1% | 0.3% | 0.3% | 0.2% | 0.4% | 0.2% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| Mult[%] | -6% |  |  | -7% |  |  |
| Adds[%] | -8% |  |  | -9% |  |  |
| Access[%] | 2% |  |  | 1% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 4% |  |  | 6% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 5% |  |  | 5% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.5% | -0.3% | 0.0% | -0.7% | -0.3% | -0.5% |
| Class C | -0.3% | 0.1% | 0.2% | -0.4% | 0.5% | 0.6% |
| Class D | 0.8% | 0.3% | 1.2% | 0.8% | 1.0% | 0.6% |
| Class E | -1.5% | -1.5% | -0.5% | -1.2% | -0.8% | -1.0% |
| Overall | -0.3% | -0.3% | 0.3% | -0.4% | 0.1% | 0.0% |
|  | -0.3% | -0.3% | 0.2% | -0.4% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
| Mult[%] | -5% |  |  | -7% |  |  |
| Adds[%] | -8% |  |  | -9% |  |  |
| Access[%] | 2% |  |  | 1% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 3% |  |  | 5% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 2% |  |  | 4% |  |  |
| Gain Ave.[%] | -0.01% | 0.15% | 0.17% |  |  |  |
| Gain Min. [%] | -2.25% | -1.86% | -1.47% |  |  |  |
| Gain Max.[%] | 2.55% | 2.51% | 4.08% |  |  |  |
| Enc Time[%] | 100.54% |  |  |  |  |  |
| Dec Time[%] | 100.50% |  |  |  |  |  |
| Mult Ave.[%] | -6% |  |  |  |  |  |
| Mult Worst[%] | -9% |  |  |  |  |  |
| Adds Ave.[%] | -8% |  |  |  |  |  |
| Adds Worst[%] | -11% |  |  |  |  |  |
| Access[%] | 2% |  |  |  |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  |  |  |  |
| MemBand(2D) Max.[%] | 4% |  |  |  |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |  |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  |  |  |  |
| MemBand(1D) Max.[%] | 4% |  |  |  |  |  |
| MemBand(1D) Worst[%] | -5% |  |  |  |  |  |

Table 4 Rate-distortion performance, speed comparison and complexity analysis of 6H7Q with 3/16 offset

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.2% | -0.4% | 0.3% | 0.3% |
| Class B | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class C | 0.6% | 0.3% | 0.5% | 1.3% | 0.5% | 0.5% |
| Class D | 1.1% | 1.1% | 1.0% | 3.1% | 1.3% | 1.3% |
| Class E |  |  |  |  |  |  |
| Overall | 0.4% | 0.3% | 0.3% | 0.9% | 0.5% | 0.5% |
|  | 0.3% | 0.3% | 0.3% | 0.9% | 0.5% | 0.5% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |
| Mult[%] | -6% |  |  | -6% |  |  |
| Adds[%] | -8% |  |  | -9% |  |  |
| Access[%] | 2% |  |  | 2% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 1% |  |  | 5% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 1% |  |  | 4% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.1% | -0.2% | -0.3% | -0.1% | -0.1% |
| Class C | 0.4% | 0.6% | 0.5% | 0.9% | 0.7% | 0.4% |
| Class D | 1.6% | 1.5% | 1.8% | 2.5% | 1.7% | 1.5% |
| Class E | -0.6% | -0.2% | 0.5% | -1.3% | -0.1% | -0.9% |
| Overall | 0.3% | 0.5% | 0.6% | 0.5% | 0.6% | 0.3% |
|  | 0.3% | 0.5% | 0.5% | 0.5% | 0.5% | 0.3% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| Mult[%] | -4% |  |  | -4% |  |  |
| Adds[%] | -6% |  |  | -7% |  |  |
| Access[%] | 2% |  |  | 2% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 3% |  |  | 8% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 5% |  |  | 7% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6% | -0.6% | -0.5% | -1.6% | -0.3% | -0.7% |
| Class C | -0.5% | -0.3% | 0.1% | -0.4% | 0.1% | 0.1% |
| Class D | 0.7% | 0.5% | 0.1% | 0.7% | 0.5% | 0.4% |
| Class E | -1.6% | -0.9% | -0.6% | -4.1% | -1.9% | -1.2% |
| Overall | -0.4% | -0.3% | -0.2% | -1.2% | -0.3% | -0.3% |
|  | -0.4% | -0.4% | -0.3% | -1.2% | -0.3% | -0.4% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
| Mult[%] | -3% |  |  | -3% |  |  |
| Adds[%] | -6% |  |  | -6% |  |  |
| Access[%] | 3% |  |  | 3% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  | 0% |  |  |
| MemBand(2D) Max.[%] | 2% |  |  | 9% |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  | 0% |  |  |
| MemBand(1D) Max.[%] | 2% |  |  | 9% |  |  |
| Gain Ave.[%] | 0.09% | 0.20% | 0.18% |  |  |  |
| Gain Min. [%] | -6.89% | -2.69% | -2.63% |  |  |  |
| Gain Max.[%] | 8.36% | 5.25% | 4.72% |  |  |  |
| Enc Time[%] | 100.68% |  |  |  |  |  |
| Dec Time[%] | 100.68% |  |  |  |  |  |
| Mult Ave.[%] | -4% |  |  |  |  |  |
| Mult Worst[%] | -9% |  |  |  |  |  |
| Adds Ave.[%] | -7% |  |  |  |  |  |
| Adds Worst[%] | -11% |  |  |  |  |  |
| Access[%] | 2% |  |  |  |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  |  |  |  |
| MemBand(2D) Max.[%] | 5% |  |  |  |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |  |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  |  |  |  |
| MemBand(1D) Max.[%] | 5% |  |  |  |  |  |
| MemBand(1D) Worst[%] | -5% |  |  |  |  |  |

Table 5 Rate-distortion performance, speed comparison and complexity analysis of 6H7Q with 15/64 offset

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | -0.1% | -0.2% | 0.4% | 0.3% |
| Class B | -0.2% | 0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.2% | 0.2% | 0.2% | 0.3% |
| Class D | -0.3% | 0.2% | 0.1% | 0.3% | 0.3% | 0.4% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.1% | 0.1% | 0.0% | 0.2% | 0.2% |
|  | -0.1% | 0.1% | 0.1% | 0.0% | 0.2% | 0.2% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
| Mult[%] | -9% |  |  | -10% |  |  |
| Adds[%] | -11% |  |  | -12% |  |  |
| Access[%] | 0% |  |  | -1% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 6% |  |  | 5% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | 6% |  |  | 5% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.2% | 0.0% | -0.2% | 0.2% | 0.1% |
| Class C | -0.1% | 0.6% | 0.5% | 0.2% | 0.6% | 0.2% |
| Class D | 0.5% | 1.9% | 1.5% | 1.0% | 1.3% | 0.9% |
| Class E | 0.2% | -0.1% | 0.7% | -0.2% | -0.3% | -0.3% |
| Overall | 0.1% | 0.7% | 0.6% | 0.2% | 0.5% | 0.3% |
|  | 0.1% | 0.6% | 0.6% | 0.2% | 0.5% | 0.3% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
| Mult[%] | -11% |  |  | -11% |  |  |
| Adds[%] | -13% |  |  | -14% |  |  |
| Access[%] | -1% |  |  | -1% |  |  |
| MemBand(2D) Ave.[%] | -3% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 3% |  |  | 2% |  |  |
| MemBand(1D) Ave.[%] | -3% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | 2% |  |  | 1% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.5% | -0.8% | 0.2% | -0.1% |
| Class C | 0.1% | 0.8% | 0.9% | -0.1% | 0.5% | 0.6% |
| Class D | 1.3% | 1.5% | 1.7% | 1.2% | 1.8% | 1.7% |
| Class E | -0.2% | 0.0% | 0.1% | -1.7% | -1.5% | -0.9% |
| Overall | 0.3% | 0.7% | 0.8% | -0.3% | 0.4% | 0.4% |
|  | 0.3% | 0.7% | 0.8% | -0.3% | 0.4% | 0.4% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
| Mult[%] | -11% |  |  | -12% |  |  |
| Adds[%] | -14% |  |  | -14% |  |  |
| Access[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Ave.[%] | -3% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 0% |  |  | 2% |  |  |
| MemBand(1D) Ave.[%] | -3% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | 0% |  |  | 2% |  |  |
| Gain Ave.[%] | 0.04% | 0.41% | 0.40% |  |  |  |
| Gain Min. [%] | -3.45% | -2.66% | -2.58% |  |  |  |
| Gain Max.[%] | 3.14% | 3.92% | 3.75% |  |  |  |
| Enc Time[%] | #NUM! |  |  |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |  |  |
| Mult Ave.[%] | -11% |  |  |  |  |  |
| Mult Worst[%] | -9% |  |  |  |  |  |
| Adds Ave.[%] | -13% |  |  |  |  |  |
| Adds Worst[%] | -11% |  |  |  |  |  |
| Access[%] | -1% |  |  |  |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  |  |  |  |
| MemBand(2D) Max.[%] | 3% |  |  |  |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |  |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  |  |  |  |
| MemBand(1D) Max.[%] | 3% |  |  |  |  |  |
| MemBand(1D) Worst[%] | -5% |  |  |  |  |  |

Table 6 Rate-distortion, speed performance comparison and complexity analysis of 8H8Q with 3/16 offset

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.1% | 0.0% | 0.3% | 0.3% |
| Class B | 0.0% | 0.1% | 0.1% | -0.3% | 0.0% | 0.1% |
| Class C | -0.1% | 0.1% | 0.1% | -0.3% | 0.1% | 0.1% |
| Class D | -0.2% | 0.1% | 0.0% | -0.4% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.1% | 0.1% | -0.3% | 0.1% | 0.1% |
|  | -0.1% | 0.1% | 0.1% | -0.3% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
| Mult[%] | 7% |  |  | 7% |  |  |
| Adds[%] | 7% |  |  | 7% |  |  |
| Access[%] | 4% |  |  | 4% |  |  |
| MemBand(2D) Ave.[%] | 2% |  |  | 2% |  |  |
| MemBand(2D) Max.[%] | 8% |  |  | 9% |  |  |
| MemBand(1D) Ave.[%] | 2% |  |  | 2% |  |  |
| MemBand(1D) Max.[%] | 9% |  |  | 9% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | -0.2% | 0.4% | 0.7% |
| Class C | -0.1% | 0.1% | 0.1% | -0.5% | 0.5% | 0.2% |
| Class D | -0.1% | 0.3% | 0.4% | -0.6% | 0.5% | 0.5% |
| Class E | 0.4% | -0.2% | 0.5% | -0.2% | 0.0% | -0.6% |
| Overall | 0.1% | 0.1% | 0.3% | -0.4% | 0.4% | 0.3% |
|  | 0.1% | 0.0% | 0.2% | -0.4% | 0.3% | 0.3% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
| Mult[%] | 9% |  |  | 10% |  |  |
| Adds[%] | 10% |  |  | 10% |  |  |
| Access[%] | 5% |  |  | 5% |  |  |
| MemBand(2D) Ave.[%] | 2% |  |  | 3% |  |  |
| MemBand(2D) Max.[%] | 10% |  |  | 9% |  |  |
| MemBand(1D) Ave.[%] | 2% |  |  | 3% |  |  |
| MemBand(1D) Max.[%] | 10% |  |  | 9% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.0% | 0.0% | 0.1% | 0.2% | 0.1% |
| Class C | -1.0% | -0.1% | -0.3% | -0.6% | 0.3% | 0.4% |
| Class D | -1.3% | -0.6% | -0.6% | -1.2% | 0.1% | 0.1% |
| Class E | -0.5% | 0.3% | 0.4% | 0.9% | -0.5% | 0.4% |
| Overall | -0.8% | -0.1% | -0.1% | -0.3% | 0.1% | 0.2% |
|  | -0.8% | -0.2% | -0.1% | -0.3% | 0.0% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
| Mult[%] | 10% |  |  | 9% |  |  |
| Adds[%] | 10% |  |  | 9% |  |  |
| Access[%] | 5% |  |  | 5% |  |  |
| MemBand(2D) Ave.[%] | 2% |  |  | 3% |  |  |
| MemBand(2D) Max.[%] | 9% |  |  | 9% |  |  |
| MemBand(1D) Ave.[%] | 2% |  |  | 3% |  |  |
| MemBand(1D) Max.[%] | 9% |  |  | 8% |  |  |
| Gain Ave.[%] | -0.28% | 0.09% | 0.13% |  |  |  |
| Gain Min. [%] | -3.60% | -1.26% | -1.58% |  |  |  |
| Gain Max.[%] | 1.39% | 1.26% | 1.84% |  |  |  |
| Enc Time[%] | 100.44% |  |  |  |  |  |
| Dec Time[%] | 100.74% |  |  |  |  |  |
| Mult Ave.[%] | 9% |  |  |  |  |  |
| Mult Worst[%] | 0% |  |  |  |  |  |
| Adds Ave.[%] | 9% |  |  |  |  |  |
| Adds Worst[%] | 0% |  |  |  |  |  |
| Access[%] | 5% |  |  |  |  |  |
| MemBand(2D) Ave.[%] | 2% |  |  |  |  |  |
| MemBand(2D) Max.[%] | 9% |  |  |  |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |  |  |  |
| MemBand(1D) Ave.[%] | 2% |  |  |  |  |  |
| MemBand(1D) Max.[%] | 9% |  |  |  |  |  |
| MemBand(1D) Worst[%] | 0% |  |  |  |  |  |

# References

[1] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0

[2] E. Alshina, “CE3: Cross-check for Motorola Mobility proposals on interpolation MC by Samsung,” JCT-VC document, JCTVC-G820, November 2011

[3] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-F900, July 2011

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).