| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G697 |
| --- | --- |

| Title: | CE3: Joint sub-pixel interpolation filter tests for bi-predicted motion compensation by Motorola Mobility |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Koohyar Minoo 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-2521 kminoo@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This document reports the results of Motorola Mobility’s Joint Sub-Pixel Interpolation Filters (JSPIF) for bi-predicted motion compensation for HEVC. The simulations were conducted using HM4.0 software with Motorola Mobility’s modifications. Three sets of 6-tap fixed interpolation filters for bi-predicted motion compensation are used. For uni-prediction, 8H+7Q fixed filters with 3/16 offset are used. Compared with the current interpolation filter in HM4.0, set0 achieves -0.3%, -0.3%, -0.8% and -0.8% bitrate differences in RAHE, RALC, LBHE and LBLC settings; set1 achieves -0.2%, -0.2%, -0.7% and -0.7% bitrate differences in RAHE, RALC, LBHE and LBLC settings; set2 achieves -0.4%, -0.4%, -0.8% and -0.7% bitrate differences in RAHE, RALC, LBHE and LBLC settings. All the three sets achieve -0.8% and -1.3% bitrate differences in LPHE and LPLC settings. Cross-check will be provided by Samsung. The attached spreadsheet contains detailed data of the results.

# Introduction

The target of this document is to verify the performance of Motorola Mobility’s 6-bit Joint Sub-Pixel Interpolation Filter [1] for bi-predicted motion compensation for HEVC. The simulations were conducted using HM4.0 software [2] with Motorola Mobility’s modifications. Cross-check will be provided by Samsung [3].

Fig. 1 illustrates the positions of the resulting half-pel and quarter-pel pixels between full-pel pixels along a pixel line within an image, where L3, L2, L1, L0, R0, R1, R2, and R3 are full-pel pixels, H is a half-pel pixel between L0 and R0, FL is a sub-pel pixel between L0 and H, and FR is a sub-pel pixel between H and R0.

Fig. 1 Sub-Pel Positions

# Proposed JSPIF

## JSPIF Set 0

For List 0

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 0, 0)

{      -1,       3,      -8,      60,      13,      -4,       1,       0},	// ( 1, 0)

{      -1,       4,     -11,      40,      40,     -11,       4,      -1},	// ( 2, 0)

{       1,      -4,      13,      60,      -8,       3,      -1,       0}	// ( 3, 0)

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 0, 1)

{       1,      -3,       7,     -17,      62,      12,       0,       0},	// ( 1, 1)

{      -9,      40,      43,     -16,       6,      -2,       0,       0},	// ( 2, 1)

{      -4,      13,      61,     -10,       5,      -1,       0,       0}	// ( 3, 1)

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 0, 2)

{       1,      -3,       9,     -13,      61,      11,       0,       0},	// ( 1, 2)

{      37,      43,     -24,      12,      -6,       2,       0,       0},	// ( 2, 2)

{      11,      61,     -10,      10,      -3,       1,       0,       0}	// ( 3, 2)

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 0, 3)

{      -1,       5,     -10,      61,      13,      -4,       0,       0},	// ( 1, 3)

{      -2,       6,     -16,      43,      39,     -12,       0,       0},	// ( 2, 3)

{       1,      -3,       6,     -11,      15,      58,       0,       0}	// ( 3, 3)

For List 1

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 0, 0)

{      -1,       3,      -8,      60,      13,      -4,       1,       0},	// ( 0, 1)

{      -1,       4,     -11,      40,      40,     -11,       4,      -1},	// ( 0, 2)

{       1,      -4,      13,      60,      -8,       3,      -1,       0}	// ( 0, 3)

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 1, 0)

{      58,      15,     -11,       6,      -3,       1,       0,       0},	// ( 1, 1)

{      -9,      40,      43,     -16,       6,      -2,       0,       0},	// ( 1, 2)

{      -4,      13,      61,     -10,       5,      -1,       0,       0}	// ( 1, 3)

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 2, 0)

{       1,      -3,       9,     -13,      61,      11,       0,       0},	// ( 2, 1)

{       2,      -6,      12,     -24,      43,      37,       0,       0},	// ( 2, 2)

{      11,      61,     -10,      10,      -3,       1,       0,       0}	// ( 2, 3)

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 3, 0)

{      -1,       5,     -10,      61,      13,      -4,       0,       0},	// ( 3, 1)

{      -2,       6,     -16,      43,      39,     -12,       0,       0},	// ( 3, 2)

{      12,      62,     -17,       7,      -3,       1,       0,       0}	// ( 3, 3)

## JSPIF Set 1

For List 0

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 0, 0)

{      -9,      60,      14,      -6,       3,      -1,       0,       0},	// ( 1, 0)

{      -2,       5,     -11,      38,      41,     -14,       0,       0},	// ( 2, 0)

{      -1,       3,      -6,      14,      60,      -9,       0,       0}	// ( 3, 0)

{      -1,       4,       0,      64,       0,       0,       0,      0},	// ( 0, 1)

{       1,      -3,       7,     -17,      62,      12,       0,       0},	// ( 1, 1)

{      -9,      40,      43,     -16,       6,      -2,       0,       0},	// ( 2, 1)

{      -4,      13,      61,     -10,       5,      -1,       0,       0}	// ( 3, 1)

{      66,      -2,       7,       0,      -1,       1,       0,       0},	// ( 0, 2)

{       1,      -3,       9,     -13,      61,      11,       0,       0},	// ( 1, 2)

{      37,      43,     -24,      12,      -6,       2,       0,       0},	// ( 2, 2)

{      11,      61,     -10,      10,      -3,       1,       0,       0}	// ( 3, 2)

{      64,       0,       4,      -1,       0,       0,       0,       0},	// ( 0, 3)

{      -1,       5,     -10,      61,      13,      -4,       0,       0},	// ( 1, 3)

{      -2,       6,     -16,      43,      39,     -12,       0,       0},	// ( 2, 3)

{       1,      -3,       6,     -11,      15,      58,       0,       0}	// ( 3, 3)

For List 1

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 0, 0)

{      -9,      60,      14,      -6,       3,      -1,       0,       0},	// ( 0, 1)

{      -2,       5,     -11,      38,      41,     -14,       0,       0},	// ( 0, 2)

{      -1,       3,      -6,      14,      60,      -9,       0,       0}	// ( 0, 3)

{      -1,       4,       0,      64,       0,       0,       0,      0},	// ( 1, 0)

{      58,      15,     -11,       6,      -3,       1,       0,       0},	// ( 1, 1)

{      -9,      40,      43,     -16,       6,      -2,       0,       0},	// ( 1, 2)

{      -4,      13,      61,     -10,       5,      -1,       0,       0}	// ( 1, 3)

{      66,      -2,       7,       0,      -1,       1,       0,       0},	// ( 2, 0)

{       1,      -3,       9,     -13,      61,      11,       0,       0},	// ( 2, 1)

{       2,      -6,      12,     -24,      43,      37,       0,       0},	// ( 2, 2)

{      11,      61,     -10,      10,      -3,       1,       0,       0}	// ( 2, 3)

{      64,       0,       4,      -1,       0,       0,       0,       0},	// ( 3, 0)

{      -1,       5,     -10,      61,      13,      -4,       0,       0},	// ( 3, 1)

{      -2,       6,     -16,      43,      39,     -12,       0,       0},	// ( 3, 2)

{      12,      62,     -17,       7,      -3,       1,       0,       0}	// ( 3, 3)

## JSPIF Set 2

For List 0

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 0, 0)

{      -1,       3,      -8,      60,      13,      -4,       1,       0},	// ( 1, 0)

{      -1,       4,     -11,      40,      40,     -11,       4,      -1},	// ( 2, 0)

{       1,      -4,      13,      60,      -8,       3,      -1,       0}	// ( 3, 0)

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 0, 1)

{       1,      -3,       6,     -15,      61,      14,       0,       0},	// ( 1, 1)

{      -9,      40,      44,     -15,       6,      -2,       0,       0},	// ( 2, 1)

{      -4,      13,      61,     -10,       5,      -1,       0,       0},	// ( 3, 1)

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 0, 2)

{       1,      -3,       9,     -13,      61,       9,       0,       0},	// ( 1, 2)

{      37,      43,     -24,      12,      -6,       2,       0,       0},	// ( 2, 2)

{      10,      60,     -12,       8,      -2,       0,       0,       0},	// ( 3, 2)

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 0, 3)

{      -1,       5,     -10,      61,      13,      -4,       0,       0},	// ( 1, 3)

{      -2,       6,     -16,      45,      39,      -8,       0,       0},	// ( 2, 3)

{       1,      -3,       6,     -12,      15,      57,       0,       0},	// ( 3, 3)

For List 1

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 0, 0)

{      -1,       3,      -8,      60,      13,      -4,       1,       0},	// ( 0, 1)

{      -1,       4,     -11,      40,      40,     -11,       4,      -1},	// ( 0, 2)

{       1,      -4,      13,      60,      -8,       3,      -1,       0}	// ( 0, 3)

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 1, 0)

{      57,      15,     -12,       6,      -3,       1,       0,       0},	// ( 1, 1)

{      -9,      40,      44,     -15,       6,      -2,       0,       0},	// ( 1, 2)

{      -4,      13,      61,     -10,       5,      -1,       0,       0},	// ( 1, 3)

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 2, 0)

{       1,      -3,       9,     -13,      61,       9,       0,       0},	// ( 2, 1)

{       2,      -6,      12,     -24,      43,      37,       0,       0},	// ( 2, 2)

{      10,      60,     -12,       8,      -2,       0,       0,       0},	// ( 2, 3)

{      64,       0,       0,       0,       0,       0,       0,      0},	// ( 3, 0)

{      -1,       5,     -10,      61,      13,      -4,       0,       0},	// ( 3, 1)

{      -2,       6,     -16,      45,      39,      -8,       0,       0},	// ( 3, 2)

{      14,      61,     -15,       6,      -3,       1,       0,       0},	// ( 3, 3)

## Uni-prediction Filters

For uni-prediction, 8H+7Q fixed filters with 3/16 offset are used. Table 1 summarizes the Luma filter coefficients.

Table 1 Sub-pel pixel Luma interpolation filter coefficients

| Position | Coefficients |
| --- | --- |
| FL | {-1, 3, -8, 60, 13, -4, 1, 0,} |
| H | {-1, 4, -11, 40, 40, -11, 4, -1,} |
| FR | {0, 1, -4, 13, 60, -8, 3, -1,} |

## Chroma Filters

Table 2 summarizes the Chroma filter coefficients.

Table 2 Sub-pel pixel Chroma interpolation filter coefficients

| Position | Coefficients |
| --- | --- |
| 1/8 | {-2, 61, 6, -1, } |
| 1/4 | {-4, 54, 16, -2,} |
| 3/8 | {-5, 44, 29, -4,} |
| 1/2 | {-4, 36, 36, -4,} |
| 5/8 | {-4, 29, 44, -5,} |
| 3/4 | {-2, 16, 54, -4,} |
| 7/8 | {-1, 6, 61, -2,} |

# Simulation environment

## Software

The software used to run the simulations is HM4.0. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [4]. YUV file output during the decoding was disabled when collecting the decoding time statistics.

## Hardware

All the simulation experiments were conducted on the PCs with identical settings, i.e., Microsoft Windows 7 64bits, Intel Xeon 3.2GHz with 14G memory. Therefore, both the encoding time and decoding time comparisons are reliable.

# Simulation Results

The experimental results of the rate-distortion performance, speed comparison and complexity analysis are summarized in Table 3-5. Detailed results are included in the attachments (JCTVC-G697(set0).xls, JCTVC-G697(set1).xls, and JCTVC-G697(set2).xls).

Table 3 Rate-distortion performance, speed comparison and complexity analysis of JSPIF set0

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% | 0.3% | 0.4% | 0.4% |
| Class B | 0.0% | 0.2% | 0.2% | 0.3% | 0.3% | 0.3% |
| Class C | -0.4% | -0.1% | -0.1% | -0.5% | 0.1% | 0.1% |
| Class D | -1.1% | -0.4% | -0.5% | -1.4% | -0.4% | -0.5% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -0.1% | -0.1% | -0.3% | 0.1% | 0.1% |
|  | -0.3% | -0.1% | -0.1% | -0.3% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 96% |  |  |
| Mult[%] | -11% |  |  | -12% |  |  |
| Adds[%] | -14% |  |  | -15% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 1% |  |  | 10% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | 1% |  |  | 10% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.5% | -0.1% | -0.4% | -0.4% | 0.3% | 0.4% |
| Class C | -0.8% | -0.3% | -0.2% | -0.8% | 0.4% | 0.0% |
| Class D | -1.2% | -0.6% | -0.3% | -1.2% | 0.0% | -0.1% |
| Class E | -0.7% | -0.1% | 0.4% | -0.9% | 0.2% | -0.8% |
| Overall | -0.8% | -0.3% | -0.2% | -0.8% | 0.2% | 0.0% |
|  | -0.8% | -0.3% | -0.2% | -0.8% | 0.2% | 0.0% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
| Mult[%] | -7% |  |  | -7% |  |  |
| Adds[%] | -10% |  |  | -10% |  |  |
| Access[%] | 1% |  |  | 1% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 3% |  |  | 4% |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 3% |  |  | 4% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6% | -0.4% | -0.4% | -1.2% | 0.1% | -0.3% |
| Class C | -0.9% | -0.2% | -0.2% | -0.8% | 0.0% | 0.0% |
| Class D | -0.6% | -0.3% | -0.2% | -0.6% | 0.1% | 0.0% |
| Class E | -1.4% | -0.8% | -0.6% | -3.0% | -1.5% | -1.1% |
| Overall | -0.8% | -0.4% | -0.3% | -1.3% | -0.2% | -0.3% |
|  | -0.8% | -0.4% | -0.4% | -1.3% | -0.3% | -0.4% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
| Mult[%] | 3% |  |  | 4% |  |  |
| Adds[%] | 2% |  |  | 2% |  |  |
| Access[%] | 4% |  |  | 4% |  |  |
| MemBand(2D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(2D) Max.[%] | 6% |  |  | 7% |  |  |
| MemBand(1D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(1D) Max.[%] | 6% |  |  | 7% |  |  |
| Gain Ave.[%] | -0.72% | -0.11% | -0.14% |  |  |  |
| Gain Min. [%] | -5.60% | -2.59% | -2.78% |  |  |  |
| Gain Max.[%] | 0.70% | 1.17% | 1.43% |  |  |  |
| Enc Time[%] | 99.77% |  |  |  |  |  |
| Dec Time[%] | 98.70% |  |  |  |  |  |
| Mult Ave.[%] | -5% |  |  |  |  |  |
| Mult Worst[%] | -18% |  |  |  |  |  |
| Adds Ave.[%] | -7% |  |  |  |  |  |
| Adds Worst[%] | -21% |  |  |  |  |  |
| Access[%] | 2% |  |  |  |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  |  |  |  |
| MemBand(2D) Max.[%] | 5% |  |  |  |  |  |
| MemBand(2D) Worst[%] | -25% |  |  |  |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  |  |  |  |
| MemBand(1D) Max.[%] | 5% |  |  |  |  |  |
| MemBand(1D) Worst[%] | -17% |  |  |  |  |  |

Table 4 Rate-distortion performance, speed comparison and complexity analysis of JSPIF set1

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.2% | 0.0% | 0.4% | 0.3% | 0.5% |
| Class B | 0.2% | 0.4% | 0.4% | 0.6% | 0.4% | 0.5% |
| Class C | -0.4% | 0.0% | 0.0% | -0.5% | 0.1% | 0.2% |
| Class D | -1.1% | -0.4% | -0.3% | -1.4% | -0.5% | -0.4% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | 0.1% | 0.0% | -0.2% | 0.1% | 0.2% |
|  | -0.2% | 0.1% | 0.0% | -0.2% | 0.1% | 0.2% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 97% |  |  |
| Mult[%] | -12% |  |  | -13% |  |  |
| Adds[%] | -15% |  |  | -16% |  |  |
| Access[%] | 4% |  |  | 4% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 2% |  |  | 4% |  |  |
| MemBand(1D) Ave.[%] | -3% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | 1% |  |  | 4% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.3% | 0.2% | -0.1% | 0.5% | 0.4% |
| Class C | -0.8% | -0.1% | -0.1% | -0.9% | 0.5% | 0.1% |
| Class D | -1.3% | -0.5% | -0.3% | -1.4% | 0.2% | 0.2% |
| Class E | -0.5% | -0.1% | -0.2% | -0.4% | 0.2% | -0.8% |
| Overall | -0.7% | -0.1% | -0.1% | -0.7% | 0.4% | 0.1% |
|  | -0.7% | -0.1% | -0.1% | -0.7% | 0.4% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
| Mult[%] | -9% |  |  | -10% |  |  |
| Adds[%] | -12% |  |  | -13% |  |  |
| Access[%] | 6% |  |  | 5% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 5% |  |  | 4% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | 4% |  |  | 4% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6% | -0.4% | -0.4% | -1.2% | 0.1% | -0.3% |
| Class C | -0.9% | -0.2% | -0.2% | -0.8% | 0.0% | 0.0% |
| Class D | -0.6% | -0.3% | -0.2% | -0.6% | 0.1% | 0.0% |
| Class E | -1.4% | -0.8% | -0.6% | -3.0% | -1.5% | -1.1% |
| Overall | -0.8% | -0.4% | -0.3% | -1.3% | -0.2% | -0.3% |
|  | -0.8% | -0.4% | -0.4% | -1.3% | -0.3% | -0.4% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
| Mult[%] | 3% |  |  | 4% |  |  |
| Adds[%] | 2% |  |  | 2% |  |  |
| Access[%] | 4% |  |  | 4% |  |  |
| MemBand(2D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(2D) Max.[%] | 6% |  |  | 7% |  |  |
| MemBand(1D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(1D) Max.[%] | 6% |  |  | 7% |  |  |
| Gain Ave.[%] | -0.64% | -0.03% | -0.07% |  |  |  |
| Gain Min. [%] | -5.73% | -2.59% | -2.29% |  |  |  |
| Gain Max.[%] | 0.94% | 1.44% | 1.22% |  |  |  |
| Enc Time[%] | 99.92% |  |  |  |  |  |
| Dec Time[%] | 99.82% |  |  |  |  |  |
| Mult Ave.[%] | -6% |  |  |  |  |  |
| Mult Worst[%] | -18% |  |  |  |  |  |
| Adds Ave.[%] | -9% |  |  |  |  |  |
| Adds Worst[%] | -21% |  |  |  |  |  |
| Access[%] | 5% |  |  |  |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  |  |  |  |
| MemBand(2D) Max.[%] | 5% |  |  |  |  |  |
| MemBand(2D) Worst[%] | -25% |  |  |  |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  |  |  |  |
| MemBand(1D) Max.[%] | 4% |  |  |  |  |  |
| MemBand(1D) Worst[%] | -17% |  |  |  |  |  |

Table 5 Rate-distortion performance, speed comparison and complexity analysis of JSPIF set2

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.0% | 0.1% | 0.2% | 0.2% |
| Class B | -0.1% | 0.2% | 0.1% | 0.1% | 0.2% | 0.2% |
| Class C | -0.4% | -0.1% | -0.1% | -0.5% | 0.0% | 0.0% |
| Class D | -1.1% | -0.5% | -0.4% | -1.3% | -0.5% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.4% | 0.0% | -0.1% | -0.4% | 0.0% | 0.1% |
|  | -0.4% | 0.0% | -0.1% | -0.4% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 96% |  |  |
| Mult[%] | -11% |  |  | -12% |  |  |
| Adds[%] | -14% |  |  | -15% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 0% |  |  | 4% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | -1% |  |  | 4% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | 0.0% | 0.1% | -0.5% | 0.4% | 0.5% |
| Class C | -0.9% | -0.1% | -0.2% | -0.6% | 0.1% | 0.0% |
| Class D | -1.2% | -0.9% | -0.1% | -1.0% | 0.3% | 0.2% |
| Class E | -0.7% | -0.7% | 0.1% | -1.0% | 0.5% | -0.5% |
| Overall | -0.8% | -0.4% | 0.0% | -0.7% | 0.3% | 0.1% |
|  | -0.8% | -0.4% | -0.1% | -0.7% | 0.3% | 0.1% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
| Mult[%] | -7% |  |  | -8% |  |  |
| Adds[%] | -10% |  |  | -11% |  |  |
| Access[%] | 1% |  |  | 1% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 11% |  |  | 4% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 10% |  |  | 4% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6% | -0.4% | -0.4% | -1.2% | 0.1% | -0.3% |
| Class C | -0.9% | -0.2% | -0.2% | -0.8% | 0.0% | 0.0% |
| Class D | -0.6% | -0.3% | -0.2% | -0.6% | 0.1% | 0.0% |
| Class E | -1.4% | -0.8% | -0.6% | -3.0% | -1.5% | -1.1% |
| Overall | -0.8% | -0.4% | -0.3% | -1.3% | -0.2% | -0.3% |
|  | -0.8% | -0.4% | -0.4% | -1.3% | -0.3% | -0.4% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
| Mult[%] | 3% |  |  | 4% |  |  |
| Adds[%] | 2% |  |  | 2% |  |  |
| Access[%] | 4% |  |  | 4% |  |  |
| MemBand(2D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(2D) Max.[%] | 6% |  |  | 7% |  |  |
| MemBand(1D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(1D) Max.[%] | 6% |  |  | 7% |  |  |
| Gain Ave.[%] | -0.72% | -0.13% | -0.09% |  |  |  |
| Gain Min. [%] | -5.37% | -3.31% | -2.48% |  |  |  |
| Gain Max.[%] | 0.59% | 0.83% | 1.87% |  |  |  |
| Enc Time[%] | 99.78% |  |  |  |  |  |
| Dec Time[%] | 98.95% |  |  |  |  |  |
| Mult Ave.[%] | -5% |  |  |  |  |  |
| Mult Worst[%] | -18% |  |  |  |  |  |
| Adds Ave.[%] | -8% |  |  |  |  |  |
| Adds Worst[%] | -21% |  |  |  |  |  |
| Access[%] | 2% |  |  |  |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  |  |  |  |
| MemBand(2D) Max.[%] | 5% |  |  |  |  |  |
| MemBand(2D) Worst[%] | -25% |  |  |  |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  |  |  |  |
| MemBand(1D) Max.[%] | 5% |  |  |  |  |  |
| MemBand(1D) Worst[%] | -17% |  |  |  |  |  |

# References

[1] K. Minoo and J. Lou, “Joint Sub-pixel Interpolation for bi-predicted Motion Compensation,” JCT-VC document, JCTVC-F601, July 2011

[2] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0

[3] E. Alshina, “CE3: Cross-check for Motorola Mobility proposals on interpolation MC by Samsung,” JCT-VC document, JCTVC-G820, November 2011

[4] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-F900, July 2011

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).