| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G698 |
| --- | --- |

| Title: | On Chroma interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Koohyar Minoo Limin Wang Ajay Luthra 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-2521 kminoo@motorola.com +1-858-404-3893 limin.wang@motorola.com +1-858-404-3470 aluthra@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This contribution document proposes a new 4-tap half-pel Chroma interpolation filter for HEVC. Compared to the current half-pel Chroma interpolation filter, better Rate-Distortion performance could be achieved. When combined with the default HM Luma filters, it could achieve -0.01%/-0.77%/-0.94% bitrate savings on Y/U/V. When combined with the 8H7Q filters with 1/4 offset, it could achieve -0.03%/-0.67%/-0.80% bitrate savings on Y/U/V. When combined with the 8H7Q filters with 3/16 offset, it could achieve -0.03%/-0.62%/-0.77% bitrate savings on Y/U/V. The cross-check will be provided by Samsung.

# Introduction

In the current HEVC, fixed 4-tap Chroma interpolation filters are used. This contribution proposes a new 4-tap half-pel Chroma interpolation filter for HEVC.

# Proposed Chroma interpolation filters

In the current HEVC, filter {-4, 36, 36, -4}/64 is used for half-pel Chroma interpolation. It is proposed to use {-7, 39, 39, -7}/64 for HEVC for better Rate-Distortion performance while maintaining the same complexity. The proposed Chroma filter could also work with other Luma filters as 8H7Q with 1/4 offset, 8H7Q with 3/16 offset [1] and etc.

# Simulation environment

## Software

The software used to run the simulations is HM4.0 [2]. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [3]. YUV file output during the decoding was disabled when collecting the decoding time statistics.

## Hardware

All the simulation experiments were conducted on the PCs with identical settings, i.e., Microsoft Windows 7 64bits, Intel Xeon 3.2GHz with 14G memory. Therefore, both the encoding time and decoding time comparisons are reliable.

# Simulation Results

The experimental results of the rate-distortion performance and speed comparison are summarized in Table 1 when the proposed Chroma filter is combined with the default Luma filters. Table 2 and Table 3 provide the experimental results when the proposed filter is combined with 8H7Q (1/4 offset) Luma filters and corresponding Chroma filters, and 8H7Q (3/16 offset) Luma filters and corresponding Chroma filters. Note that when combining the proposed Chroma filter with other filters, different anchors are used. Detailed results are included in the attachments (JCTVC-G698(with anchor).xls, JCTVC-G698(with 8H7Q 4_16).xls, and JCTVC-G698(with 8H7Q 3_16).xls). The cross-check will be provided by Samsung [4].

Table 1 Rate-distortion, speed performance comparison and complexity analysis of the proposed Chroma filter with default Luma filters

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.7% | -0.5% | 0.0% | -1.3% | -0.9% |
| Class B | 0.0% | -0.5% | -0.5% | 0.0% | -1.0% | -0.9% |
| Class C | 0.0% | -0.5% | -0.5% | 0.0% | -0.9% | -1.0% |
| Class D | 0.0% | -0.7% | -0.8% | 0.0% | -1.4% | -1.9% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.6% | -0.6% | 0.0% | -1.1% | -1.1% |
|  | 0.0% | -0.6% | -0.5% | 0.0% | -1.1% | -1.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Max.[%] | 8% |  |  | 6% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Max.[%] | 8% |  |  | 6% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.4% | -0.9% | 0.0% | -1.5% | -1.6% |
| Class C | 0.0% | -0.6% | -0.6% | -0.1% | -1.2% | -1.9% |
| Class D | -0.1% | -0.6% | -2.0% | 0.0% | -1.9% | -2.8% |
| Class E | 0.0% | 1.1% | 1.6% | 0.0% | -0.1% | 0.2% |
| Overall | 0.0% | -0.2% | -0.6% | -0.1% | -1.3% | -1.6% |
|  | 0.0% | -0.2% | -0.6% | -0.1% | -1.3% | -1.7% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Max.[%] | 12% |  |  | 7% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Max.[%] | 11% |  |  | 7% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.4% | -0.4% | 0.0% | -1.1% | -1.2% |
| Class C | 0.0% | -0.3% | -0.4% | 0.0% | -1.2% | -1.5% |
| Class D | 0.0% | -0.8% | -1.6% | -0.1% | -1.8% | -2.8% |
| Class E | 0.0% | 1.0% | 1.8% | 0.1% | -0.6% | 0.0% |
| Overall | 0.0% | -0.2% | -0.3% | 0.0% | -1.2% | -1.4% |
|  | 0.0% | -0.2% | -0.3% | 0.0% | -1.2% | -1.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Max.[%] | 4% |  |  | 4% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Max.[%] | 5% |  |  | 6% |  |  |
| Gain Ave.[%] | -0.01% | -0.77% | -0.94% |  |  |  |
| Gain Min. [%] | -0.19% | -4.32% | -6.98% |  |  |  |
| Gain Max.[%] | 0.23% | 2.38% | 2.13% |  |  |  |
| Enc Time[%] | 99.98% |  |  |  |  |  |
| Dec Time[%] | 99.44% |  |  |  |  |  |
| Mult Ave.[%] | 0% |  |  |  |  |  |
| Mult Worst[%] | 0% |  |  |  |  |  |
| Adds Ave.[%] | 0% |  |  |  |  |  |
| Adds Worst[%] | 0% |  |  |  |  |  |
| Access[%] | 0% |  |  |  |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  |  |  |  |
| MemBand(2D) Max.[%] | 7% |  |  |  |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |  |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  |  |  |  |
| MemBand(1D) Max.[%] | 7% |  |  |  |  |  |
| MemBand(1D) Worst[%] | 0% |  |  |  |  |  |

Table 2 Rate-distortion, speed performance comparison and complexity analysis of the proposed Chroma filter with 8H7Q (1/4 offset) Luma filters and corresponding Chroma filters (Anchor is from 8H7Q (1/4 offset) Luma filters and corresponding Chroma filters)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.5% | -0.4% | 0.0% | -1.0% | -0.8% |
| Class B | 0.0% | -0.4% | -0.6% | 0.0% | -0.9% | -0.8% |
| Class C | -0.1% | -0.4% | -0.4% | 0.0% | -0.8% | -0.8% |
| Class D | 0.0% | -0.5% | -0.9% | 0.0% | -1.1% | -1.5% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.5% | -0.5% | 0.0% | -1.0% | -1.0% |
|  | 0.0% | -0.4% | -0.5% | 0.0% | -1.0% | -1.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Max.[%] | 6% |  |  | 3% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Max.[%] | 5% |  |  | 3% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.4% | -0.7% | 0.0% | -1.3% | -1.4% |
| Class C | 0.0% | -0.4% | -0.2% | -0.1% | -1.2% | -1.3% |
| Class D | 0.0% | -0.6% | -1.2% | -0.1% | -2.0% | -2.4% |
| Class E | 0.0% | 0.5% | 1.1% | -0.1% | 0.1% | 0.4% |
| Overall | 0.0% | -0.2% | -0.4% | -0.1% | -1.2% | -1.3% |
|  | 0.0% | -0.2% | -0.4% | -0.1% | -1.2% | -1.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Max.[%] | 4% |  |  | 9% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Max.[%] | 4% |  |  | 9% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.7% | 0.0% | -1.2% | -1.0% |
| Class C | 0.0% | -0.3% | -0.4% | 0.0% | -0.9% | -1.2% |
| Class D | -0.1% | -0.4% | -1.2% | -0.1% | -1.8% | -2.5% |
| Class E | 0.0% | 0.9% | 0.7% | 0.1% | -0.1% | 0.5% |
| Overall | 0.0% | -0.1% | -0.5% | 0.0% | -1.1% | -1.1% |
|  | 0.0% | -0.1% | -0.4% | 0.0% | -1.0% | -1.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Max.[%] | 3% |  |  | 5% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Max.[%] | 3% |  |  | 5% |  |  |
| Gain Ave.[%] | -0.03% | -0.67% | -0.80% |  |  |  |
| Gain Min. [%] | -0.29% | -4.22% | -5.88% |  |  |  |
| Gain Max.[%] | 0.17% | 1.83% | 1.59% |  |  |  |
| Enc Time[%] | 99.96% |  |  |  |  |  |
| Dec Time[%] | 100.14% |  |  |  |  |  |
| Mult Ave.[%] | 0% |  |  |  |  |  |
| Mult Worst[%] | 0% |  |  |  |  |  |
| Adds Ave.[%] | 0% |  |  |  |  |  |
| Adds Worst[%] | 0% |  |  |  |  |  |
| Access[%] | 0% |  |  |  |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  |  |  |  |
| MemBand(2D) Max.[%] | 5% |  |  |  |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |  |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  |  |  |  |
| MemBand(1D) Max.[%] | 5% |  |  |  |  |  |
| MemBand(1D) Worst[%] | 0% |  |  |  |  |  |

Table 3 Rate-distortion, speed performance comparison and complexity analysis of the proposed Chroma filter with 8H7Q (3/16 offset) Luma filters and corresponding Chroma filters (Anchor is from 8H7Q (3/16 offset) Luma filters and corresponding Chroma filters)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.5% | -0.4% | 0.0% | -1.0% | -0.7% |
| Class B | 0.0% | -0.4% | -0.4% | 0.0% | -0.8% | -0.8% |
| Class C | 0.0% | -0.2% | -0.3% | 0.0% | -0.7% | -0.8% |
| Class D | 0.0% | -0.5% | -0.7% | 0.0% | -0.9% | -1.4% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.4% | -0.5% | 0.0% | -0.9% | -0.9% |
|  | 0.0% | -0.4% | -0.4% | 0.0% | -0.9% | -0.9% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 3% |  |  | 2% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | 3% |  |  | 1% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.4% | -0.4% | -0.1% | -1.3% | -1.3% |
| Class C | 0.0% | -0.5% | -0.4% | 0.0% | -1.1% | -1.6% |
| Class D | -0.1% | -0.8% | -1.4% | -0.1% | -1.9% | -2.7% |
| Class E | 0.0% | 0.3% | 0.5% | -0.2% | 0.3% | 0.8% |
| Overall | 0.0% | -0.4% | -0.5% | -0.1% | -1.1% | -1.3% |
|  | 0.0% | -0.3% | -0.5% | -0.1% | -1.1% | -1.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Max.[%] | 4% |  |  | 7% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Max.[%] | 4% |  |  | 7% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | -1.2% | -0.8% |
| Class C | 0.0% | -0.5% | -0.5% | 0.0% | -0.9% | -1.2% |
| Class D | 0.0% | -0.7% | -1.7% | -0.1% | -1.4% | -2.1% |
| Class E | 0.0% | 1.1% | 1.4% | 0.1% | 0.7% | 0.1% |
| Overall | 0.0% | -0.2% | -0.4% | 0.0% | -0.8% | -1.0% |
|  | 0.0% | -0.2% | -0.4% | 0.0% | -0.8% | -1.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | 0% |  |  | 0% |  |  |
| Adds[%] | 0% |  |  | 0% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Max.[%] | 5% |  |  | 4% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Max.[%] | 5% |  |  | 4% |  |  |
| Gain Ave.[%] | -0.03% | -0.62% | -0.77% |  |  |  |
| Gain Min. [%] | -0.26% | -3.78% | -6.94% |  |  |  |
| Gain Max.[%] | 0.19% | 2.11% | 1.59% |  |  |  |
| Enc Time[%] | 99.95% |  |  |  |  |  |
| Dec Time[%] | 99.86% |  |  |  |  |  |
| Mult Ave.[%] | 0% |  |  |  |  |  |
| Mult Worst[%] | 0% |  |  |  |  |  |
| Adds Ave.[%] | 0% |  |  |  |  |  |
| Adds Worst[%] | 0% |  |  |  |  |  |
| Access[%] | 0% |  |  |  |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  |  |  |  |
| MemBand(2D) Max.[%] | 4% |  |  |  |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |  |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  |  |  |  |
| MemBand(1D) Max.[%] | 4% |  |  |  |  |  |
| MemBand(1D) Worst[%] | 0% |  |  |  |  |  |

# References

[1] E. Alshina, A. Alshin, J.-H. Park, J. Lou, and K. Minoo, “CE3: 7 taps interpolation filters for quarter pel position MC from Samsung and Motorola Mobility” JCT-VC document, JCTVC-G778, November 2011

[2] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0

[3] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-F900, July 2011

[4] E. Alshina, “Non CE3: Cross-check for Motorola Mobility proposals on Chroma interpolation filter by Samsung,” JCT-VC document, JCTVC-G824, November 2011

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).