| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G0700 |
| --- | --- |

| Title: | On coeff_abs_level_minus3 coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Limin Wang Xue Fang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-3893 limin.wang@motorola.com +1-858-404-2723 X.Fang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

In the current CABAC of HEVC, syntax coeff_ab_level_minu3 is used to represent the absolute value of a quantized transform coefficient level minus 3. Due to the recent several changes in CABAC level coding, it is proposed to remove unnecessary operations and provide reasonable binarization to reduce the complexity and potentially increase the compression performance. Cross-check will be provided by HHI. The attached spreadsheet contains detailed data of the results.

# Introduction

In the current CABAC of HEVC, syntax coeff_ab_level_minu3 is used to represent the absolute value of a quantized transform coefficient level minus 3. Recently, several changes have been adopted in the HEVC level coding, such as applying adaptive Golomb-Rice code [1] and unification of scans for significance map coding and coefficient level coding [2]. It is proposed to remove unnecessary operations and provide reasonable binarization to reduce the complexity and potentially increase the compression performance.

Trucated Rice codes are used for coeff_abs_level_minus3 coding to increase the number of bypass bins. The Truncated Rice code is generated by two parameters, cRiceParam and cTRMax. In the current HEVC, cRiceParam could have a value of 0, 1, 2, or 3 and it controls the flatness of the codeword distribution. Generally, Truncated Rice code with smaller cRiceParam value is preferred to code the symbols with smaller values, as it needs few bits to represent. For each cRiceParam value, there is a related cTRMax value which controls the truncation point for the rice code. Table 1 gives the value of cTRMax for each cRiceParam value. If the symbol is larger than cTRMax, 0-th order Exp-Golomb code will be applied to code the value of (symbol-cTRMax-1), which is appended to the last Truncated Rice codeword.

Table 1 Specification of cTRMax depending on cRiceParam

| cRiceParam | cTRMax |
| --- | --- |
| 0 | 7 |
| 1 | 20 |
| 2 | 42 |
| 3 | 70 |

Before the unification of scans for significance map coding and coefficient level coding was adopted, for coding absolute transform coefficient levels, each block is mapped onto an ordered set of 4×4 sub-blocks by using a forward zig-zag scan; while the transform coefficient levels inside a sub-block are processed in a backward zig-zag scan (Fig. 1). Within each sub-block, cRiceParam is initialized as 0 and then updated from the previous cRiceParam and coeff_abs_level_minus3[n-1] according to Table 2. Generally, this updating scheme should work well as the levels are getting larger along the reverse scan within each 4×4 sub-block.

Fig. 1. Scanning order for level coding in HM2.0

Table 2 cRiceParam update table

|  | coeff_abs_level_minus3[n-1] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | ≥15 |
| Previous cRiceParam | 0 | 0 | 0 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 |
|  | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 |
|  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 | 3 |
|  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |

This initialization and updating process was kept when the unification of scans for significance map coding and coefficient level coding was adopted. In each subset of 16 continous coefficients, cRiceParam is initialized as 0 and then updated from the previous cRiceParam and coeff_abs_level_minus3[n-1] according to Table 2. Even though we could assume the levels are getting larger along the reverse scan within subset of 16 continous coefficients, it is not reasonable to reset cRiceParam to 0 for each sub-set.

Fig. 2. Scanning order for level coding in HM4.0

# Proposed changes to coeff_abs_level_minus3 coding

## Removing resetting cRiceParam for each subset

It is proposed to remove resetting cRiceParam value to 0 for each subset excluding the 1st one. With this change, the operations of resetting for each subset are saved and once the cRiceParam reaches 3, the symbols will always be binarized with the same set truncated rice codes (cRiceParam equals to 3), thus potentially reducing the hardware complexity.

A non-trained cRiceParam update table is also proposed (Table 3).

Table 3 New cRiceParam update table

|  | coeff_abs_level_minus3[n-1] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | ≥15 |
| Previous cRiceParam | 0 | 0 | 0 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 |
|  | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 |
|  | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 3 |
|  | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |

Combination logic representation

1) Old Table

cRiceParam = cRiceParam

+ (coeff_abs_level_minus3[n-1] >= 2   &&  cRiceParam == 0)

+ (coeff_abs_level_minus3[n-1] >= 4   &&  cRiceParam <= 1)

+ (coeff_abs_level_minus3[n-1] >= 13  &&  cRiceParam == 0)

+ (coeff_abs_level_minus3[n-1] >= 11  &&  cRiceParam == 1)

+ (coeff_abs_level_minus3[n-1] >= 10  &&  cRiceParam == 2));

2) Proposed table

cRiceParam = cRiceParam

+ (coeff_abs_level_minus3[n-1] >= 2   &&  cRiceParam == 0)

+ (coeff_abs_level_minus3[n-1] >= 5   &&  cRiceParam <= 1)

+ (coeff_abs_level_minus3[n-1] >= 11  &&  cRiceParam <= 2);

In the current HM4.0, the Trucated Rice Codes with different cRiceParam is generated as those in Table 4. Table 3 is generated from Table 4 by analyzing the number of bits needed for each symbol with different cRiceParam value while assuming the next level value is statistically no smaller than the current level value along the reverse scan. In Table 4, the codewords highlighted in green shows transition point in Table 3.

For example,

- If the current symbol is 2 and cRiceParam is 0, the chance that the next symbol is larger than 2 is high and applying Truncated Rice code with cRiceParam equal to 1 might reduce the number of bits

- If the current symbol is 5 and cRiceParam is 1, the chance that the next symbol is larger than 5 is high and applying Truncated Rice code with cRiceParam equal to 2 might reduce the number of bits

- If the current symbol is 11 and cRiceParam is 2, the chance that the next symbol is larger than 11 is high and applying Truncated Rice code with cRiceParam equal to 3 might reduce the number of bits

Table 4 Codewords of the Truncated Rice code with different cRiceParam and cTRMax

| cRiceParam | 0 |  | 1 |  | 2 |  | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Symbol | Codeword | Bits | Codeword | Bits | Codeword | Bits | Codeword | Bits |
| 0 | 0 | 1 | 00 | 2 | 000 | 3 | 0000 | 4 |
| 1 | 10 | 2 | 01 | 2 | 001 | 3 | 0001 | 4 |
| 2 | 110 | 3 | 100 | 3 | 010 | 3 | 0010 | 4 |
| 3 | 1110 | 4 | 101 | 3 | 011 | 3 | 0011 | 4 |
| 4 | 11110 | 5 | 1100 | 4 | 1000 | 4 | 0100 | 4 |
| 5 | 111110 | 6 | 1101 | 4 | 1001 | 4 | 0101 | 4 |
| 6 | 1111110 | 7 | 11100 | 5 | 1010 | 4 | 0110 | 4 |
| 7 | 11111110 | 8 | 11101 | 5 | 1011 | 4 | 0111 | 4 |
| 8 | 11111111,EG0 | 9 | 111100 | 6 | 11000 | 5 | 10000 | 5 |
| 9 |  |  | 111101 | 6 | 11001 | 5 | 10001 | 5 |
| 10 |  |  | 1111100 | 7 | 11010 | 5 | 10010 | 5 |
| 11 |  |  | 1111101 | 7 | 11011 | 5 | 10011 | 5 |
| 12 |  |  | 11111100 | 8 | 111000 | 6 | 10100 | 5 |
| 13 |  |  | 11111101 | 8 | 111001 | 6 | 10101 | 5 |
| 14 |  |  | 111111100 | 9 | 111010 | 6 | 10110 | 5 |
| 15 |  |  | 111111101 | 9 | 111011 | 6 | 10111 | 5 |
| 16 |  |  | 1111111100 | 10 | 1111000 | 7 | 110000 | 6 |
| 17 |  |  | 1111111101 | 10 | 1111001 | 7 | 110001 | 6 |
| 18 |  |  | 11111111100 | 11 | 1111010 | 7 | 110010 | 6 |
| 19 |  |  | 11111111101 | 11 | 1111011 | 7 | 110011 | 6 |
| 20 |  |  | 11111111110 | 11 | 11111000 | 8 | 110100 | 6 |
| 21 |  |  | 11111111111,EG0 | 12 | 11111001 | 8 | 110101 | 6 |
| 22 |  |  |  |  | 11111010 | 8 | 110110 | 6 |
| 23 |  |  |  |  | 11111011 | 8 | 110111 | 6 |
| 24 |  |  |  |  | 111111000 | 9 | 1110000 | 7 |
| 25 |  |  |  |  | 111111001 | 9 | 1110001 | 7 |
| 26 |  |  |  |  | 111111010 | 9 | 1110010 | 7 |
| 27 |  |  |  |  | 111111011 | 9 | 1110011 | 7 |
| 28 |  |  |  |  | 1111111000 | 10 | 1110100 | 7 |
| 29 |  |  |  |  | 1111111001 | 10 | 1110101 | 7 |
| 30 |  |  |  |  | 1111111010 | 10 | 1110110 | 7 |
| 31 |  |  |  |  | 1111111011 | 10 | 1110111 | 7 |
| 32 |  |  |  |  | 11111111000 | 11 | 11110000 | 8 |
| 33 |  |  |  |  | 11111111001 | 11 | 11110001 | 8 |
| 34 |  |  |  |  | 11111111010 | 11 | 11110010 | 8 |
| 35 |  |  |  |  | 11111111011 | 11 | 11110011 | 8 |
| 36 |  |  |  |  | 111111111000 | 12 | 11110100 | 8 |
| 37 |  |  |  |  | 111111111001 | 12 | 11110101 | 8 |
| 38 |  |  |  |  | 111111111010 | 12 | 11110110 | 8 |
| 39 |  |  |  |  | 111111111011 | 12 | 11110111 | 8 |
| 40 |  |  |  |  | 1111111111000 | 13 | 111110000 | 9 |
| 41 |  |  |  |  | 1111111111001 | 13 | 111110001 | 9 |
| 42 |  |  |  |  | 1111111111010 | 13 | 111110010 | 9 |
| 43 |  |  |  |  | 1111111111011,EG0 | 14 | 111110011 | 9 |
| 44 |  |  |  |  |  |  | 111110100 | 9 |
| 45 |  |  |  |  |  |  | 111110101 | 9 |
| 46 |  |  |  |  |  |  | 111110110 | 9 |
| 47 |  |  |  |  |  |  | 111110111 | 9 |
| 48 |  |  |  |  |  |  | 1111110000 | 10 |
| 49 |  |  |  |  |  |  | 1111110001 | 10 |
| 50 |  |  |  |  |  |  | 1111110010 | 10 |
| 51 |  |  |  |  |  |  | 1111110011 | 10 |
| 52 |  |  |  |  |  |  | 1111110100 | 10 |
| 53 |  |  |  |  |  |  | 1111110101 | 10 |
| 54 |  |  |  |  |  |  | 1111110110 | 10 |
| 55 |  |  |  |  |  |  | 1111110111 | 10 |
| 56 |  |  |  |  |  |  | 11111110000 | 11 |
| 57 |  |  |  |  |  |  | 11111110001 | 11 |
| 58 |  |  |  |  |  |  | 11111110010 | 11 |
| 59 |  |  |  |  |  |  | 11111110011 | 11 |
| 60 |  |  |  |  |  |  | 11111110100 | 11 |
| 61 |  |  |  |  |  |  | 11111110101 | 11 |
| 62 |  |  |  |  |  |  | 11111110110 | 11 |
| 63 |  |  |  |  |  |  | 11111110111 | 11 |
| 64 |  |  |  |  |  |  | 11111111000 | 11 |
| 65 |  |  |  |  |  |  | 11111111001 | 11 |
| 66 |  |  |  |  |  |  | 11111111010 | 11 |
| 67 |  |  |  |  |  |  | 11111111011 | 11 |
| 68 |  |  |  |  |  |  | 11111111100 | 11 |
| 69 |  |  |  |  |  |  | 11111111101 | 11 |
| 70 |  |  |  |  |  |  | 11111111110 | 11 |
| 71 |  |  |  |  |  |  | 11111111111,EG0 | 12 |

It is suggested to change the working draft as follows

#### 9.3.2.10 Binarization process for coeff_abs_level_minus3

The variable cRiceParam is derived as follows.

–	If cLastSE is equal to 0 or 1 and cLastRiceParam is equal to 0, cRiceParam is set equal to 0.

–	Otherwise if cLastSE is equal to 2 or 3 and cLastRiceParam is equal to 0 or 1, cRiceParam is set equal to 1.

–	Otherwise if one of the following conditions is true, cRiceParam is set equal to 2.

–		cLastSE is in the range of 4 to 12 and cLastRiceParam is equal to 0.

–	cLastSE is in the range of 4 to 10 and cLastRiceParam is equal to 1.

–	cLastSE is in the range of 4 to 9 and cLastRiceParam is equal to 2.

–	Otherwise, cRiceParam is set equal to 3.

cRiceParam = cLastRiceParam + (cLastSE >= 2 && cLastRiceParam == 0)

+ (cLastSE >= 5 && cLastRiceParam <= 1)

+ (cLastSE >= 11 && cLastRiceParam <= 2);

## Problem with the Trucated Rice Coding in HM4.0

In HM4.0, the max length of Truncated Rice code prefix is controlled by

const UInt g_auiGoRicePrefixLen[4] = { 8, 10, 11, 8};

With these values, Trucated Rice Codes with different cRiceParam is generated as those in Table 4. Notice that when symbol is larger than 39 and cRiceParam is 2 (highlighted in yellow), the codeword has redundancies. So it is suggested to change g_auiGoRicePrefixLen to

const UInt g_auiGoRicePrefixLen[4] = { 8, 10, 10, 8};

With this change, the code words will be generated as those in Table 5.

Table 5 Truncated Rice Codes for symbols larger than 39 with cRiceParam equal to 2

| Symbol | HM4.0 |  | Fixed |  |
| --- | --- | --- | --- | --- |
|  | Codeword | Bits | Codeword | Bits |
| 40 | 1111111111000 | 13 | 111111111100 | 12 |
| 41 | 1111111111001 | 13 | 111111111101 | 12 |
| 42 | 1111111111010 | 13 | 111111111110 | 12 |
| 43 | 1111111111011,EG0 | 14 | 111111111111,EG0 | 13 |

In order to make minimum changes in the working draft while following the Truncated Rice code binarization in Table 6, it is suggested to change the working draft as follows

Table 6 Specification of cTRMax depending on cRiceParam

| cRiceParam | cTRMax |
| --- | --- |
| 0 | 8 |
| 1 | 20 |
| 2 | 42 |
| 3 | 70 |

9.3.2.10 Binarization process for coeff_abs_level_minus3

The binarization of coeff_abs_level_minus3 consists of a prefix part and (when present) a suffix part. The prefix part of the binarization is derived by invoking the TR binarization process as specified in subclause 9.3.2.3 for the syntax element coeff_abs_level_minus3[ n ] with the variables cRiceParam and cTRMax as the inputs.

When the prefix bit string is not equal to the bin string that consists of ( cTRMax + 1 >> cRiceParam ) + cRiceParam ones, the bin string consists of a prefix bin string and a suffix bin string. The suffix bin string is derived for coeff_abs_level_minus3[ n ] - cTRMax -1 using the EG0 binarization as specified in subclause 9.3.2.4.

9.3.2.3 Truncated Rice (TR) binarization process

The semantics of the working draft in section “9.3.2.3 Truncated Rice (TR) binarization process” is defined as

Input to this process is a request for a TR binarization for a syntax element, cRiceParam and cTRMax.

Output of this process is the TR binarization of the syntax element.

A TR bin string is a concatenation of a prefix bit string and (only when cRiceParam is not equal to 0) a suffix bit string. The prefix of the binarization is specified by invoking the TU binarization process for the prefix part of the value specified by synElVal min (synElVal, cTRMax) >> cRiceParam with cMax = cTRMax >> cRiceParam. The suffix of the TR bin string is the binary representation of synElVal – ( ( synElVal >> cRiceParam ) << cRiceParam ), and its length is equal to cRiceParam.

NOTE – For the input parameter cRiceParam = 0 the TR binarization is exactly the TU binarization.

# Simulation environment

## Software

The software used to run the simulations is HM4.0 [3]. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [4]. YUV file output during the decoding was disabled when collecting the decoding time statistics.

## Hardware

The simulation experiments were conducted on the PCs with different settings, so the encoder time and decoding time comparisons are not provided.

# Simulation Results

The experimental results of the rate-distortion performance are summarized in Table 5-7. Table 5 gives the results by removing cRiceParam resetting only. Table 6 gives the results by removing cRiceParam resetting with the new cRiceParam update table. Table 7 gives the results by changing g_auiGoRicePrefixLen. Detailed results are included in the attachments (JCTVC-G700RemoveResetOnly.xls, JCTVC-G700 RemoveReset+NewUpdateTable.xls, and JCTVC-G700NewGoRicePrefixLen.xls). Cross-check will be provided by HHI [5].

Table 5 Rate-distortion performance when removing cRiceParam resetting only

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.2% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.2% | -0.2% | 0.0% |
| Class B | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 0.2% | -0.1% |
| Class E |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% |
| Class C | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.1% | 0.1% | 0.9% |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Overall | 0.0% | 0.0% | 0.1% |
|  | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |

Table 6 Rate-distortion performance when removing cRiceParam resetting and with new cRiceParam update table

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.1% | 0.1% | -0.4% |
| Class B | 0.0% | 0.2% | 0.0% |
| Class C | 0.0% | 0.0% | 0.2% |
| Class D | 0.0% | 0.1% | 0.1% |
| Class E |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Overall | 0.0% | 0.1% | 0.0% |
|  | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.2% | -0.5% |
| Class C | 0.0% | 0.0% | -0.2% |
| Class D | 0.0% | 0.2% | 0.1% |
| Class E | 0.1% | -0.8% | -0.2% |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Overall | 0.0% | -0.2% | -0.2% |
|  | 0.0% | -0.3% | -0.2% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |

Table 7 Rate-distortion performance with new g_auiGoRicePrefixLen table

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.0% | -0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.1% | 0.1% |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Overall | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |

Additional experimental results of the rate-distortion performance with the new cRiceParam update table at low QP settings are summarized in Table 8 and Table 9. In this additional experiment, QP values of 2, 7, 12, and 17 are employed. Detailed results are included in the attachments (JCTVC-G0700_30frames_NewUpdateTable.xls and JCTVC-G0700_Full_NewUpdateTable.xls).

Table 8 Rate-distortion performance with new cRiceParam update table only on QP 2, 7, 12, 17 (30 frames)

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.1% | 0.0% |
| Class E | 0.0% | 0.1% | 0.1% |
| Overall | 0.04% | 0.03% | 0.04% |
|  | 0.04% | 0.03% | 0.03% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |
| Overall | -0.03% | -0.01% | -0.02% |
|  | -0.03% | -0.01% | -0.02% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | -0.1% |
| Class E | 0.0% | -0.1% | -0.1% |
| Overall | -0.02% | -0.06% | -0.08% |
|  | -0.02% | -0.10% | -0.10% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | -0.04% | 0.00% | 0.01% |
|  | -0.04% | 0.00% | -0.01% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |

Table 9 Rate-distortion performance with new cRiceParam update table only on QP 2, 7, 12, 17 (full sequences)

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.04% | 0.01% | 0.02% |
| Class B | 0.00% | -0.02% | -0.02% |
| Class C | 0.04% | 0.03% | 0.03% |
| Class D | 0.05% | 0.04% | 0.05% |
| Class E | 0.06% | 0.04% | 0.04% |
| Overall | 0.03% | 0.02% | 0.02% |
|  | 0.03% | 0.02% | 0.02% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.03% | -0.03% | -0.02% |
| Class B | #VALUE! | #VALUE! | #VALUE! |
| Class C | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.02% | -0.02% | -0.03% |
| Class E |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |
|  | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.04% | -0.06% | -0.06% |
| Class C | -0.04% | -0.03% | -0.05% |
| Class D | -0.04% | 0.00% | -0.05% |
| Class E | -0.02% | -0.06% | -0.05% |
| Overall | -0.03% | -0.04% | -0.05% |
|  | -0.03% | -0.05% | -0.04% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.03% | -0.04% | -0.03% |
| Class C | -0.03% | -0.02% | -0.01% |
| Class D | -0.03% | -0.05% | -0.07% |
| Class E | -0.03% | -0.02% | -0.02% |
| Overall | -0.03% | -0.03% | -0.03% |
|  | -0.03% | -0.03% | -0.04% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |

# References

[1] T. Nguyen, “CE11: Coding of transform coefficient levels with Golomb-Rice codes,” JCT-VC document, JCTVC-E253, March 2011

[2] J. Sole, R. Joshi, and M. Karczewicz, “CE11: Unified scans for the significance map and coefficient level coding in high efficiency,” JCT-VC document, JCTVC-F288, July 2011

[3] https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0

[4] JCT-VC, “Common test conditions and software reference configurations”, JCTVC-F900, July 2011

[5] T. Nguyen “Crosscheck - On coeff_abs_level_minus3 coding (G700),” JCT-VC document, JCTVC-G854, November 2011

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).