| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G703 |
| --- | --- |

| Title: | CE11: Adaptive Scan for Large Blocks for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu Krit Panusopone Jian Lou Limin Wang | Tel: Email: | +1-858-404-2251 yyu@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This document reports the results of Motorola Mobility’s adaptive scan for big block size for intra coding to the new video coding standard, HEVC (High Efficiency Video Coding). The simulations were conducted using HM4.0 with Motorola Mobility’s modifications. Without increasing complexity at both encoder and decoder, average coding gains of 0.3% and 0.1% are achieved for low complexity and high efficiency intra coding cases, respectively. Cross-check will be provided by HHI.

# Introduction

The target of this document is to verify the performance of Motorola Mobility’s adaptive scan for big block size for intra coding proposed in [4]. Two new coding scans, namely new horizontal and new vertical scans, for large, such as 16x16 and 32x32, TU blocks in intra coding, are proposed to improve coding efficiency.

- The Proposed Adaptive Scan

The new horizontal and vertical scans are illustrated in Fig. 1-Fig.2, where a current block of NxN, N can be either 16 or 32, is partitioned into four quadrants.

Fig. 1. New horizontal (left) and vertical (right) scans for 16x16 Block

Fig. 2. New horizontal (left) and vertical (right) scans for 32x32 block

The new horizontal and vertical scans in Fig. 1-2 can be used for coding the coefficients of a TU block of 16x16 or 32x32.

Coefficient Coding for Low Complexity Configuration

In low complexity configuration, the quantized coefficients of a TU block of 16x16 or 32x32 in intra mode are coded along with one of three scans, namely wavefront, new horizontal or new vertical scan.

Coefficient Coding for High Efficiency Configuration

In high efficiency configuration, the quantized coefficients of a TU are coded in three parts. They are significant map, levels, and signs.

The significant map of a current TU is of the same size as the TU. The elements in significant map of a TU block of 16x16 or 32x32 in intra node are coded along with one of three scans, namely wavefront, new horizontal, and new vertical scan, depending upon the intra prediction direction associated with the TU block of 16x16 or 32x32.

Regarding level and sign coding of a TU block, we propose use of three scan patterns, namely wavefront, horizontal, and vertical scans.

# Simulation Results

The proposed method was implemented in the HM-4.0 and tested to evaluate its coding performance and complexity. The binaries were compiled using Microsoft Visual Studio 2010 64 bits. The coding conditions used for the simulations are specified in [3]. Experimental results for all test conditions are given in Table 1. Detailed results are included in the attachment (JCTVC-G703.xls).

Table 1: Coding performance and complexity of the proposed method

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.5% | -0.4% | -0.5% | -1.3% | -1.2% |
| Class B | 0.0% | -0.5% | -0.6% | -0.3% | -0.7% | -0.7% |
| Class C | -0.1% | -0.2% | -0.3% | -0.1% | -0.2% | -0.2% |
| Class D | 0.0% | -0.3% | -0.3% | -0.1% | -0.2% | -0.2% |
| Class E | -0.3% | -0.9% | -0.7% | -0.4% | -1.2% | -0.8% |
| Class F |  |  |  |  |  |  |
| Overall | -0.1% | -0.5% | -0.5% | -0.3% | -0.7% | -0.6% |
|  | -0.1% | -0.5% | -0.5% | -0.3% | -0.7% | -0.6% |
| Enc Time[%] | 100% |  |  | 108% |  |  |
| Dec Time[%] | 101% |  |  | 110% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.1% | -0.4% | -0.4% | -0.5% | -0.4% |
| Class B | 0.0% | -0.2% | -0.3% | -0.1% | -0.5% | -0.4% |
| Class C | 0.0% | -0.2% | 0.1% | -0.1% | -0.2% | 0.1% |
| Class D | 0.0% | 0.0% | -0.2% | 0.0% | -0.2% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | -0.2% | -0.2% | -0.4% | -0.2% |
|  | 0.0% | -0.1% | -0.2% | -0.2% | -0.4% | -0.2% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | -0.2% | 0.0% | -0.1% | 0.1% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.2% |
| Class D | 0.0% | 0.2% | 0.4% | 0.0% | -0.3% | -0.1% |
| Class E | 0.1% | 0.1% | 0.0% | -0.3% | -0.4% | 0.3% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.0% | -0.1% | -0.2% | 0.0% |
|  | 0.0% | 0.1% | 0.0% | -0.1% | -0.2% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% |
| Class C | 0.0% | 0.2% | -0.3% | 0.0% | -0.1% | 0.0% |
| Class D | 0.1% | -0.1% | -0.7% | -0.1% | -0.2% | -0.5% |
| Class E | 0.1% | 0.3% | -0.4% | -0.2% | -1.0% | -0.5% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | -0.3% | -0.1% | -0.3% | -0.2% |
|  | 0.0% | 0.1% | -0.4% | -0.1% | -0.3% | -0.2% |
| Enc Time[%] | 114% |  |  | 115% |  |  |
| Dec Time[%] | 117% |  |  | 119% |  |  |

# Conclusions

# In this report, the performance of an adaptive scan method for large blocks for intra coding [4] is confirmed. Without increasing complexity at both encoder and decoder, average coding gains of 0.3% and 0.1% are achieved for low complexity and high efficiency intra coding cases, respectively.

# References

[1] “HEVC Specification Working Draft 2,” JCT-VC document, JCTVC-D503, Jan. 2011.

[2] Y. Zheng, M. Coban, X. Wang, J. Sole, R. Joshi, M. Karczewicz, “CE11: Mode Dependent Coefficient Scanning,” JCT-VC document, JCTVC-D393, Jan. 2011.

[3] Frank Bossen, “Common test conditions and software reference configurations,” JCT-VC document, JCTVC-D600, Jan. 2011.

[4] Yue Yu, Krit Panusopone, Jian Lou, and Limin Wang, “Adaptive Scan for Large Blocks for HEVC,” JCT-VC document, JCTVC-F569, July,  2011.

[5] Vivienne Sze, Tung Nguyen, Jianle Chen, Joel Sole, and Krit Panusopone, “Description of Core Experiment (CE11): Coefficient scanning and coding,” JCT-VC document, JCTVC-F911, July,  2011.

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).